3GPP TSG-RAN WG4#110bis	R4-2405454

Changsha, China, April 15th – 19th, 2024

Agenda item:	5.1.1.2

Source: 	Qualcomm France

Title: 	UL CA_n5A-n13A

Document for:	Approval

# Introduction

Analysis for UL CA_n5A-n13A is provided in this contribution.

- Background

This UL CA combination with BCS4/5 was added into RAN4 work in RAN#102 [1].

WF for this combination was agreed in RAN4#110 [2]:

Way forward: Alignment with DC_13A_n5A

-	Option 1: Evaluate and specify requirements for CA_n5A-n13A independent of the requirements specified for DC_13A_n5A in which case the requirements could be different in TS38.101-1 and in TS38.101-3

-	Option 2: Reuse requirements specified for DC_13A_n5A as such for CA_n5A-n13A i.e. no analysis is conducted for CA_n5A-n13A. NOTE: In case Option 2 is chosen, then the topics below are redundant

-	Option 3: Specify requirements for CA_n5A-n13A and then revise the requirements for DC_13A-n5A accordingly to have the same requirements for both

Way forward: IMD3 MSD for n5

Option 1: The following MSD test point is used for IMD3 MSD for n5

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n13 | n5 | 828 | 5 | 25 (RBstart=0) | 873 | 25 | FDD | IMD3 |
|  | n13 | 783 | 5 | 20 (RBstart=0) | 752 | N/A | FDD | N/A |

Way forward: IMD3 MSD for n13

-	Option 1: Evaluate and specify IMD3 MSD for n13 using the following test point:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n13 | n5 | 826.5 | 5 | 25 (RBstart=0) | 871.5 | N/A | FDD | N/A |
|  | n13 | 782 | 10 | 20 (RBstart=32) | 751 | TBD | FDD | IMD3 |

-	Option 2: Do not specify IMD3 MSD for n13

Way forward: Cross-band MSD for n5 and n13

-	Option 1: Specify cross-band MSD for n5 and 13 if supported by MSD analysis

-	Option 2: Do not specify cross-band MSD for n5 and n13

Way forward: ΔTIB, ΔRIB

-	Option 1: Evaluate ΔTIB, ΔRIB in similar manner as for the other LB-LB NR CA combinations

- Discussion

We had contribution [3] in RAN4#110. In this contribution we basically have same input, which we discuss in the context of agreed WF.

IMD3 for n5:

WF agreed test point for n13 IMD3 MSD, so it can be used as is.

Proposal 1:

: The following MSD test point is used for IMD3 MSD for n5

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n13 | n5 | 828 | 5 | 25 (RBstart=0) | 873 | 25 | FDD | IMD3 |
|  | n13 | 783 | 5 | 20 (RBstart=0) | 752 | N/A | FDD | N/A |

IMD3 for n13:

In [3] we provided our evaluation for this test point, if it is specified. We suggest further discussion among interested companies in RAN4#110bis to conclude on this topic. There are two ways to look at this; using normal narrowest BW’s or even wider BW’s with normal UL RB allocations/RBstarts, the IMD3 would not hit n13. However, for the case outlined in [3] and in WF[2], it does hit n13. If there is no MSD specified, and RAN5 were to specify such test case for “0dB MSD”, then UE would fail to meet the requirements.

Proposal 2: Further discuss in RAN4#110bis on if MSD test point for n13 DL is specified.

Cross-band MSD:

We suggest specifying cross-band MSD, if justified by companies analysis/averaging. This is just to be more aligned with several other LB-LB combinations specified recently.

Our proposals copied from [3] are:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n13 | 826.5 | 20 | 15 | 20 (RBstart=0) | 753.5 | 5 | 2.4 | >ACLR2 |
| n13 | n5 | 782 | 10 | 15 | 20 (RBstart=32) | 871.5 | 5 | 2.1 | >ACLR2 |

Proposal 3: Specify cross-band MSD, if justified by companies analysis/averaging

ΔTIB and ΔRIB:

We propose to use the same values as for DC_13A_n5A.

Proposal 4: Use the following ΔTIB and ΔRIB:

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] | ΔRIB,c [dB] |
| --- | --- | --- | --- |
| CA_n5A-n13A | n5 | 0.5 | 0 |
|  | n13 | 0.5 | 0 |

- Conclusion

Analysis for UL CA_n5A-n13A was provided with the following proposals.

Proposal 1: The following MSD test point is used for IMD3 MSD for n5

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n13 | n5 | 828 | 5 | 25 (RBstart=0) | 873 | 25 | FDD | IMD3 |
|  | n13 | 783 | 5 | 20 (RBstart=0) | 752 | N/A | FDD | N/A |

Proposal 2: Further discuss in RAN4#110bis on if MSD test point for n13 DL is specified

Proposal 3: Specify cross-band MSD, if justified by companies analysis/averaging. Our proposals are:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n13 | 826.5 | 20 | 15 | 20 (RBstart=0) | 753.5 | 5 | 2.4 | >ACLR2 |
| n13 | n5 | 782 | 10 | 15 | 20 (RBstart=32) | 871.5 | 5 | 2.1 | >ACLR2 |

Proposal 4: Use the following ΔTIB and ΔRIB:

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] | ΔRIB,c [dB] |
| --- | --- | --- | --- |
| CA_n5A-n13A | n5 | 0.5 | 0 |
|  | n13 | 0.5 | 0 |

- Reference

[1] RP-232920, “Band combinations for NR CA_DC 2 band DL with up to 2 band UL”, ZTE Corporation

[2] R4-2403869, “WF on CA_n5-n13 architecture and MSD”, Qualcomm

[3] R4-2300641, “UL CA_n5A-n13A”, Qualcomm