3GPP TSG-RAN WG4 Meeting #110bis	R4-2405464 
Changsha, China, 15th – 19th April, 2024

Agenda item:	6.10.1

Source:	CAICT

Title:	Analysis of 3GPP Rel-18 FR2 MIMO OTA lab alignment results

Document for:	Discussion

1	Introduction

The FR2 MIMO OTA lab alignment activity has made good progress. At the last RAN4 meeting and this meeting, 4 of the volunteer labs have submitted their measurement results of the performance alignment devices (PADs) [1]-[4]. As agreed at the last meeting, the target is to conclude the 1st round lab alignment activity at this meeting, and the remaining lab can submit the measurement results at RAN4 #111 meeting. The agreements are captured in the WF [5] as below:

Issue 2-2: FR2 MIMO OTA lab alignment activity Agreement: - Complete the 1st round lab alignment at the next meeting with the results from at least three labs. Allow remaining labs to submit measurement results afterwards. - Return the PADs to ETS-L by RAN4 #110bis (Apr. 2024). - All labs can submit data for Measurement Campaign, whether the data can be included in the data pool for specifying requirements depends on lab alignment outcome. - The deadline to submit lab alignment results and the data for Measurement Campaign is RAN4 #111 (May 2024).

Durig this meeting, the pass/fail criteria was defined as captured in the WF [6]:

Issue 2-2-1: Pass/fail criteria for FR2 MIMO OTA lab alignment Agreement: - For FR2 MIMO OTA lab alignment activity, apparent outliers (if identified) should not be considered in the average process for deriving reference values. The PAD measurement result deviates over [1.5]*preliminary MU (i.e., 7.575 dB for band n261) from all the other labs’ results should be identified as apparent outlier. - Define the same pass/fail limit as that for FR1 lab alignment, i.e., 0.75*preliminary MU.

This contribution presents an analysis of the 1st round FR2 MIMO OTA lab alignment results. The detailed analysis data is attached in the Excel worksheet of this contribution.

2	Discussion

The 1st round FR2 MIMO OTA lab alignment results are summarized and analysed as shown in Table 1.

Table 1. Summary of the 1st round FR2 MIMO OTA lab alignment results

| Device | Band | MASC70 measurement result [dBm/120kHz] |  |  |  |  | Average approach | Average value | Max-Min deviation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Lab A | Lab B | Lab C | Lab D | Lab E |  |  |  |
| PAD 1 | n261 | -104.97 | -101.79 | -100.64 |  |  | Linear average (in dB) | -102.47 | 4.33 |
| PAD 2 | n261 | -108.56 | -104.92 | -103.87 |  | -105.41 |  | -105.69 | 4.69 |
| PAD 3 | n261 | -105.40 | -101.92 | -101.22 |  | -103.31 |  | -102.96 | 4.18 |
| PAD 4 | n257 | -108.76 | -105.80 | -104.59 |  | -106.01 |  | -106.29 | 4.17 |
| Device | Band | TRMS offset [dBm/120kHz] |  |  |  |  | Pass/fail limit |  |  |
|  |  | Lab A | Lab B | Lab C | Lab D | Lab E |  |  |  |
| PAD 1 | n261 | -2.51 | 0.68 | 1.83 |  |  | ± 0.75*preliminary MU, i.e., ± 3.79 dB |  |  |
| PAD 2 | n261 | -2.87 | 0.77 | 1.82 |  | 0.28 |  |  |  |
| PAD 3 | n261 | -2.44 | 1.04 | 1.74 |  | -0.35 |  |  |  |
| PAD 4 | n257 | -2.47 | 0.49 | 1.70 |  | 0.28 |  |  |  |
| Lab alignment conclusion |  | Pass | Pass | Pass | TBA | Pass |  |  |  |

Figure 2 presents the 1st round MASC measurement results, and Figure 3 visualizes the deviations between each MASC measurement results and average values.

Figure 2. The 1st round FR2 MIMO OTA lab alignment measurement results

Figure 3. Deviation between each measurement result and the average value

Observation 1: Four volunteer labs provided their measurement results for FR2 MIMO OTA lab alignment in the 1st round. The maximum deviation between different labs is 4.69 dB, which is within one preliminary MU. No outlier value is observed.

Observation 2: The average value of each PAD is derived by the linear average (in dB) of all the 1st round measurement results. The maximum deviation between measurement results and average values is -2.87 dB, which is about 0.57*preliminary MU.

Observation 3: The performance rankings of the PADs are consistent across different labs.

Observation 4: The maximum deviation between measurement results and reference values is within the agreed pass/fail limit, +/- 0.75*preliminary MU (3.79 dB). The four labs in the 1st round can be confirmed as aligned.

To provide a comprehensive overview of the measurement results, the measured sensitivity values @ 70% TP at each of test points are illustrated in Annex.

3	Conclusion

This contribution presents an analysis of the 1st round FR2 MIMO OTA lab alignment results.

Observation 1: Four volunteer labs provided their measurement results for FR2 MIMO OTA lab alignment in the 1st round. The maximum deviation between different labs is 4.69 dB, which is within one preliminary MU. No outlier value is observed.

Observation 2: The average value of each PAD is derived by the linear average (in dB) of all the 1st round measurement results. The maximum deviation between measurement results and average values is -2.87 dB, which is about 0.57*preliminary MU.

Observation 3: The performance rankings of the PADs are consistent across different labs.

Observation 4: The maximum deviation between measurement results and reference values is within the agreed pass/fail limit, +/- 0.75*preliminary MU (3.79 dB). The four labs in the 1st round can be confirmed as aligned.

Annex: Measurement results at each test point

To provide a comprehensive overview of the measurement results, the measured sensitivity values @ 70% TP at each of test points are shown in Fig. A-1.

(a) PAD 1

(b) PAD 2

(c) PAD 3

(d) PAD 4

Figure A-1. Measured sensitivity values at each test point

References

- R4-2400194, On FR2 MIMO OTA Lab Alignment PADs measurement results, Apple, 3GPP RAN4#110, Feb. 2024.

- R4-2405463, Measurement results for 3GPP FR2 MIMO OTA Lab Alignment Activity, CAICT, 3GPP RAN4#110bis, Apr. 2024.

- R4-2405312, Measurement results for 3GPP FR2 MIMO OTA Lab Alignment Activity, CMCC, 3GPP RAN4#110bis, Apr. 2024.

- R4-2406133, Measurement results for 3GPP FR2 MIMO OTA Lab Alignment Activity, Huawei, HiSilicon, RAN4#110bis, Apr. 2024.

- R4-2402876, Way forward on [110][332] NR_MIMO_OTA_enh, CAICT, 3GPP RAN4#110, Feb. 2024.

- R4-2406083, Way Forward for [110bis][337] NR_MIMO_OTA_enh, RAN4#110bis, Apr. 2024.