3GPP TSG-RAN WG4 Meeting #110bis                                                       R4-2405477

Changsha, China, 15 – 19 Apr. 2024

Agenda Item:	6.11.2.2

Source:	Ericsson

Title:	Updated simulation results for phase II

Document for:	Information

# 1	Introduction

After last RAN4 #110 meeting’s discussion, test cases without modulation order blind detection have been 1st round down-selected, only a few options left for further decision. For test cases with modulation order blind detection, there are still plenty of candidates that needs more simulation campaigns to help make further down-selection. A way forward was approved to carry all the agreements and open issues [1].

In this contribution, we shared our updated simulation results for being the reference of making test case down-selection.

# 2	Discussion

## 2.1 Assumptions

In this section, we summarized the simulation assumptions that we used for our evaluations.

Following assumptions are assumed:

- Maximum transmission bandwidth configuration for all UEs

- Assume all scheduled DMRS ports have same QCL assumptions.

- Assume the scrambling ID for DMRS sequence is the same for the target UE the co-scheduled UE(s)

- Same CDM group for rank 1+1, different CDM groups for rank 2+2

The following table 2.1-1 is showing the general parameter configurations, which are basically using what were agreed in the previous meetings.

Table 2.1-1 General parameter configurations

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Number |  |  | 1 | 1 |
| Channel Bandwidth/SCS |  | MHz/kHz | 10/15 for FDD |  |
| Duplex mode |  |  | FDD |  |
| MCS |  |  | 13/17 | QPSK/16QAM |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | DMRS port allocation |  | For rank 1+1: Port 1000 For rank 2+2: Port 1000 and 1001 | For rank 1+1: Port 1001 For rank 2+2: Port 1002 and 1003 |
| MIMO configuration |  |  | 2T2R, 4T4R with different correlations for verification |  |
| CDM groups without data |  |  | Same for rank 1+1 / different for rank 2+2, 1+1+1 |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - For case1, 3, 21~28 with rank 1+1: Random - For case2, 4, 29~32 with rank 1+1: Orthogonal - For case with rank 2+2: Orthogonal |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | TDLC300-100 + TDLA30-10 |  |

## 2.2 Test cases and results summary

We’ve evaluated the following test cases and summarized the results as follows.

Table 2.2-1 Simulation results for cases without modulation order blind detection

| Test num | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Rank (target UE + co-scheduled UE) | Correlation matrix and antenna configuration | Reference value SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | Gain | E-IRC | RML |
| Rank 1+1 with 2T2R |  |  |  |  |  |  |  |  |  |
| 1 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA medium | 7.3 | 21.9 | 13.7 |
| 5 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA medium | 5.1 | 20.6 | 13.0 |
| Rank 1+1 with 2T4R |  |  |  |  |  |  |  |  |  |
| 4 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x4, ULA medium | 12.7 | 24.1 | 11.8 |
| 6 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x4, ULA medium | 8.1 | 19.8 | 11.0 |
| Rank 2+2 with 4T4R |  |  |  |  |  |  |  |  |  |
| 7 | 10 / 15 | MCS13 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Low | 2.1 | 12.2 | 10.3 |
| 8 | 10 / 15 | MCS13 | QPSK | TDLA30-10 | 2+2 | 4x4, XP medium | 2.9 | 13.7 | 11.6 |
| 9 | 10 / 15 | MCS17 | 16QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | 0.8 | 16.6 | 16.9 |
| 10 | 10 / 15 | MCS17 | 16QAM | TDLA30-10 | 2+2 | 4x4, XP medium | 1.3 | 18.2 | 18.9 |
| 10a | 10 / 15 | MCS17 | QPSK | TDLA30-10 | 2+2 | 4x4, XP medium | TBA | TBA | TBA |

Table 2.2-2 Simulation results for cases with modulation order blind detection

| Test num | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Rank (target UE + co-scheduled UE) | Correlation matrix and antenna configuration | Reference value SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | Gain | E-IRC | RML |
| Rank 1+1 with 2T2R |  |  |  |  |  |  |  |  |  |
| 21 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA medium | 7.1 | 23.6 | 15.1 |
| 22 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA low | 3.3 | 15.2 | 12.7 |
| 25 | 10 / 15 | MCS17 | 16QAM | TDLC300-100 | 1+1 | 2x2, ULA medium | Inf | N/A | 23.2 |
| 26 | 10 / 15 | MCS17 | 16QAM | TDLC300-100 | 1+1 | 2x2, ULA low | Inf | N/A | 18.1 |
| 29 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA medium | 5.0 | 18.9 | 14.1 |
| 31 | 10 / 15 | MCS17 | 16QAM | TDLC300-100 | 1+1 | 2x2, ULA medium | 5.2 | 25.7 | 20.6 |
| Rank 1+1 with 2T4R |  |  |  |  |  |  |  |  |  |
| 23 | 10 / 15 | MCS13 | QPSK | TDLA30-10 | 2+2 | 2x4, ULA low | 0.9 | 7.1 | 6.2 |
| 24 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 2+2 | 2x4, ULA medium | 8.1 | 24.7 | 13.7 |
| 27 | 10 / 15 | MCS17 | 16QAM | TDLA30-10 | 2+2 | 2x4, ULA low | 0.2 | 10.1 | 8.5 |
| 28 | 10 / 15 | MCS17 | 16QAM | TDLC300-100 | 2+2 | 2x4, ULA medium | Inf | N/A | 22.1 |
| 30 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 2+2 | 2x4, ULA medium | 6.2 | 18.5 | 12.2 |
| 32 | 10 / 15 | MCS17 | 16QAM | TDLC300-100 | 2+2 | 2x4, ULA medium | Inf | N/A | 21.1 |
| Rank 2+2 with 4T4R |  |  |  |  |  |  |  |  |  |
| 33 | 10 / 15 | MCS13 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA low | 1.1 | 11.7 | 10.3 |
| 34 | 10 / 15 | MCS13 | QPSK | TDLA30-10 | 2+2 | 4x4, XP medium | 1.4 | 13.3 | 12.7 |

# 3	Summary

In this contribution, we shared our simulation results based on the parameter configurations and assumptions listed in the WF of last RAN4 #110 meeting [1].

# References

[1]	R4-2402864: WF on the advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #110, Athens, Greece.