3GPP TSG-RAN WG4 Meeting #110bis                                                       R4-2405479

Changsha, China, 15th – 19th Apr. 2024

Agenda Item:	6.25.1

Source:	Ericsson

Title:	Simulation results for Rel-18 DSS enhancement demodulation

Document for:	Discussion

# 1	Introduction

In the last RAN4#110 meeting, all simulation parameters have been settled for defining demodulation requirement purpose. In this paper, we provided our simulation results for both FDD and TDD based on the agreed simulation assumption listed in the WF [1].

# 2	Discussion

## 2.1 Simulation assumptions

Table 2.1-1 FDD Simulation parameters

| Parameters | Values |  |
| --- | --- | --- |
| TDD UL-DL pattern | FDD | TDD |
|  | / | FR1.15-1 |
| SCS | 15 kHz |  |
| Bandwidth | 10 MHz |  |
| Channel model | TDLA30-10 and TDLC300-100 |  |
| Antenna configuration | 2x2, 2x4, 4x2, 4x4 |  |
| CRS | 4 port CRS |  |
| DCI payload (excluding CRC) | 39 bits |  |
| Interleaving | Non-interleaved |  |
| Precoding | Precoder cycling per REG bundle |  |
| REG bundle size | 6 |  |
| Shift index | 0 |  |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |  |
| CORSET RB | 48 |  |
| AL | 4, 8 |  |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |  |

## 2.2 Simulation results

Table 2.2-1 FDD Simulation results

| Test case | SCS(KHz)/CBW (MHz) | Duplex | CORESET RB | Payload | AL | Propagation condition | Antenna configuration | SNR(dB) @1%BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15/10 | FDD | 48 | 39 | 4 | TDLA30-10 | 2x4 Low | -2.85 |
| 2 | 15/10 | FDD | 48 | 39 | 8 | TDLC300-100 | 4x2 Low | -3.21 |
| 3 | 15/10 | TDD | 48 | 39 | 4 | TDLA30-10 | 2x4 Low | -2.85 |
| 4 | 15/10 | TDD | 48 | 39 | 8 | TDLC300-100 | 4x2 Low | -3.21 |

# 3	Summary

In this contribution, we provided our simulation results based on the agreed simulation assumption listed in the WF [1].

# References

[1]	R4-2402873: Way forward on [110][328] NR_DSS_enh, Ericsson, RAN4 #110, Athens, Greece.