3GPP TSG-RAN WG4 Meeting #110bis                                                       R4-2405482

Changsha, China, 15th – 19th Apr. 2024

Agenda Item:	6.16.8.2

Source:	Ericsson

Title:	Simulation results for NR NTN enhancement UE demodulation requirement

Document for:	Information

# 1	Introduction

Several agreements related to the test parameter have been made in RAN4 #110 meeting in Athens. In this contribution, we provided our simulation results for PDSCH and PDCCH with newly agreed configurations. For PDSCH, we verified the feasibility of proposed Doppler for DL and disable HARQ. For PDCCH, we provided simulation results for down-selecting the aggregation level.

# 2	Discussion

## 2.1 PDSCH simulation

In this section, we provided our simulation results with updated parameter assumptions.

### 2.1.1 Parameter assumption

Table 2.1.1-1 Updated simulation parameters for NR NTN PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Center Frequency |  | GHz | 20 |
| Bandwidth |  | MHz | 200 |
| Subcarrier spacing |  | kHz | 120 |
| Duplex Mode |  |  | FDD |
| TDD Slot Configuration |  |  | N/A |
| Channel model |  |  | NTN-TDLC5-600/1200/2000 600, 1200, 2000 configured as Doppler spread |
| MCS/Rank |  |  | Rank 1 MCS4 (QPSK 0.3) MCS13 (16QAM 0.48) |
| Antenna configuration |  |  | 1T1R |
| Transform precoding |  |  | CP-OFDM |
| HARQ | Maximum number of HARQ transmissions |  | 16/32 and disabled Disable HARQ: Set the number of HARQ Processes as: 4 with feedback disabled, 12 with feedback enabled in 16 HARQ processes with re-Tx disable for all HARQ processes and only transmit initial transmissions with NDI toggling. Throughput shall be measured on processes with HARQ enabled. Which 4 processes to disable are randomly select at test configuration. |
|  | RV sequence |  | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type |  | 1 |
|  | DM-RS duration |  | single-symbol DM-RS |
|  | Additional DM-RS position |  | pos 1 |
|  | Number of DM-RS CDM group(s) without data |  | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
|  | DM-RS port |  | {0} |
|  | DM-RS sequence generation |  | NID0=0, nSCID =0 |
| Time domain resource assignment | PDSCH mapping type |  | A |
|  | Start symbol |  | 2 |
|  | Allocation length |  | 12 |
| Frequency domain resource assignment | RB assignment |  | 200MHz for 120kHz SCS |
| Test metric |  |  | 70% of max throughput at target SNR. |

### 2.1.2 Simulation results

Table 2.1.2-1 Simulation results for NR NTN PDSCH

| Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | k-factor(dB) | Antenna configuration / Rank |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) |  | DM-RS Additional position | HARQ | MCS | SNR (dB) |
| 200MHz/120kHz | NTN-TDLC5- 2000Hz | 8,4 | 1x1, Rank 1 | 70 |  | Pos1 | Disabled | 4 | -0.09 |  |
|  |  |  |  |  |  |  | 16 HAQR | 4 | -1.26 |  |
|  |  |  |  |  |  |  | 32 HARQ | 4 | -1.26 |  |
|  |  |  |  |  |  |  | Disabled | 13 | 7.83 |  |
|  |  |  |  |  |  |  | 16 HAQR | 13 | 6.51 |  |
|  |  |  |  |  |  |  | 32 HARQ | 13 | 6.51 |  |
| 200MHz/120kHz | NTN-TDLC5- 600Hz(doppler) | 8,4 | 1x1, Rank 1 | 70 |  | Pos1 | Disabled | 4 | -0.23 |  |
|  |  |  |  |  |  |  | 16 HAQR | 4 | -1.48 |  |
|  |  |  |  |  |  |  | 32 HARQ | 4 | -1.48 |  |
|  |  |  |  |  |  |  | Disabled | 13 | 7.69 |  |
|  |  |  |  |  |  |  | 16 HAQR | 13 | 6.23 |  |
|  |  |  |  |  |  |  | 32 HARQ | 13 | 6.23 |  |
| 200MHz/120kHz | NTN-TDLC5- 1200Hz(doppler) | 8,4 | 1x1, Rank 1 | 70 |  | Pos1 | Disabled | 4 | -0.10 |  |
|  |  |  |  |  |  |  | 16 HAQR | 4 | -1.46 |  |
|  |  |  |  |  |  |  | 32 HARQ | 4 | -1.46 |  |
|  |  |  |  |  |  |  | Disabled | 13 | 7.82 |  |
|  |  |  |  |  |  |  | 16 HAQR | 13 | 6.29 |  |
|  |  |  |  |  |  |  | 32 HARQ | 13 | 6.29 |  |

Observation 1: Around 1dB loss for cases with disabled HARQ comparing with 16 and 32 HARQ

Observation 2: Less than 0.5dB loss for cases with 2000Hz Doppler spread comparing with 600Hz and 1200Hz

## 2.2 PDCCH simulation

In this section, we provided our initial simulation results based on current agreed parameter assumptions to assist down-selecting the aggregation level for NTN UE PDCCH requirements.

### 2.2.1 Parameter assumption

Table 2.2.1-1 Parameter assumption for NR NTN PDCCH [2]

| Parameter | Unit | 1 Tx Antenna |
| --- | --- | --- |
| CCE to REG mapping type |  | Non-Interleaved |
| REG bundle size |  | 6 |
| Shift index |  | 0 |
| Test metric |  | 1% BLER |
| DCI Format |  | 1_1 |
| Payload (without CRC) |  | 56 |

### 2.2.2 Simulation results

Table 2.2.2-1 Simulation results for NR NTN PDCCH

| Test num | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 200 | 60 | 1 | 4 | TBD | NTN-TDLC5- 2000Hz | 1x1 | 1 | 3,90 |
| 2 | 200 | 60 | 1 | 4 | TBD | NTN-TDLC5- 1200Hz | 1x1 | 1 | 3,84 |
| 3 | 200 | 60 | 1 | 4 | TBD | NTN-TDLC5- 600Hz | 1x1 | 1 | 3,86 |
| 4 | 200 | 60 | 1 | 8 | TBD | NTN-TDLC5- 2000Hz | 1x1 | 1 | 1,49 |
| 5 | 200 | 60 | 1 | 8 | TBD | NTN-TDLC5- 1200Hz | 1x1 | 1 | 1,44 |
| 6 | 200 | 60 | 1 | 8 | TBD | NTN-TDLC5- 600Hz | 1x1 | 1 | 1,49 |
| 7 | 200 | 60 | 1 | 16 | TBD | NTN-TDLC5- 2000Hz | 1x1 | 1 | -2,24 |
| 8 | 200 | 60 | 1 | 16 | TBD | NTN-TDLC5- 1200Hz | 1x1 | 1 | -2,24 |
| 9 | 200 | 60 | 1 | 16 | TBD | NTN-TDLC5- 600Hz | 1x1 | 1 | -2,16 |

Observation 3: The aggregation level 16 might be needed to guarantee the PDSCH performance of 16HARQ and 32HARQ process

# 3	Summary

In this contribution, we provided our simulation results for NR NTN PDSCH and PDCCH demodulation requirements.

We summarized our observations as follows:

Observation 1: Around 1dB loss for cases with disabled HARQ comparing with 16 and 32 HARQ

Observation 2: Less than 0.5dB loss for cases with 2000Hz Doppler spread comparing with 600Hz and 1200Hz

Observation 3: The aggregation level 16 might be needed to guarantee the PDSCH performance of 16HARQ and 32HARQ process

# References

[1]	R4-2402865: WF on NR_NTN_enh_SAN_UE_demod, Huawei, HiSilicon, RAN4 #110, Athens, Greece.

[2]	TS38.101-4