3GPP TSG-RAN WG4 Meeting #110bis 	R4-240xxxx

Changsha, China, 15th – 19th April, 2024

Agenda Item:	6.3.3.2

Source: 	Ericsson

Title:	On RRM measurement accuracy requirements for multi-rx

Document for:	Discussion

# Introduction

The following was discussed on multi-rx accuracy requirements [25]:

#### Issue 2-10: Accuracy requirements for multi-Rx in Rel-18

- FFS following options

  - Option 1a:

    - The legacy accuracy requirements for L1-RSRP measurement apply for L1-RSRP measurements with group-based beam reporting.

  - Option 1b:

    - The legacy accuracy requirements in section 10.1.20 of TS 38.133 apply for L1-RSRP measurements under multi-rx operation, with a clarification that multi-rx chain L1-RSRP accuracy requirements apply for FR2-1.

    - No new accuracy requirements section is created for L1-RSRP measurements under multi-rx operation.

  - Option 1c:

    - Under multi-Rx chain DL reception scenario, the legacy accuracy requirements for L1-RSRP measurement should be satisfied for both Rx chains.

    - In this sense, no new accuracy requirements section is created for L1-RSRP measurements under multi-rx operation

  - Option 2:

    - Do not define group-based L1-RSRP measurement accuracy requirements unless the requirements are applicable only when there are negligible mutual interferences across different pair of Tx-Rx beams.

# Discussion

## RRM accuracy requirements

Related earlier agreements:

RAN4#104bis-e

- Enhanced RRM requirements for multi-Rx chain UE should maintain the existing accuracy requirements. [R4-2217246]

- L1-RSRP/SINR Measurements: The number of samples assumed to define the accuracy requirements should not be changed. [R4-2217587]

Based on the earlier RAN4 agreements in [3,4], the legacy accuracy requirements shall apply for multi-rx chain operation, which also suggests no need for any new accuracy requirements section for L1-RSRP. However, it shall be clarified that the requirements apply for UE with multi-rx capability operating in FR2-1.

- Proposal 1 (Accuracy requirements): The legacy accuracy requirements in section 10.1.20 of TS 38.133 apply for L1-RSRP measurements under multi-rx operation, with a clarification that multi-rx chain L1-RSRP accuracy requirements also apply for UE with multi-rx capability operating in FR2-1.

- Proposal 2 (Accuracy requirements): No new accuracy requirements section is created for L1-RSRP measurements under multi-rx operation.

# Summary

The following have been proposed in this contribution.

- Proposal 1 (Accuracy requirements): The legacy accuracy requirements in section 10.1.20 of TS 38.133 apply for L1-RSRP measurements under multi-rx operation, with a clarification that multi-rx chain L1-RSRP accuracy requirements apply for UE with multi-rx capability operating in FR2-1.

- Proposal 2 (Accuracy requirements): No new accuracy requirements section is created for L1-RSRP measurements under multi-rx operation.

# References

[1] RP-221753, WI description, Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Inc., June 2022.

[2] R4-2214344, WF on FR2 multi-Rx RRM, vivo, Aug. 2022.

[3] R4-2217246, WF on RRM impacts and general aspects for multi-Rx, vivo, Oct. 2022.

[4] R4-2217587, WF on L1 measurements, beam sweeping factors and simultaneous reception, Qualcomm, Oct. 2022.

[5] R4-2217248, WF on TCI state switching for multi-RX chain DL reception, Ericsson, Oct. 2022.

[6] R4-2220742, WF on NR FR2 multi-Rx chain DL reception – Part 1, Nov. 2022.

[7] R4-2220743, WF on NR FR2 multi-Rx chain DL reception – Part 2, Nov. 2022.

[8] R4-2220402, WF on NR FR2 multi-Rx chain DL reception – Part 3, Nov. 2022.

[9] R4-2303193, Ad-hoc minutes for NR FR2 multi-Rx chain DL reception RRM, vivo, March 2023.

[10] R4-2303302, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, March 2023.

[11] R4-2303317, WF on NR FR2 multi-Rx chain DL reception RRM requirements(part2), Qualcomm, March 2023.

[12] R4-2306318, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, Apr. 2023.

[13] R4- 2306319, WF on FR2_multiRx_part2, Ericsson, Apr. 2023.

[14] R4-2306392, Topic summary for [106bis-e][202] FR2_multiRx_part1, vivo, Apr. 2023.

[15] R4-2306234, Topic summary for [106bis-e][203] FR2_multiRx_part2, Ericsson, Apr. 2023.

[16] R4-2310046, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, May 2023.

[17] R4-2310047, WF on FR2_multiRx_part2, Ericsson, May 2023.

[18] R4-2309952, Topic summary for [106][209] FR2_multiRx _part1, vivo, May 2023.

[19] R4-2309953, Topic summary for [107][208] FR2_multiRx_part2, Ericsson, May 2023.

[20] R4-2314477, WF on NR FR2 multi-RX operation RRM requirements (part 1), vivo, Aug. 2023.

[21] R4-2314283, WF on FR2 multi-Rx chain DL reception (part2), Ericsson, Aug. 2023.

[22] R4-2317332, WF on NR FR2 multi-RX operation RRM requirements (part 1), vivo, Oct. 2023.

[23] R4-2317427, WF on FR2_multiRx_part2, Ericsson, Oct. 2023.

[24] R4-2321602, WF on core maintenance and performance part for multi-Rx WI, vivo, Ericsson, Nov. 2023.

[25] R4-2403560, WF on core maintenance and performance part for multi-Rx WI, vivo, Ericsson, March 2024.