3GPP TSG-RAN WG4 Meeting #110bis 	R4-240xxxx

Changsha, China, 15th – 19th April, 2024

Agenda Item:	6.12.3.1

Source: 	Ericsson

Title:	On SL positioning performance

Document for:	Discussion

# Introduction

The latest agreed WF is in [16].

In this contribution, we discuss issues related to SL positioning performance.

# Discussion

## Side conditions for SL PRS measurements in RAN4 requirements

Side conditions need to be defined for core requirements and accuracy requirements.

- Proposal 1 (side conditions): For SL RSTD, the side conditions assumed in RAN4 requirements are: ≥0 dB for the reference link and ≥-6 dB for the measured link.

- Proposal 2 (side conditions): For SL Rx-Tx, the requirements are defined down to -6 dB.

- Proposal 3 (side conditions): Given that SL PRS-RSRP can be configured with other SL-PRS based measurements, the requirements for SL PRS-RSRP can be defined down to -6 dB.

- Proposal 4 (side conditions): Given that SL PRS-RSRPP can be configured with other SL-PRS based measurements, the requirements for SL PRS-RSRPP can be defined down to -6 dB.

- Proposal 5 (side conditions): For SL AoA, the core requirements apply down to -6 dB.

- Proposal 6 (side conditions): For SL RTOA, the core requirements apply down to -6 dB.

## Measurement accuracy requirements

- Proposal 7 (measurement accuracy requirements): Two sets of accuracy requirements are to be defined for:

  - 1 sample, and

  - 4 samples.

## Test cases for SL positioning measurements

Agreements from [16]:

- Define measurement delay test cases for SL RSTD, SL RTOA, SL Rx-Tx and SL-AoA/ZoA.

- Define measurement accuracy test cases for SL RSTD, SL Rx-Tx.

- Further discuss whether and how to define delay test cases for SL RSRP and SL RSRPP.

- Further discuss whether and how to define accuracy test cases for SL RSRP and SL RSRPP.

- Proposal 8 (test cases): Test cases for all SL positioning measurements are specified (SL RSTD, RSRP/RSRPP, Rx-Tx, AoA, and RTOA).

  - FFS: SL-RSRP and SL-RSRPP test cases are joint with other measurements.

- Proposal 9 (test cases): For each SL positioning measurement type, test cases for core requirements for SL positioning measurements are defined at least for both:

  - 1 sample, and

  - 4 samples.

- Proposal 10 (test cases): For each SL positioning measurement type (for which accuracy requirements are defined), test cases for measurement accuracy requirements are defined for both:

  - 1 sample, and

  - 4 samples.

- Proposal 11 (test cases): No need to define test cases for additional paths.

There have been no SL positioning test cases in the specification, so new top section needs to be created.

- Proposal 12 (test case structure): The following clause structure is proposed in TS 38.133 for SL positioning test cases:

A.9A 		Tests for NR SL Measurements for Positioning

A.9A.1 		Tests for NR SL Measurements for Positioning in FR1

A.9A.1.1 	Measurement delay tests

A.9A.1.2 	Measurement accuracy tests

# Summary

The following have been proposed in the current contribution:

- Proposal 1 (side conditions): For SL RSTD, the side conditions assumed in RAN4 requirements are: ≥0 dB for the reference link and ≥-6 dB for the measured link.

- Proposal 2 (side conditions): For SL Rx-Tx, the requirements are defined down to -6 dB.

- Proposal 3 (side conditions): Given that SL PRS-RSRP can be configured with other SL-PRS based measurements, the requirements for SL PRS-RSRP can be defined down to -6 dB.

- Proposal 4 (side conditions): Given that SL PRS-RSRPP can be configured with other SL-PRS based measurements, the requirements for SL PRS-RSRPP can be defined down to -6 dB.

- Proposal 5 (side conditions): For SL AoA, the core requirements apply down to -6 dB.

- Proposal 6 (side conditions): For SL RTOA, the core requirements apply down to -6 dB.

- Proposal 7 (measurement accuracy requirements): Two sets of accuracy requirements are to be defined for:

  - 1 sample, and

  - 4 samples.

- Proposal 8 (test cases): Test cases for all SL positioning measurements are specified (SL RSTD, RSRP/RSRPP, Rx-Tx, AoA, and RTOA).

  - FFS: SL-RSRP and SL-RSRPP test cases are joint with other measurements.

- Proposal 9 (test cases): For each SL positioning measurement type, test cases for core requirements for SL positioning measurements are defined at least for both:

  - 1 sample, and

  - 4 samples.

- Proposal 10 (test cases): For each SL positioning measurement type (for which accuracy requirements are defined), test cases for measurement accuracy requirements are defined for both:

  - 1 sample, and

  - 4 samples.

- Proposal 11 (test cases): No need to define test cases for additional paths.

- Proposal 12 (test case structure): The following clause structure is proposed in TS 38.133 for SL positioning test cases:

A.9A 		Tests for NR SL Measurements for Positioning

A.9A.1 		Tests for NR SL Measurements for Positioning in FR1

A.9A.1.1 	Measurement delay tests

A.9A.1.2 	Measurement accuracy tests

# References

[1] RP-230328, Revised WID on Expanded and Improved NR Positioning, Intel Corp, March, 2023.

[2] R4-2303190, WF on Expanded and Improved NR Positioning RRM requirements, Ericsson, March, 2023.

[3] R1-2301950, Summary #1 on Measurements and reporting for SL positioning, vivo, March 2023.

[4] R1-2301951, Summary #2 on Measurements and reporting for SL positioning, vivo, March 2023.

[5] R1-2301952	Summary #3 on Measurements and reporting for SL positioning, vivo, March 2023.

[6] R4-2306351, WF on RRM requirements for NR sidelink positioning and carrier phase positioning, CATT, April 2023.

[7] R4-2310163, WF on NR Positioning – SL positioning and CPP, CATT, May 2023.

[8] R4-2310077, Simulation assumptions for SL positioning, CATT, May 2023.

[9] R4-2314343, Ad-hoc minutes on R18 NR Positioning RRM requirements, Ericsson, Aug. 2023.

[10] R4-2314354, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Aug. 2023.

[11] R4-2314358, LS on SL positioning and CPP measurements report mapping, Aug. 2023.

[12] R4-2312724, On SL positioning, Ericsson, Aug. 2023.

[13] RP-232670, Revised WID on Expanded and Improved NR Positioning, Intel Corporation, CATT, MediaTek, Sep. 2023.

[14] R4-2317380, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Oct. 2023.

[15] R4-2321524, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, Nov. 2023.

[16] R4-2403368, WF on R18 NR positioning – SL positioning and Carrier Phase Positioning, CATT, March 2024.