3GPP TSG-RAN WG4 Meeting #110bis 	R4-2405543

# Changsha, Hunan, China,  15th April – 19th April, 2024

Source:	Ericsson

Title:	Simulation results for Rel-18 NR NTN FR2-1 PUCCH and FR1 PUSCH DM-RS bundling

Agenda item:	6.16.8.1

Document for:	Information

# Introduction

As per the WF in RAN4#110 meeting [1], companies agreed to share simulation results on FR2-1 PUSCH, PUCCH, PRACH and FR1 PUSCH DM-RS bundling. In our contribution [2] in the previous meeting, we shared results for FR2-1 PUSCH, PRACH and DM-RS bundling.

In this contribution, we share the results for FR2-1 PUCCH and FR1 PUSCH DM-RS bundling.

# Simulation result

  - Performance requirements for FR2-1 Normal PUCCH

- Requirements for PUCCH format 0

Table 1:  Test parameters of PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 for 2 symbols |
| Test metric | DTX to ACK <1% ACK missed <1% |

Table 1.1-1: Minimum requirements for PUCCH format 0, 60 kHz SCS and 50 MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex X) | SNR (dB) |
| --- | --- | --- | --- |
| 1 | 1 | NTN-TDLC5-3000 Low | 2.1 |
|  | 2 | NTN-TDLC5-3000 Low | -1.3 |

Table 1.1-2: Minimum requirements for PUCCH format 0, 120 kHz SCS and 50 MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex X) | SNR (dB) |
| --- | --- | --- | --- |
| 1 | 1 | NTN-TDLC5-3000 Low | 2.2 |
|  | 2 | NTN-TDLC5-3000 Low | -1.2 |

- Requirements for PUCCH format 1

Table 2:  Test parameters of PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Test metric | DTX to ACK<1% NACK to ACK <0.1% ACK missed <1% |

Table 2.1-1: Minimum requirements for PUCCH format 1, 60 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | NACK to ACK | ACK miss |
| 1 | 1 | Normal | NTN-TDLC5-3000 Low | -5.4 | -4.8 |
|  | 2 | Normal | NTN-TDLC5-3000 Low | -8.9 | -8 |

Table 2.1-2: Minimum requirements for PUCCH format 1, 120 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | NACK to ACK | ACK miss |
| 1 | 1 | Normal | NTN-TDLC5-3000 Low | -5.4 | -4.8 |
|  | 2 | Normal | NTN-TDLC5-3000 Low | -9.3 | -8.4 |

- Requirements for PUCCH format 2

Table 3   Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Test metric | DTX to ACK <1% ACK missed <1% |

Table 3.1-1: Minimum requirements for PUCCH format 2, 60 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-3000 Low | 2.9 |
|  | 2 | Normal | NTN-TDLC5-3000 Low | -2.5 |

Table 3.1-2: Minimum requirements for PUCCH format 2, 120 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-3000 Low | 2.8 |
|  | 2 | Normal | NTN-TDLC5-3000 Low | -2.5 |

Table 4:  Test parameters of PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER<1% |

Table 4.1-1: Minimum requirements for PUCCH format 2, 60 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-3000 Low | 0.7 |
|  | 2 | Normal | NTN-TDLC5-3000 Low | -4.6 |

Table 4.1-2: Minimum requirements for PUCCH format 2, 120 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | SNR (dB) |
| --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-3000 Low | 0.9 |
|  | 2 | Normal | NTN-TDLC5-3000 Low | -4.5 |

- Requirements for PUCCH format 3

Table 5:  Test parameters of PUCCH format 3

| Parameter | Test 1 |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |
| Test metric | BLER <1% |

Table 5.1-1: Minimum requirements for PUCCH format 3, 60 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | Additioan DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-3000 Low | No additional DM-RS | -1.4 |
|  |  |  |  | Additional DM-RS | -1.6 |
|  | 2 | Normal | NTN-TDLC5-3000 Low | No additional DM-RS | -4.4 |
|  |  |  |  | Additional DM-RS | -5.1 |

Table 5.1-2: Minimum requirements for PUCCH format 3, 120 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | Additioan DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-3000 Low | No additional DM-RS | -1.4 |
|  |  |  |  | Additional DM-RS | -1.6 |
|  | 2 | Normal | NTN-TDLC5-3000 Low | No additional DM-RS | -4.5 |
|  |  |  |  | Additional DM-RS | -5.1 |

- Requirements for PUCCH format 4

Table 6:  Test parameters of PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Test metric | BLER<1% |

Table 6.1-1: Minimum requirements for PUCCH format 4, 60 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | Additioan DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-3000 Low | No additional DM-RS | -0.9 |
|  |  |  |  | Additional DM-RS | -1.1 |
|  | 2 | Normal | NTN-TDLC5-3000 Low | No additional DM-RS | -4.1 |
|  |  |  |  | Additional DM-RS | -4.8 |

Table 6.1-2: Minimum requirements for PUCCH format 4, 120 kHz SCS and 50MHz channel bandwidth

| Number of TX antennas | Number of RX antennas | Cyclis Prefix | Propagation conditions and correlation matrix (Annex X) | Additioan DM-RS configuration | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-3000 Low | No additional DM-RS | -0.9 |
|  |  |  |  | Additional DM-RS | -1 |
|  | 2 | Normal | NTN-TDLC5-3000 Low | No additional DM-RS | -4.1 |
|  |  |  |  | Additional DM-RS | -4.7 |

2.2 Performance requirements for FR1 PUSCH with DM-RS bundling

Table 7: Test parameters for testing PUSCH with DM-RS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Example UL-DL pattern |  | FDD |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence (Note 1) | 0, 0, 0, 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n4/ n8 |
| pusch-TimeDomainWindowLength |  | 4/ 8 slots |
| Frequency domain resource assignment | RB assignment | Option 1: Full applicable test bandwidth 15kHz SCS: 5MHz 30kHz SCS: 10MHz Option 2: 6 PRBs for both 15 and 30kHz SCS |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Table 7.1-1: Minimum requirements for PUSCH with 70% of maximum throughput, MCS 4, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | Aggregation Factor | Mapping Type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | NTN-TDLA100-200 Low | 70 % | 4 | A | -7.75 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 4 | B | -7.76 |
| 1 | 2 | Normal | NTN-TDLA100-200 Low | 70 % | 8 | A | -9.03 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 8 | B | -9.06 |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | 4 | A | -4.11 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 4 | B | -4.16 |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | 8 | A | -6.09 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 8 | B | -6.13 |

Table 7.1-2: Minimum requirements for PUSCH with 70% of maximum throughput, MCS 4, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | Aggregation Factor | Mapping Type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | NTN-TDLA100-200 Low | 70 % | 4 | A | -8.21 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 4 | B | -8.21 |
| 1 | 2 | Normal | NTN-TDLA100-200 Low | 70 % | 8 | A | -10.17 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 8 | B | -10.18 |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | 4 | A | -4.11 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 4 | B | -4.09 |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | 8 | A | -7.16 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 8 | B | -7.16 |

Table 7.1-3: Minimum requirements for PUSCH with 70% of maximum throughput, MCS 4, 6 PRBs channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | Aggregation Factor | Mapping Type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | NTN-TDLA100-200 Low | 70 % | 4 | A | -7.09 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 4 | B | -7.10 |
| 1 | 2 | Normal | NTN-TDLA100-200 Low | 70 % | 8 | A | -8.06 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 8 | B | -8.07 |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | 4 | A | -3.74 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 4 | B | -3.77 |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | 8 | A | -5.42 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 8 | B | -5.46 |

Table 7.1-4: Minimum requirements for PUSCH with 70% of maximum throughput, MCS 4, 6 PRBs channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | Aggregation Factor | Mapping Type | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | NTN-TDLA100-200 Low | 70 % | 4 | A | -7.86 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 4 | B | -7.84 |
| 1 | 2 | Normal | NTN-TDLA100-200 Low | 70 % | 8 | A | -9.35 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 8 | B | -9.34 |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | 4 | A | -3.79 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 4 | B | -3.80 |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70 % | 8 | A | -6.41 |
|  |  | Normal | NTN-TDLA100-200 Low | 70 % | 8 | B | -6.40 |

# Conclusions

Simulation results for Rel-18 NR FR2-1 PUCCH and FR1 PUSCH DM-RS bundling are delivered.

# References

[1] R4-2402865, “WF on NR_NTN_enh_SAN_UE_demod”, Huawei, HiSilicon, RAN4#110.

[2] R4-2401403, “Simulation results for NR NTN enhancement SAN demodulation requirements”, Ericsson, RAN4#110.