3GPP TSG-RAN WG4 Meeting #110bis	R4-2405545

# Changsha, China, April 15th – 20th 2024

Source:	Ericsson

Title:	Simulation results for less than 5MHz BS demodulation requirements

Agenda item:	6.8.7.2

Document for:	Information

# Introduction

According to the WF in RAN4#110 [1], it is agreed that no new requirements are defined for 3MHz PUSCH and PRACH. In that case, only new 3MHz normal PUCCH format 2 UCI BLER requirement will be introduced.

In this contribution, simulation results with impairment value are provided for 3MHz PUCCH format 2 UCI BLER requirements.

# Simulation results

Table 2-1 PUCCH format 2 parameters

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | UCI BLER <1% |

Table 2-2 simulation results for 15 kHz SCS, 3MHz CBW

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Metric | Propagation conditions and correlation matrix (Annex G) | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ideal | impairment |
| 1 | 2 | Normal | UCI BLER <1% | TDLC-300-100 Low | 0.41 | 2.53 |
|  | 4 |  |  |  | -4.55 | -2.35 |
|  | 8 |  |  |  | -8.11 | -5.81 |

# Conclusions

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

No table of figures entries found.

# References

[1]	R4-2402863, Way Forward for NR_FR1_lessthan_5MHz_BW_demod, Nokia, Nokia Shanghai Bell