3GPP TSG-RAN WG4 Meeting #110bis 	R4-2405551

# Changsha, Hunan, China  15th April – 19th April, 2024

Source:	Ericsson

Title:	Simulation results for Rel-18 NR PRACH repetitions demodulation requirements

Agenda item:	6.17.2

Document for:	Information

# Introduction

In RAN4#110 meeting, companies agreed to check the simulation assumptions including TDD pattern and PRACH repetition interval for both FR1 and FR2. Therefore, we introduced slot interval between two PRACH repetitions, as discussed in [2]. Based on the WF [1], further results are delivered in this meeting.

# Simulation result

## 2.1	Simulation assumptions

Table 2-1: Test parameters for PRACH repetition demodulation requirement for FR1

| PRACH | PRACH SCS | Time error tolerance |  |
| --- | --- | --- | --- |
| preamble | (kHz) | AWGN | TDLC300-100 |
| B4, A2, C2 | 15 | 0.52 us | 2.03 us |
|  | 30 | 0.26 us | 1.77 us |

Table 2-1: Test parameters for PRACH repetition demodulation requirement for FR2

| PRACH | PRACH SCS | Time error tolerance |  |
| --- | --- | --- | --- |
| preamble | (kHz) | AWGN | TDLA30-300 |
| B4, A2, C2 | 60 | 0.13 us | 0.28 us |
|  | 120 | 0.07 us | 0.22 us |

## 2.2	Simulation results

Table 2-1: PRACH repetition missed detection requirements for Normal Mode, 15 kHz SCS in FR1

| Number | Number of | Propagation |  |  |  |
| --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | conditions and correlation matrix (Annex G) | Repetition | Slot interval | SNR (dB) |
|  |  |  |  |  | Format A2 |
| 1 | 2 | AWGN | 2 | 1 | -17.4 |
|  |  | TDLC300-100 Low | 2 | 1 | -11.3 |
|  |  |  |  | 5 | -11.76 |
|  |  |  |  | 10 | -12.54 |
|  |  |  |  | 15 | -12.54 |
|  |  |  |  | 20 | -12.69 |
|  |  |  |  | 25 | -12.63 |

Table 2-2: PRACH repetition missed detection requirements for Normal Mode, 30 kHz SCS in FR1

| Number | Number of | Propagation |  |  |  |
| --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | conditions and correlation matrix (Annex G) | Repetition | Slot interval | SNR (dB) |
|  |  |  |  |  | Format A2 |
| 1 | 2 | AWGN | 2 | 1 | -17.5 |
|  |  | TDLC300-100 Low | 2 | 1 | -11.4 |
|  |  |  |  | 5 | -11.32 |
|  |  |  |  | 10 | -12.11 |
|  |  |  |  | 15 | -12.73 |
|  |  |  |  | 20 | -12.91 |
|  |  |  |  | 25 | -12.97 |

Table 2-3: PRACH repetition missed detection requirements for Normal Mode, 60 kHz SCS in FR2-1

| Number | Number of | Propagation |  |  |  |
| --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | conditions and correlation matrix (Annex G) | Repetition | Slot interval | SNR (dB) |
|  |  |  |  |  | Format A2 |
| 1 | 2 | AWGN | 2 | 1 | -17.9 |
|  |  | TDLA30-300 Low | 2 | 1 | -10.3 |
|  |  |  |  | 5 | -10.72 |
|  |  |  |  | 10 | -11.92 |
|  |  |  |  | 15 | -11.98 |
|  |  |  |  | 20 | -11.84 |
|  |  |  |  | 25 | -11.96 |

Table 2-4: PRACH repetition missed detection requirements for Normal Mode, 120 kHz SCS in FR2-1

| Number | Number of | Propagation |  |  |  |
| --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | conditions and correlation matrix (Annex G) | Repetition | Slot interval | SNR (dB) |
|  |  |  |  |  | Format A2 |
| 1 | 2 | AWGN | 2 | 1 | -17.7 |
|  |  | TDLA30-300 Low | 2 | 1 | -10.6 |
|  |  |  |  | 5 | -10.69 |
|  |  |  |  | 10 | -11.52 |
|  |  |  |  | 15 | -12.21 |
|  |  |  |  | 20 | -12.50 |
|  |  |  |  | 25 | -12.93 |

# Conclusions

Simulation results for Rel-18 NR PRACH enhancement demodulation requirements are delivered.

# References

[1] R4-2402866, “WF on NR_cov_enh2_demod”, China Telecom, Nokia, RAN4#110

[2] R4-2405550, Discussion on NR further coverage enhancement demodulation requirements, Ericsson