3GPP TSG-RAN WG4 Meeting # 110bis							         		R4-2405554

Changsha, China, 15 – 19 Apr, 2024

Title: 	Template example for 2 band DL with 1 or 2 band UL up to 3 UL CCs

Source: 	Skyworks Solutions Inc.

Agenda Item:	11.3	RAN4 basket WI work plan (according to WF R4-2403721)

Document for:	Approval

# 1	Background

In RAN4#110, it was possible to discuss the paper [1] presenting our plans to improve the R19 block approval process and technical scope which resulted in way forward [2]. In this contribution, we share the template example for two band DL based on MSD issue detection tables proposed in [4-7] together with additional improvements to the TP structure and tables.

# Discussion

## 2.1 RAN4#100bis Proposed MSD issue detection tables

In last meeting RAN4#110 we previously shared multiple templates for the two DL band inter-band CA with one or two UL bands and up to three UL CCs. These documents were not treated as there was no specific AI for R19 in RAN4#110. Based on way forward [2], we have resubmitted a number of documents with further supporting companies to cover the same in AI 11.3 which can be used as the elements to build a complete two band DL TP template:

- [4] R4-2404243 Improved harmonic related MSDs template, Skyworks Solutions Inc., Nokia, RAN4#110bis

  - Proposes an improved template for UL harmonic and harmonic mixing detection table with clarifications on direct-hit and near-miss cases. Applies to 2 band DL / 1 band UL / 1UL CC and uses proposed UL/DL orders for harmonic mixing from [3]

- [5] R4-2404250 On cross band isolation MSD analysis, Skyworks Solutions Inc., Nokia, Murata, RAN4#110bis

  - Provides a missing template to detect cross-band isolation issues. Applies to 2 band DL / 1 band UL / 1UL CC

- [6] R4-2404249 On simplifying analysis for 2DL-1 band intra-band ULCA IMD products, Skyworks Solutions Inc., Nokia, Murata, RAN4#110bis

- [7] R4-2404248 On simplifying analysis for triple beat products, Skyworks Solutions Inc., Nokia, Murata, RAN4#110bis.

Separately, this document proposes:

- A small improvement for the 2 band DL / 2Band UL / 1UL CC per band detection table.

- A new TP structure based on number of UL bands and number of UL CCs

- Some guidelines and additional tables to sort the valid UL configurations and point at the necessary sections for analysis.

## 2.1 Proposed improvements to the 2DL/1or2UL coexistence analysis

Based on the above and the per MSD types contributions [4-7] in this meeting, we make the following proposal for the overall two band DL TP template.

Proposed enhancements for 2DL/1or2UL bands block approval TP template for Release 19:

- Addition at the end of section “5.XX.1.2 Channel bandwidths per operating band for CA” of:

  - A question related to the support of SimRx/Tx, or otherwise for TDD/TDD cases.

  - A table that sorts the applicable UL configuration and their related MSD studies

- For the 2DL/1UL section:

  - The addition of a specific section for “Co-existence studies for 1UL band with 1CC”

    - UL harmonic and harmonic mixing tables are updated in a matrix form with additional guidelines as discussed in [4]

    - A new calculation table for cross-band isolation MSD is added, as discussed in [5]

  - The addition of a specific section for “Co-existence studies for 1UL band with 2CC intra-band”

    - The IMD range table is updated and simplified as discussed in [6]

- For this meeting, the delta T/R, REFSENS and OOB exception sections are not covered, However, these may be part of further guidelines/proposals on how to design MSD test points.

- For the 2DL/1UL section:

  - Slightly updated 2DL 2UL with 1CC/band IMD table, with an analysis and Note section

  - Added section “5.XX.2.2.1	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)”, with a calculation table that includes an analysis and Note section, as discussed in [7]

- For the TR and/or TP, annexes may be added to provide further guidelines and calculations.

In future meetings, we may decide to propose further enhancements to the two DL band TP for other sections, most notably with a focus on guidelines for the design of the REFSENS test points related to the different MSD types.

Note that a defined part of the template is relevant to derive the one and three DL band templates:

- The new calculation table for cross-band isolation MSD added as discussed in [4], is relevant to the FDD intra-band DLCA with 1UL CC.

- The section for “Co-existence studies for 1UL band with 2CC intra-band” with the IMD range table is updated and simplified, as discussed in [5], is relevant to the FDD intra-band DLCA with intra-band ULCA.

- The 2DL/1UL section, with the updated 2DL 2UL with 1CC/band IMD table is relevant to the 3DL/2ULwith 1CC/band case.

- Similarly, the section “5.XX.2.2.1 Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)”, as discussed in [6], is relevant to the 3DL/2UL 3CC case.

## 2.2 Proposed template for 2DL/1or2UL block approval TPs

In the proposed template the fields to be completed by the proponent in are highlighted in grey, added parts are designated with blue text.

The note part of the table may become redundant for the different TPs captured in the TR. The rapporteur updating the TR may decide to omit the Note section of the tables when copying the TPs in the related TR.

************************* Start of template **********************************

## 5.XX		CA_nX-nY

### 5.XX.1	Common for 1 band UL and 2 bands UL CA

#### 5.XX.1.1 Operating bands for CA

Note: For band definition, relevant rows can be copied directly from Table 5.2-1 in 38.101-1 to Table 5.XX.1.1-1 below.

Table 5.XX.1.1-1: CA band combination of band nX+nY

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| nX | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |
| nY | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |

#### 5.XX.1.2	Channel bandwidths per operating band for CA

Table 5.XX.1.2-1: Supported bandwidths per CA band combination of band nX+nY

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_nXA-nYA | CA_n1A-n102A | nX | Channel BWs | X |
|  |  | nY | Channel BWs |  |
| CA_nX(A/B/C(2A)-nY(A/B/C(2A) | CA_nX(A/B/C(2A)-nY(A/B/C(2A) | nX | Channel BW or CA BCS | X |
|  |  | nY | Channel BW or CA BCS |  |

To determine the coexistence study cases to be analyzed, the following question and UL configuration types table should be completed. The allowable UL configurations are listed in annex B.

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? XXX

Table 5.XX.1.2-2: Supported UL configurations and required coexistence studies

| Type of UL Configuration | UL Configuration | Power class | Condition | Coexistence analysis to be performed | Coexistence study Tables |
| --- | --- | --- | --- | --- | --- |
| 1UL band with 1CC | nX, nY | 3 3 | One band is FDD Or SimRx/Tx TDD/TDD | UL harmonic, harmonic mixing and cross-band MSD should be studied | 5.XX.1.3.1-1 5.XX.1.3.1-2 |
| 1UL band with 2CC | CA_nXornYB/C/(2A) | 3 |  | IMDs of the two intra-band UL CCs | 5.XX.1.3.2-1 |
| 2UL bands 1CC per band | CA_nXA-nYA | 3 | FDD/FDD Or FDD/TDD | IMDs of the two UL bands | 5.XX.2.2.1-1 |
| 2UL bands incl. 1UL band with 2CC | CA_nXA/B/C-nYA/B/C | 3 3 |  | Triple beat of the three UL CCs if the two bands are in adjacent band groups | 5.XX.2.2.2-1 |

#### 5.XX.1.3	Co-existence studies

#### 5.XX.1.3.1	Co-existence studies for 1UL band with 1CC

Table 5.XX.1.3.1-1 summarizes frequency ranges of UL harmonics and/or harmonic mixing collisions may occur for CA_nX-nY.

Table 5.XX.1.3.1-1: UL/DL harmonics collision table for CA_nX-nY

| UL/DL harmonics |  | nX | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | fULlow | 2*fULlow | 3*fULlow | 4*fULlow | 5*fULlow |  |
| nY | fLow | fHigh | fULhigh | 2*fULhigh | 3*fULhigh | 4*fULhigh | 5*fULhigh |  |
| DL1 | fDLlow | fDLhigh | N/A |  |  |  |  | UL Harmonic |
| DL22 | 2*fDLlow | 2*fDLhigh |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL33 | 3*fDLlow | 3*fDLhigh |  |  | N/A |  | N/A |  |
| DL4 | 4*fDLlow | 4*fDLhigh |  | N/A | N/A | N/A | N/A |  |
| DL53 | 5*fDLlow | 5*fDLhigh |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| UL/DL harmonics |  | nY | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
|  |  | fLow | fULlow | 2*fULlow | 3*fULlow | 4*fULlow | 5*fULlow |  |
| nX | fLow | fhigh | fULhigh | 2*fULhigh | 3*fULhigh | 4*fULhigh | 5*fULhigh |  |
| DL1 | fDLlow | fDLhigh | N/A |  |  |  |  | UL Harmonic |
| DL22 | 2*fDLlow | 2*fDLhigh |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL33 | 3*fDLlow | 3*fDLhigh |  |  | N/A |  | N/A |  |
| DL4 | 4*fDLlow | 4*fDLhigh |  | N/A | N/A | N/A | N/A |  |
| DL53 | 5*fDLlow | 5*fDLhigh |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | text |  |  |  |  |  |
| Note 1: When a collision is detected with an overlap >0Hz between the UL(X) with DL(Y) frequency ranges, the UL(X)/DL(Y) cell is marked “D” for direct hit. When the gap between UL(X) and DL(Y) frequency range is from 0Hz to X*MinULCBW, the UL(X)/DL(Y) cell is marked “N” for Near miss. Note 2: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 3: For harmonic mixing, near-miss cases only apply for UL1 and odd DL orders. |  |  |  |  |  |  |  |  |

Table 5.XX.1.3.1-2 summarizes frequency ranges where cross-band isolation issues may occur for CA_nX-nY.

Table 5.XX.1.3.1-2: Cross-band isolation analysis of CA_nXA-nYA with nX and nY UL

| Bands | nX |  | nY |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) |  |  |  |  |
| fDL (MHz) |  |  |  |  |
| CBW (MHz)2 |  |  |  |  |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) |  |  |  |  |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) |  |  |  |  |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) |  |  |  |  |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) |  |  |  |  |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) |  |  |  |  |
| Analysis | text |  |  |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: -The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in annex Y -If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest harmonic 2 frequency -As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.XX.1.3.2	Co-existence studies for 1UL band with 2CC intra-band

Table 5.XX.1.3.2-1 provides the one UL band with intra-band ULCA IMD frequency ranges for the CA_nX/YB/C/(2A) UL configuration for DL CA_nX-nY.

Table 5.XX.1.3.2-1: Intra-band ULCA IMD overlap for CA_nXA/B/C/(2A)-nYA/B/C/(2A)/CA_nX/YB/C/(2A) DL/UL

| All in MHz | flow/Min | fhigh/Max | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| fUL |  |  | Order | flow | fhigh |
| 2CCBW1 |  |  | IMD2 (1-1) | Min2CCBW | Max2CCBW |
| fDL |  |  |  |  |  |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  |  |  |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  |  |  | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  |  |  | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
| Analysis | text |  |  |  |  |
| Note 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) Note 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. Note 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) -IMD2 is not considered assuming CA with 450MHz bands is not considered -IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz -IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz Note 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

*************** Delta T/R and 2DL/1UL REFSENS sections omitted in this version of the proposal ****************

### 5.XX.2		Specific for 2 bands UL CA

#### 5.XX.2.1	Maximum output power for inter-band CA

Table 5.XX.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_nXA-nYA | 23 | +2/-3 |
| CA_nX/YA/B/C/(2A) | 23 | +2/-3 |
| CA_nXA/B/C-nXA/B/C | 23 | +2/-3 |

#### 5.XX.2.2		UE co-existence

#### 5.XX.2.2.1	Co-existence studies for 2UL band with 1CC per band

Table 5.XX.2.2.1-1 provides the two UL bands with one CC per band IMD interference analysis for CA_ nXA-nYA.

Table 5.XX.2.2-1: IMD analysis

| Bands | nX |  | nY |  |
| --- | --- | --- | --- | --- |
| Frequency | fx_low | fx_high | fy_low | fy_high |
| UL (MHz) |  |  |  |  |
| DL (MHz) |  |  |  |  |
| IMD2 products | ¦fyULlow – fxULhigh¦ | ¦fyULhigh – fxULlow¦ | ¦fyULlow + fxULlow¦ | ¦fyULhigh + fxULhigh¦ |
| IMD2 (MHz) |  |  |  |  |
| IMD3 products | ¦2*fxULlow – fyULhigh¦ | ¦2*fxULhigh – fyULlow¦ | ¦2*fyULlow – fxULhigh¦ | ¦2*fyULhigh – fxULlow¦ |
| IMD3 (MHz) |  |  |  |  |
| IMD3 products | ¦2*fxULlow + fyULlow¦ | ¦2*fxULhigh + fyULhigh¦ | ¦2*fyULlow + fxULlow¦ | ¦2*fyULhigh + fxULhigh¦ |
| IMD3 (MHz) |  |  |  |  |
| IMD4 products | ¦3*fxULlow –1* fyULhigh¦ | ¦3*fxULhigh – 1*fyULlow¦ | ¦3*fyULlow – 1*fxULhigh¦ | ¦3*fyULhigh – 1*fxULlow¦ |
| IMD4 (MHz) |  |  |  |  |
| IMD4 products | ¦2*fxULlow –2* fyULhigh¦ | ¦2*fxULhigh –2* fyULlow¦ | ¦2*fxULlow +2* fyULlow¦ | ¦2*fxULhigh +2* fyULhigh¦ |
| IMD4 (MHz) |  |  |  |  |
| IMD4 products | ¦3*fxULlow +1* fyULlow¦ | ¦3*fxULhigh + 1*fyULhigh¦ | ¦3*fyULlow + 1*fxULlow¦ | ¦3*fyULhigh + 1*fxULhigh¦ |
| IMD4 (MHz) |  |  |  |  |
| IMD5 products | ¦fxULlow – 4*fyULhigh¦ | ¦fxULhigh – 4*fyULlow¦ | ¦fyULlow – 4*fxULhigh¦ | ¦fyULhigh – 4*fxULlow¦ |
| IMD5 (MHz) |  |  |  |  |
| IMD5 products | ¦2*fxULlow - 3*fyULhigh¦ | ¦2*fxULhigh - 3*fyULlow¦ | ¦2*fyULlow - 3*fxULhigh¦ | ¦2*fyULhigh -3*fxULlow¦ |
| IMD5 (MHz) |  |  |  |  |
| IMD5 products | ¦fxULlow + 4*fyULlow¦ | ¦fxULhigh + 4*fyULhigh¦ | ¦fyULlow + 4*fxULlow¦ | ¦fyULhigh + 4*fxULhigh¦ |
| IMD5 (MHz) |  |  |  |  |
| IMD5 products | ¦2*fxULlow + 3*fyULlow¦ | ¦2*fxULhigh + 3*fyULhigh¦ | ¦2*fyULlow + 3*fxULlow¦ | ¦2*fyULhigh + 3*fxULhigh¦ |
| IMD5 (MHz) |  |  |  |  |
| Analysis | text |  | text |  |
| Note: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

#### 5.XX.2.2.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.XX.2.2.2-1 provides the two UL band with one band, along with 2CC intra-band uplink CA triple beat products interference analysis for CA_nXA/B/C-nYA/B/C with nX/YB/C transmitting with a XXXMHz maximum instantaneous bandwidth.

Table 5.XX.2.2.2-1: Triple beat IMD analysis of CA_nXA/B/C-nYA/B/C

| Band / CA1 | nX |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| F_UL (MHz)3 |  |  |  |  |
| F_DL (MHz)3 |  |  |  |  |
| 2CCBW (MHz)2 | N/A | N/A |  |  |
| IMD3 products | fxULlow-max2CCBW | fxULlow | fxULhigh | fxULhigh+max2CCBW |
| IMD3 (MHz) |  |  |  |  |
| Analysis | text |  |  |  |
| Note 1: If the two bands are not part of the same or adjacent band groups as defined in Annex D, the analysis can be ignored. Note 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

*********************REFSENS section omitted in this version of the template – end of template*********

## 2.2 proposed annexes for TR and/or TPs

Since the band group definition is used as criteria for cross-band isolation MSD and Triple beat MSD.

********************* Start of annex *********

Annex X

For some MSD cases, a criterion applies in terms of band group as defined in Table B:

- For 2DL 1UL/1CC cross band isolation, and for the Tx noise floor MSD to be considered, the two bands should be part of the same or adjacent band group.

- For 2DL 2UL/3CC triple beat MSD to be considered, the two bands should be part of the same or adjacent band group

- For 3DL 2UL/3CC triple beat MSD to be considered, the third band should be part of the same or adjacent band group, rather than one from the UL band.

Table X: Band group definition for same or adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-a (LB) | FR1-b (MB) | FR1-c (HB) | FR1-d (VHB) | FR1-e (UHB) |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5250-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

********************* End of annex *********

## 2.2 Example of a resulting TP for CA_n1-n102

The chosen example is based on TP [7] on CA_n1-n102 and DC_n1-n102, as it contains all of the different types of UL configurations and can be considered to be fairly complete in terms of MSD studies. Although it does not result in an actual MSD specified for all the MSD types, it provides a good example of how the proposed template could function for a concrete case.

Some of the text is color coded with highlights to provide indication on how to derive the information to be filled in:

- Yellow highlights correspond to data related to the bands definitions

- Cyan highlights correspond to data related to the BCS DL, UL, channel bandwidths, and intra-band CA BW from the BCS definitions

- Green highlights correspond to data that can be calculated with the equations provided

- Pink highlights correspond to text that is provided in support of the analysis of the table results.

- When a part of the table is not relevant for a given case, the cell is with a grey background.

Some commentary text can be further added below each table. In this example, we did not retain the Note section of the tables as this will not be need duplication in the TR, but in TPs this information may be a useful reference for the TP reviewers during the flagging process.

************************* Start of example TP **********************************

## 5.XX		CA_n1-n102

### 5.XX.1	Common for 1 band UL and 2 bands UL CA

#### 5.XX.1.1 Operating bands for CA

Note: For band definition, relevant rows can be copied directly from Table 5.2-1 in 38.101-1 to Table 5.XX.1.1-1 below.

Table 5.XX.1.1-1: CA band combination of band n1+n102

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n102 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | TDD |

#### 5.XX.1.2	Channel bandwidths per operating band for CA

Table 5.XX.1.2-1: Supported bandwidths per CA band combination of band n1+n102

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n102A | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n102(2A) | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n1A-n102B | CA_n1A-n102A CA_n1A-n102B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n1A-n102C | CA_n1A-n102A CA_n1A-n102C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n1A-n102D | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n1A-n102E | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |

To determine the coexistence study cases to be analyzed, the following question and UL configuration types table should be completed. The allowable UL configurations are listed in annex B.

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)? N-A

Table 5.XX.1.2-2: Supported UL configurations and required coexistence studies

| Type of UL Configuration | UL Configuration | Power class | Condition | Coexistence analysis to be performed | Coexistence study Tables |
| --- | --- | --- | --- | --- | --- |
| 1UL band with 1CC | n1, n102 | 3 3 | One band is FDD Or SimRx/Tx TDD/TDD | UL harmonic, harmonic mixing and cross-band MSD should be studied | 5.XX.1.3.1-1 5.XX.1.3.1-2 |
| 1UL band with 2CC | CA_n102B, CA_n102C in CA_n1A-n102B, CA_n1A-n102C | 3 |  | IMDs of the two intra-band UL CCs | 5.XX.1.3.2-1 |
| 2UL bands 1CC per band | CA_n1A-n102A | 3 | FDD/FDD Or FDD/TDD | IMDs of the two UL bands | 5.XX.2.2.1-1 |
| 2UL bands incl. 1UL band with 2CC | CA_n1A-n102B CA_n1A-n102C | 3 3 |  | Triple beat of the three UL CCs, if the two bands are in adjacent band groups | 5.XX.2.2.2-1 |

#### 5.XX.1.3	Co-existence studies

#### 5.XX.1.3.1	Co-existence studies for 1UL band with 1CC

Table 5.XX.1.3.1-1 summarizes frequency ranges of UL harmonics and/or harmonic mixing collisions may occur for CA_n1-n102.

Table 5.XX.1.3.1-1: UL/DL harmonics collision table for CA_n1-n102

| UL/DL harmonics |  | n102 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 5925 | 11850 | 17775 | 23700 | 29625 |  |
| n1 | fLow | fHigh | 6425 | 12850 | 19275 | 25700 | 32125 |  |
| DL1 | 2110 | 2170 | N/A | - | - | - | - | UL Harmonic |
| DL2 | 4220 | 4340 | - | N/A | - | N/A | N/A | Harmonic Mixing |
| DL3 | 6330 | 6510 | D | - | N/A | - | N/A |  |
| DL4 | 8440 | 8680 | - | N/A | N/A | N/A | N/A |  |
| DL5 | 10550 | 10850 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Direct hit n102 UL1 / n1 DL3 |  |  |  |  |  |
| UL/DL harmonics |  | n1 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
|  |  | fLow | 1920 | 3840 | 5760 | 7680 | 9600 |  |
| n102 | fLow | fHigh | 1980 | 3960 | 5940 | 7920 | 9900 |  |
| DL1 | 5925 | 6425 | N/A | - | N | - | - | UL Harmonic |
| DL2 | 11850 | 12850 | - | N/A | - | N/A | N/A | Harmonic Mixing |
| DL3 | 17775 | 19275 | - | - | N/A | - | N/A |  |
| DL4 | 23700 | 25700 | - | N/A | N/A | N/A | N/A |  |
| DL5 | 29625 | 32125 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Near miss n1 UL3 / n102 DL1 (5MHz) |  |  |  |  |  |

Based on the above table:

- n1 3rd UL harmonic falls 5MHz short of the band n102 DL, and thus may be victim of the ACLR1 range of the third harmonic. Given that n102 is a shared spectrum access band, a REFSENS exclusion range can be defined.

- n1 3rd DL harmonic mixing falls on the band n102 UL, thus harmonic mixing MSD should be considered.

Table 5.XX.1.3.1-2 summarizes frequency ranges where cross-band isolation issues may occur for CA_n1-n102.

Table 5.XX.1.3.1-2: Cross-band isolation analysis of CA_n1A-n102A with n1 and n102 UL

| Bands3 | n1 |  | n102 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 1920 | 1980 | 5925 | 6425 |
| fDL (MHz) | 2110 | 2170 | 5925 | 6425 |
| CBW (MHz)2 | 5 | 50 | 20 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1870 | 2020 | 5825 | 6525 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1820 | 2070 | 5725 | 6625 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1770 | 2120 | 5625 | 6725 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1720 | 2170 | 5525 | 6825 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1670 | 2220 | 5425 | 6925 |
| Analysis | There is no overlap of the band n1 ACLR range with the band n102 DL up to order 5 and there is sufficient rejection of the transmitter noise floor at band n102 so no cross-band MSD is needed. |  | There is no overlap of the band n102 ACLR range with the band n1 DL up to order 5 and there is sufficient rejection of the transmitter noise floor at band n1 so no cross-band MSD is needed. |  |

Based on the above table: There is no case with the UL band ACLR range overlapping with the DL range and the band 1 and band n102 UL filters, together with the diplexer isolation, provides sufficient rejection of the other band DL frequency range and justifies the conclusion to ignore the UL noise floor impact.

5.XX.1.3.2	Co-existence studies for 1UL band with 2CC intra-band

Table 5.XX.1.3.2-1 provides the one UL band with intra-band ULCA IMD frequency ranges for the CA_n102B and CA_n102C UL configurations, which are part of CA_n1-n102B and CA_n1-n102C requested cases.

Table 5.XX.1.3.2-1: Intra-band ULCA IMD overlap for CA_n1A-n102C/CA_n102C DL/UL

| All in MHz | flow/Min | fhigh/Max | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| fUL5 | 5925 | 6425 | Order | flow | fhigh |
| 2CCBW1 | 0 | 160 | IMD2 (1-1) | Min2CCBW | Max2CCBW |
| fDL | 2110 | 2170 |  | 0 | 160 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 0 | 320 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 5765 | 6585 |  | 0 | 480 |
| IMD5 (3-2) | fULlow-2*Max2CCBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 5605 | 6745 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 5445 | 6905 |  | 11690 | 13010 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 5285 | 7065 |  | 11530 | 13170 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 5125 | 7225 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 4965 | 7385 |  | 17615 | 19435 |
| Analysis | The DL victim band is lower than the TDD intra-band ULCA band, which is contiguous ULCA. Thus, only the close n102C UL IMD3 to 9 should be considered, and there is no overlap with the DL band n1 |  |  |  |  |

Since this is a contiguous ULCA TDD case and the UL band is above the DL band, only up to IMD9 is considered for close to UL products while BB and close to harmonic products can be ignored. Based on the above table: There is no case with the 2CC intra-band IMD range overlapping with the DL range of band 1.

********* Delta T/R, 2DL/1UL REFSENS and OOB exception sections omitted in this version of the example *********

### 5.XX.2		Specific for 2 bands UL CA

#### 5.XX.2.1	Maximum output power for inter-band CA

Table 5.XX.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n102A | 23 | +2/-3 |
| CA_n1A-n102B | 23 | +2/-3 |
| CA_n1A-n102C | 23 | +2/-3 |

#### 5.XX.2.2		UE co-existence

#### 5.XX.2.2.1	Co-existence studies for 2UL band with 1CC per band

Table 5.XX.2.2.1-1 provides the two UL bands, with one CC per band IMD interference analysis for CA_ n1A-n102A.

Table 5.XX.2.2-1: IMD analysis

| Bands | n1 |  | n102 |  |
| --- | --- | --- | --- | --- |
| Frequency | fx_low | fx_high | fy_low | fy_high |
| UL (MHz) | 1920 | 1980 | 5925 | 6425 |
| DL (MHz) | 2110 | 2170 | 5925 | 6425 |
| IMD2 products | ¦fyULlow – fxULhigh¦ | ¦fyULhigh – fxULlow¦ | ¦fyULlow + fxULlow¦ | ¦fyULhigh + fxULhigh¦ |
| IMD2 (MHz) | 3945 | 4505 | 7845 | 8405 |
| IMD3 products | ¦2*fxULlow – fyULhigh¦ | ¦2*fxULhigh – fyULlow¦ | ¦2*fyULlow – fxULhigh¦ | ¦2*fyULhigh – fxULlow¦ |
| IMD3 (MHz) | 2585 | 1965 | 9870 | 10930 |
| IMD3 products | ¦2*fxULlow + fyULlow¦ | ¦2*fxULhigh + fyULhigh¦ | ¦2*fyULlow + fxULlow¦ | ¦2*fyULhigh + fxULhigh¦ |
| IMD3 (MHz) | 9765 | 10385 | 13770 | 14830 |
| IMD4 products | ¦3*fxULlow –1* fyULhigh¦ | ¦3*fxULhigh – 1*fyULlow¦ | ¦3*fyULlow – 1*fxULhigh¦ | ¦3*fyULhigh – 1*fxULlow¦ |
| IMD4 (MHz) | 665 | 15 | 15795 | 17355 |
| IMD4 products | ¦2*fxULlow –2* fyULhigh¦ | ¦2*fxULhigh –2* fyULlow¦ | ¦2*fxULlow +2* fyULlow¦ | ¦2*fxULhigh +2* fyULhigh¦ |
| IMD4 (MHz) | 9010 | 7890 | 15690 | 16810 |
| IMD4 products | ¦3*fxULlow +1* fyULlow¦ | ¦3*fxULhigh + 1*fyULhigh¦ | ¦3*fyULlow + 1*fxULlow¦ | ¦3*fyULhigh + 1*fxULhigh¦ |
| IMD4 (MHz) | 11685 | 12365 | 19695 | 21255 |
| IMD5 products | ¦fxULlow – 4*fyULhigh¦ | ¦fxULhigh – 4*fyULlow¦ | ¦fyULlow – 4*fxULhigh¦ | ¦fyULhigh – 4*fxULlow¦ |
| IMD5 (MHz) | 23780 | 21720 | 1995 | 1255 |
| IMD5 products | ¦2*fxULlow - 3*fyULhigh¦ | ¦2*fxULhigh - 3*fyULlow¦ | ¦2*fyULlow - 3*fxULhigh¦ | ¦2*fyULhigh -3*fxULlow¦ |
| IMD5 (MHz) | 15435 | 13815 | 5910 | 7090 |
| IMD5 products | ¦fxULlow + 4*fyULlow¦ | ¦fxULhigh + 4*fyULhigh¦ | ¦fyULlow + 4*fxULlow¦ | ¦fyULhigh + 4*fxULhigh¦ |
| IMD5 (MHz) | 25620 | 27680 | 13605 | 14345 |
| IMD5 products | ¦2*fxULlow + 3*fyULlow¦ | ¦2*fxULhigh + 3*fyULhigh¦ | ¦2*fyULlow + 3*fxULlow¦ | ¦2*fyULhigh + 3*fxULhigh¦ |
| IMD5 (MHz) | 21615 | 23235 | 17610 | 18790 |
| Analysis | IMD3 fall in FDD band n1 DL: MSD may occur |  | IMD5 fall in TDD band n102 DL: no MSD possible |  |

Based on the table 5.XX.2.2.1-1, one 3rd order intermodulation product fall inside the DL band of n1.

#### 5.XX.2.2.2	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)

Table 5.XX.2.2.2-1 provides the two UL bands with one band, with a 2CC intra-band uplink CA triple beat products interference analysis provided for CA_n1A-n102C including n102C transmitting with a 160MHz maximum instantaneous bandwidth.

Table 5.XX.2.2.2-1: Triple beat IMD analysis of CA_n1A-n102C

| Band / CA1 | n1 |  | CA_n102C |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| F_UL (MHz)3 | 1920 | 1980 | 5925 | 6425 |
| F_DL (MHz)3 | 2110 | 2170 | 5925 | 6425 |
| 2CCBW (MHz)2 | N/A | N/A | 0 | 160 |
| IMD3 products | fx_low-max2CCBW | fx_low | fx_high | fx_high+max2CCBW |
| IMD3 (MHz) | 1760 | 1920 | 1980 | 2140 |
| Analysis | There is a triple beat product of n1 UL with n102C 2CC UL falling in n1 DL. However, n1 and n102 are not part of adjacent band groups and thus MSD is not required. |  |  |  |

The uplinks of n1A and n102C have a 1st order triple beat falling inside n1 DL. As stated for figure 1 in [2] the triple beat case is not eligible for MSD if the two UL bands aren’t part of the same or adjacent band group.

*********************REFSENS section omitted in this version of the example *********

************************* End of example TP **********************************

# Conclusion

In this contribution, we presented an example of improved template for the block approval TPs for two band DL cases and have the following proposals.

Proposed enhancements for 2DL/1or2UL bands block approval TP template for Release 19:

- Addition at the end of section “5.XX.1.2 Channel bandwidths per operating band for CA” of:

  - A question related to the support of SimRx/Tx, or otherwise for TDD/TDD cases.

  - A table that sorts the applicable UL configuration and their related MSD studies

- For the 2DL/1UL section:

  - The addition of a specific section for “Co-existence studies for 1UL band with 1CC”

    - UL harmonic and harmonic mixing tables are updated in a matrix form with additional guidelines as discussed in [4]

    - A new calculation table for cross-band isolation MSD is added, as discussed in [5]

  - The addition of a specific section for “Co-existence studies for 1UL band with 2CC intra-band”

    - The IMD range table is updated and simplified as discussed in [6]

- For this meeting, the delta T/R, REFSENS and OOB exception sections are not covered, However, these may be part of further guidelines/proposals on how to design MSD test points.

- For the 2DL/1UL section:

  - Slightly updated 2DL 2UL with 1CC/band IMD table, with an analysis and Note section

  - Added section “5.XX.2.2.1	Co-existence studies for 2UL band with 3CC (2CC intra-band in one band)”, with a calculation table that includes an analysis and Note section, as discussed in [7]

- For the TR and/or TP, annexes may be added to provide further guidelines and calculations.

# References

[1] R4-2400257 Further improvements to the block approval process in R19, Skyworks Solutions Inc., Nokia, RAN4#110

[2] R4-2403721 WF on Rel-19 Band Combination Work, Nokia, Skyworks, ZTE, Samsung, Ericsson, AT&T, Qualcomm, CHTTL, Murata, T-Mobile USA, Apple, MediaTek, Huawei, Verizon, RAN4#110

[3] R4-2404244 On valid harmonic mixing orders, Skyworks Solutions Inc., Nokia, Qualcomm, RAN4#110bis

[4] R4-2404243 Improved harmonic related MSDs template, Skyworks Solutions Inc., Nokia, RAN4#110bis

[5] R4-2404250 On cross band isolation MSD analysis, Skyworks Solutions Inc., Nokia, Murata, RAN4#110bis

[6] R4-2404249 On simplifying analysis for 2DL-1 band intra-band ULCA IMD products, Skyworks Solutions Inc., Nokia, Murata, RAN4#110bis

[7] R4-2404248 On simplifying analysis for triple beat products, Skyworks Solutions Inc., Nokia, Murata, RAN4#110bis