3GPP TSG-RAN4 Meeting # 110-bis	R4-2405555

# Changsha, China,  15 – 19 April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.181 | CR | xxxx | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for TS 38.181: Introduction of FR2-NTN in Clause 4.1 Measurement uncertainties and test requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of FR2-NTN and SAN Type 2-O in TS 38.108 implies update of TS 38.181 with similar features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of MU and TT values for FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specifications do not contain MU values for FR2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1, 4.1.2, 4.1.3, Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## -------- start of change ---------

## 4.1	Measurement uncertainties and test requirements

### 4.1.1	General

The requirements of this clause apply to all applicable tests in TS 38.181(the present document), i.e. to all conducted tests defined in FR1-NTN for SAN type 1-H, radiated tests defined in FR1-NTN for SAN type 1-H and SAN type 1-O, radiated tests defined in FR2-NTN for SAN Type 2-O. The frequency ranges FR1-NTN and FR2-NTN are defined in clause 5.1 of TS 38.108 [2].

The minimum requirements are given in TS 38.108 [2]. Test Tolerances for the conducted test requirements (TT) and test Tolerances for the radiated test requirements (TTOTA) explicitly stated in the present document are given in annex C of the present document.

The test tolerances for the radiated test requirements (TTOTA) for SAN were reused from TR 37.941 [13]. Reuse of TR 37.941 [13] TTOTA values for SAN LEO radiated conformance testing is subject to the following conditions:

-	EUT suitability to fit OTA chambers considered in TR 37.941 [13], and

-	Environmental test conditions assumed for BS testing in TR 37.941 [13].

Reuse of TR 37.941 [13] TTOTA values for SAN GEO radiated conformance testing may not be justified for some products due to too large SAN GEO antenna array dimensions, and required OTA RF chamber size.

Test Tolerances are individually calculated for each test. Test Tolerances are used to relax the minimum requirements to create test requirements.

When a test requirement differs from the corresponding minimum requirement, then the Test Tolerance applied for the test is non-zero. The Test Tolerance for the test and the explanation of how the minimum requirement has been relaxed by the Test Tolerance are given in annex C.

The radiated requirements are classified according to spatial characteristics as shown in table 4.1.1-1 and table 4.1.1-2.

Table 4.1.1-1: Overview of radiated Tx requirements

| Tx requirement |  |  | Classification | Coverage range |  | Number of |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FR1-NTN | FR2-NTN | conformance directions |
| Radiated transmit power |  |  | Directional | OTA peak directions set | OTA peak directions set | 5 |
| OTA Satellite Access Node output power |  |  | TRP | N/A | See TS 38.141-2 [18], annex I | See TS 38.141-2 [18], annex I |
| OTA output power dynamics |  |  | Directional | OTA peak directions set | OTA peak directions set | 1 |
| OTA modulation quality |  |  | Directional | OTA coverage range | OTA coverage range | 5 |
| OTA frequency error |  |  | Directional | OTA coverage range | OTA coverage range | 1 |
| OTA occupied bandwidth |  |  | Directional | OTA coverage range | OTA coverage range | 1 |
| OTA ACLR |  |  | TRP | N/A | N/A | See TS 38.141-2 [18], annex I |
| OTA out-of-band emissions |  |  | TRP | N/A | N/A | See TS 38.141-2 [18], annex I |
| OTA transmitter spurious emission | General requirement |  | TRP | N/A | N/A | See TS 38.141-2 [18], annex I |
|  |  | NOTE: Directional requirement does not imply one compliance direction only. The directional requirement applies to a single direction at a time. |  |  |  |  |

Table 4.1.1-2: Overview of radiated Rx requirements

| Rx requirement |  | Classification | Applicability levels |  | Coverage range |  | Number of |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FR1-NTN | FR2-NTN | FR1-NTN | FR2-NTN | conformance directions |
| OTA sensitivity |  | Directional | Minimum EIS | N/A | OSDD | N/A | 5 |
| OTA reference sensitivity level |  | Directional | OTA REFSENS | OTA REFSENS | OTA REFSENS RoAoA | OTA REFSENS RoAoA | 5 |
| OTA Dynamic range |  | Directional | OTA REFSENS | N/A | OTA REFSENS RoAoA | N/A | 1 |
| OTA adjacent channel selectivity |  | Directional | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| OTA out-of-band blocking | General requirement | Directional | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| OTA in-channel selectivity |  | Directional | minSENS | OTA REFSENS | minSENS RoAoA | OTA REFSENS RoAoA | 1 |
| NOTE: Directional requirement does not imply one compliance direction only. The directional requirement applies to a single direction at a time. |  |  |  |  |  |  |  |

### 4.1.2	Acceptable uncertainty of Test System

#### 4.1.2.1	General

The maximum acceptable uncertainty of the Conducted Test System and OTA Test System are specified below for each test defined explicitly in the present specification, where appropriate. The maximum acceptable uncertainty of the Test System for test requirements included by reference is defined in the respective referred test specification.

The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified tolerance and the equipment under test to be measured with an uncertainty not exceeding the specified values. All tolerances and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

For conducted RF tests, it should be noted that the uncertainties in clause 4.1.2 apply to the Test System operating into a nominal 50 ohm load and do not include system effects due to mismatch between the EUT and the Test System.

For details on measurement uncertainty budget calculation, measurement methodology description (including calibration and measurement stage for each test range), MU budget format and its contributions, refer to TR 37.941 [13], where MU analyses for the BS radiated testing were captured. The maximum OTA Test System uncertainty for FR1-NTN and FR2-NTN OTA transmitter and receiver tests were reused from BS MU budgets in TR 37.941 [13]. Reuse of TR 37.941 [13] MU values for SAN LEO radiated conformance testing is subject to the following conditions:

-	EUT suitability to fit OTA chambers considered in TR 37.941 [13], and

-	Environmental test conditions assumed for BS testing in TR 37.941 [13].

Reuse of TR 37.941 [13] TTOTA values for SAN GEO radiated conformance testing may not be justified for some products due to too large SAN GEO antenna array dimensions, and required OTA RF chamber size.

#### 4.1.2.2	Measurement of transmitter

The maximum conducted Test System uncertainty for conducted transmitter tests minimum requirements is given in table 4.1.2.2-1. The maximum OTA Test System uncertainty for OTA transmitter tests minimum requirements is given in table 4.1.2.2-2 for FR1-NTN and in table 4.1.2.2-3 fro FR2-NTN.

Table 4.1.2.2-1: Maximum Test System uncertainty for conducted transmitter tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 SAN output power | ±0.7 dB, f ≤ 3 GHz |  |
| 6.3 Output power dynamics | ± 0.4 dB |  |
| 6.5.2 Frequency error | ± 12 Hz |  |
| 6.5.3 EVM | ± 1% |  |
| 6.6.2 Occupied bandwidth | 5 MHz, 10 MHz SAN Channel BW: ±100 kHz 15 MHz, 20 MHz SAN Channel BW: ±300 kHz |  |
| 6.6.3 Adjacent Channel Leakage power Ratio (ACLR) | ACLR BW ≤ 20MHz: ±0.8 dB |  |
| 6.6.4 Out-of-band emissions | ±1.5 dB, f ≤ 3 GHz |  |
| 6.6.5.5.1.1 Transmitter spurious emissions, Mandatory Requirements | 9 kHz < f ≤ 4 GHz: ±2.0 dB 4 GHz < f ≤ 15 GHz: ±4.0 dB |  |
| 6.6.5.5.1.2 Transmitter spurious emissions, Protection of SAN receiver | ±3.0 dB |  |

Table 4.1.2.2-2: Maximum OTA Test System uncertainty for FR1-NTN OTA transmitter tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 9.2 Radiated transmit power | Normal condition: [±1.1] dB, f ≤ 3 GHz |
| 9.3 OTA base station output power | [±1.4] dB, f ≤ 3.0 GHz |
| 9.4.3 OTA total power dynamic range | [±0.4] dB |
| 9.6.2 OTA frequency error | [±12] Hz |
| 9.6.3 OTA modulation quality | [±1 %] |
| 9.7.2 OTA occupied bandwidth | [±100] kHz, BWChannel 5 MHz, 10 MHz [±300] kHz, BWChannel 15 MHz, 20 MHz |
| 9.7.3 OTA ACLR | f ≤ 3.0 GHz [±1] dB, BW ≤ 20MHz |
| 9.7.4 OTA out-of-band emissions | Absolute power [±1.8] dB, f ≤ 3.0 GHz |
| 9.7.5.2 OTA transmitter spurious emissions, mandatory requirements | [±2.3] dB, 30 MHz < f ≤ 6 GHz [±4.2] dB, 6 GHz < f ≤ 15 GHz |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

Table 4.1.2.2-3: Maximum OTA Test System uncertainty for FR2-NTN OTA transmitter tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 9.2 Radiated transmit power | Normal condition: [±1.7] dB |
| 9.3 OTA base station output power | [±2.1] dB |
| 9.4.3 OTA total power dynamic range | [±0.4] dB |
| 9.6.2 OTA frequency error | [±12] Hz |
| 9.6.3 OTA modulation quality | [±1 %] |
| 9.7.2 OTA occupied bandwidth | 600 kHz |
| 9.7.3 OTA ACLR | [±2.3] dB |
| 9.7.4 OTA out-of-band emissions | Absolute power [±2.7] dB |
| 9.7.5.2 OTA transmitter spurious emissions, mandatory requirements | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz ±5.3dB, 60 GHz < f ≤ 110 GHz ±5.9dB, 110 GHz < f ≤ 142 GHz |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

#### 4.1.2.3	Measurement of receiver

The maximum conducted Test System uncertainty for conducted receiver tests minimum requirements are given in table 4.1.2.3-1. The maximum OTA Test System uncertainty for OTA receiver tests minimum requirements are given in table 4.1.2.3-2 for FR1-NTN and in table 4.1.2.3-3 for FR2-NTN.

Table 4.1.2.3-1: Maximum Test System Uncertainty for conducted receiver tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.2 Reference sensitivity level | ±0.7 dB, f ≤ 3 GHz |  |
| 7.3 Dynamic range | ±0.3 dB |  |
| 7.4.1 Adjacent channel selectivity | ±1.4 dB, f ≤ 3 GHz | Overall system uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer leakage Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer leakage effect is systematic, and is added arithmetically. Test System uncertainty = SQRT (wanted_level_error2 + interferer_level_error2) + leakage effect. f ≤ 3 GHz Wanted signal level ±0.7 dB Interferer signal level ±0.7 dB f ≤ 6 GHz Impact of interferer leakage 0.4 dB |
| 7.5.5.1 Out-of-band blocking | fwanted ≤ 3GHz 1MHz < finterferer ≤ 3 GHz: ±1.3 dB 3 GHz < finterferer ≤ 4.2 GHz: ±1.5 dB 4.2GHz < finterferer ≤ 12.75 GHz: ±3.2 dB | Overall system uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = SQRT (wanted_level_error2 + interferer_level_error2) + Broadband noise effect. Out of band blocking, using CW interferer: Wanted signal level: ±0.7 dB up to 3 GHz Interferer signal level: ±1.0 dB up to 3 GHz Impact of interferer Broadband noise 0.1 dB |
| 7.8 In-channel selectivity | ±1.4 dB, f ≤ 3 GHz |  |
| NOTE: Unless otherwise noted, only the Test System stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. |  |  |

Table 4.1.2.3-2: Maximum OTA Test System uncertainty for FR1-NTN OTA receiver tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 10.2 OTA sensitivity | [±1.3] dB, f ≤ 3 GHz |
| 10.3 OTA reference sensitivity level | [±1.3] dB, f ≤ 3 GHz |
| 10.4 OTA dynamic range | [±0.3] dB |
| 10.5.1 OTA adjacent channel selectivity | [±1.7] dB, f ≤ 3 GHz |
| 10.6 OTA out-of-band blocking | fwanted ≤ 3 GHz: [±2.0] dB, finterferer ≤ 3 GHz [±2.1] dB, 3 GHz < finterferer ≤ 6 GHz [±3.5] dB, 6 GHz < finterferer ≤ 12.75 GHz |
| 10.9 OTA in-channel selectivity | [±1.7] dB, f ≤ 3 GHz |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

Table 4.1.2.3-3: Maximum OTA Test System uncertainty for FR2-NTN OTA receiver tests

| Clause | Maximum OTA Test System uncertainty |
| --- | --- |
| 10.3 OTA reference sensitivity level | [±2.4] dB |
| 10.5.1 OTA adjacent channel selectivity | [±3.4] dB |
| 10.6 OTA out-of-band blocking | [±3.6] dB |
| 10.9 OTA in-channel selectivity | [±3.4] dB |
| NOTE: Test system uncertainty values are applicable for normal condition unless otherwise stated. |  |

#### 4.1.2.4	Measurement of performance requirements

Table 4.1.2.4-1: Maximum Test System Uncertainty for FR1-NTN conducted performance requirements

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8 PUSCH, PUCCH, PRACH with single antenna port and fading channel | ± 0.6 dB | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8 PRACH with single antenna port and AWGN | ± 0.3 dB | Signal-to-noise ratio uncertainty ±0.3 dB |

Table 4.1.2.4-2: Maximum Test System Uncertainty for FR1-NTN OTA performance requirements

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 11 PUSCH, PUCCH, PRACH with single antenna port and fading channel | ± 0.6 dB | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 11 PRACH with single antenna port and AWGN | ± 0.3 dB | Signal-to-noise ratio uncertainty ±0.3 dB |

Table 4.1.2.4-3: Maximum Test System Uncertainty for FR2-NTN OTA performance requirements

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 11 PUSCH, PUCCH, PRACH with single antenna port and fading channel | ± 0.6 dB | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = [SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2)] Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 11 PRACH with single antenna port and AWGN | ± 0.3 dB | Signal-to-noise ratio uncertainty ±0.3 dB |

### 4.1.3	Interpretation of measurement results

## -------- end of change ---------

## -------- Start of change ---------

Annex C (informative):
Test tolerances and derivation of test requirements

The test requirements explicitly defined in this specification have been calculated by relaxing the minimum requirements of the core specification TS 38.108 [2] using the test tolerances (TT) defined here. When the TT value is zero, the test requirement will be the same as the minimum requirement. When the TT value is non-zero, the test requirements will differ from the minimum requirements, and the formula used for this relaxation is given in the following tables.

The TTOTA values are derived from OTA Test System uncertainties, regulatory requirements and criticality to system performance. As a result, the TTOTA values may sometimes be set to zero.

The TTOTA values should not be modified for any reason e.g. to take account of commonly known OTA Test System errors (such as mismatch, cable loss, etc.).

Note that a formula for applying TTOTA values is provided for all OTA tests, even those with a test tolerance of zero. This is necessary in the case where the OTA Test System uncertainty is greater than that allowed in clause 4.1.2. In this event, the excess error shall be subtracted from the defined TTOTA value in order to generate the correct tightened test requirements as defined in this annex.

# C.1	Measurement of transmitter

Table C.1-1: Derivation of test requirements for conducted transmitter tests

| Test | Minimum requirement in TS 38.108 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 SAN output power | clause 6.2 | Normal and extreme conditions: 0.7 dB | Formula: Upper limit + TT, Lower limit - TT |
| 6.3 Output power dynamics | clause 6.3 | 0.4 dB | Formula: Total power dynamic range – TT (dB) |
| 6.5.1 Frequency error | clause 6.5.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.5.2 Modulation quality (EVM) | clause 6.5.2 | 1% | Formula: EVM limit + TT |
| 6.6.2 Occupied bandwidth | clause 6.6.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 6.6.3 Adjacent Channel Leakage Power Ratio (ACLR) | clause 6.6.3 | ACLR: BW ≤ 20MHz: 0.8dB | Formula: ACLR Minimum Requirement - TT |
| 6.6.4 Out-of-band emissions | clause 6.6.4 | 0dB | Formula: Minimum Requirement + TT |
| 6.6.5 Transmitter spurious emissions | clause 6.6.5 | 0dB | Formula: Minimum Requirement + TT |
| NOTE 1: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

Table C.1-2: Derivation of test requirements for FR1-NTN OTA transmitter tests

| Test | Minimum requirement in TS 38.108 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 9.2 Radiated transmit power | clause 9.2 | Normal conditions: 1.1 dB Extreme conditions: 2.5 dB | Formula: Upper limit + TT, Lower limit – TT |
| 9.3 OTA SAN output power | clause 9.3 | 1.4 dB | Formula: Upper limit + TT, Lower limit – TT |
| 9.4 OTA output power dynamics | clause 9.4 | 0.4 dB | Formula: Total power dynamic range – TT |
| 9.6.1 OTA frequency Error | clause 9.6.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 9.6.2 OTA Modulation quality (EVM) | clause 9.6.2 | 1% | Formula: EVM limit + TT |
| 9.7.2 OTA occupied bandwidth | clause 9.7.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 9.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) | clause 9.7.3 | Relative:1 dB | Formula: Relative limit - TT |
| 9.7.4 OTA out-of-band emissions | clause 9.7.4 | 0 dB | Formula: Minimum Requirement + TT |
| 9.7.5.2 General transmitter spurious emissions | clause 9.7.5.2.2 | 0 dB | Formula: Minimum Requirement + TT |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

Table C.1-3: Derivation of test requirements for FR2-NTN OTA transmitter tests

| Test | Minimum requirement in TS 38.108 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 9.2 Radiated transmit power | clause 9.2 | Normal conditions: 1.7 dB Extreme conditions: 3.1 dB | Formula: Upper limit + TT, Lower limit – TT |
| 9.3 OTA SAN output power | clause 9.3 | 2.1 dB | Formula: Upper limit + TT, Lower limit – TT |
| 9.4 OTA output power dynamics | clause 9.4 | 0.4 dB | Formula: Total power dynamic range – TT |
| 9.6.1 OTA frequency Error | clause 9.6.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 9.6.2 OTA Modulation quality (EVM) | clause 9.6.2 | 1% | Formula: EVM limit + TT |
| 9.7.2 OTA occupied bandwidth | clause 9.7.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 9.7.3 OTA Adjacent Channel Leakage Power Ratio (ACLR) | clause 9.7.3 | Relative:2.3 dB | Formula: Relative limit - TT |
| 9.7.4 OTA out-of-band emissions | clause 9.7.4 | 0 dB | Formula: Minimum Requirement + TT |
| 9.7.5.2 General transmitter spurious emissions | clause 9.7.5.2.2 | 0 dB | Formula: Minimum Requirement + TT |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

# C.2	Measurement of receiver

Table C.2-1: Derivation of test requirements for conducted receiver tests

| Test | Minimum requirement in TS 38.108 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 7.2 Reference sensitivity level | clause 7.2 | Normal and extreme conditions: 0.7 dB | Formula: Reference sensitivity power level + TT |
| 7.3 Dynamic range | clause 7.3 | 0.3 dB | Formula: Wanted signal power + TT |
| 7.4 In-band selectivity and blocking | clause 7.4 | 0dB | Formula: Wanted signal power + TT |
| 7.5 Out-of-band blocking | clause 7.5 | 0dB | Formula: Wanted signal power + TT |
| 7.8 In-channel selectivity | clause 7.8 | 1.4 dB | Formula: Wanted signal power + TT |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

Table C.2-2: Derivation of test requirements for FR1-NTN OTA receiver tests

| Test | Minimum requirement in TS 38.108 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 10.2 OTA sensitivity | clause 10.2 | 1.3 dB | Formula: Declared Minimum EIS + TT |
| 10.3 OTA reference sensitivity level | clause 10.3 | 1.3 dB | Formula: EISREFSENS + TT |
| 10.4 OTA dynamic range | clause 10.4 | 0.3 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 10.5.1 OTA adjacent channel selectivity | clause 10.5.1 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 10.6 OTA out-of-band blocking | clause 10.6 | 0 dB | Formula: Wanted signal power + TT |
| 10.9 OTA in-channel selectivity | clause 10.9 | 1.7 dB | Formula: Wanted signal power + TT Interferer signal power unchanged |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

Table C.2-3: Derivation of test requirements for FR2-NTN OTA receiver tests

| Test | Minimum requirement in TS 38.108 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 10.3 OTA reference sensitivity level | clause 10.3 | 2.4 dB | Formula: EISREFSENS + TT |
| 10.5.1 OTA adjacent channel selectivity | clause 10.5.1 | 0 dB | Formula: Wanted signal power + TT Interferer signal power unchanged. |
| 10.6 OTA out-of-band blocking | clause 10.6 | 0 dB | Formula: Wanted signal power + TT |
| 10.9 OTA in-channel selectivity | clause 10.9 | 3.4 dB | Formula: Wanted signal power + TT Interferer signal power unchanged |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

# C.3	Measurement of performance requirements

Table C.3-1: Derivation of Test Requirements (Performance tests)

| Test | Minimum Requirement in TS 38.108 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 8.2.1 Performance requirements for PUSCH with transform precoding disabled | SNRs as specified | [0.6] dB for 1Tx cases | Formula: SNR + TT T-put limit unchanged |
| 8.2.2 Performance requirements for PUSCH with transform precoding enabled | SNRs as specified | [0.6] dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3 Performance requirements for UL timing adjustment | SNRs as specified | [0.3] dB for AWGN | Formula: SNR + TT T-put limit unchanged |
| 8.2.4 Performance requirements for PUSCH repetition Type A | SNRs as specified | [0.6] dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.1 Performance requirements for PUCCH format 0 | SNRs as specified | [0.6] dB | Formula: SNR + TT False ACK limit unchanged Correct ACK limit unchanged |
| 8.3.2 Performance requirements for PUCCH format 1 | SNRs as specified | [0.6] dB | Formula: SNR + TT False ACK limit unchanged False NACK limit unchanged Correct ACK limit unchanged |
| 8.3.3 Performance requirements for PUCCH format 2 | SNRs as specified | [0.6] dB | Formula: SNR + TT False ACK limit unchanged Correct ACK limit unchanged UCI BLER limit unchanged |
| 8.3.4 Performance requirements for PUCCH format 3 | SNRs as specified | [0.6] dB | Formula: SNR + TT UCI BLER limit unchanged |
| 8.3.5 Performance requirements for PUCCH format 4 | SNRs as specified | [0.6] dB | Formula: SNR + TT UCI BLER limit unchanged |
| 8.4.1 PRACH false alarm probability and missed detection | SNRs as specified | [0.6] dB for fading cases [0.3] dB for AWGN cases | Formula: SNR + TT PRACH false detection limit unchanged PRACH detection limit unchanged |

Table C.3-2: Derivation of test requirements (FR1-NTN OTA performance tests)

| Test | Minimum Requirement in TS 38.108 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 11.2.1 Performance requirements for PUSCH with transform precoding disabled | See clause 11.2.1.1 | [0.6] dB | Formula: SNR + TTOTA T-put limit unchanged |
| 11.2.2 Performance requirements for PUSCH with transform precoding enabled | See clause 11.2.1.2 | [0.6] dB | Formula: SNR + TTOTA T-put limit unchanged |
| 11.2.3 Performance requirements for UL timing adjustment | See clause 11.2.1.3 | [0.3] dB for AWGN cases | Formula: SNR + TTOTA T-put limit unchanged |
| 11.2.4 Performance requirements for PUSCH repetition Type A | See clause 11.2.1.4 | [0.6] dB | Formula: SNR + TTOTA BLER limit unchanged |
| 11.3.1 Performance requirements for PUCCH format 0 | See clause 11.3.1.1 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged |
| 11.3.2 Performance requirements for PUCCH format 1 | See clause 11.3.1.3 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged False NACK limit unchanged Correct ACK limit unchanged |
| 11.3.3 Performance requirements for PUCCH format 2 | See clause 11.3.1.4 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged UCI BLER limit unchanged |
| 11.3.4 Performance requirements for PUCCH format 3 | See clause 11.3.1.5 | [0.6] dB | Formula: SNR + TTOTA UCI BLER limit unchanged |
| 11.3.5 Performance requirements for PUCCH format 4 | See clause 11.3.1.6 | [0.6] dB | Formula: SNR + TTOTA UCI BLER limit unchanged |
| 11.4.1 PRACH false alarm probability and missed detection | See clause 11.4.1 | [0.6] dB for fading cases [0.3] dB for AWGN cases | Formula: SNR + TTOTA PRACH False detection limit unchanged PRACH detection limit unchanged |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

Table C.3-3: Derivation of test requirements (FR2-NTN OTA performance tests)

| Test | Minimum Requirement in TS 38.108 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 11.2.1 Performance requirements for PUSCH with transform precoding disabled | See clause 11.2.1.1 | [0.6] dB | Formula: SNR + TTOTA T-put limit unchanged |
| 11.2.2 Performance requirements for PUSCH with transform precoding enabled | See clause 11.2.1.2 | [0.6] dB | Formula: SNR + TTOTA T-put limit unchanged |
| 11.2.3 Performance requirements for UL timing adjustment | See clause 11.2.1.3 | [0.3] dB for AWGN cases | Formula: SNR + TTOTA T-put limit unchanged |
| 11.2.4 Performance requirements for PUSCH repetition Type A | See clause 11.2.1.4 | [0.6] dB | Formula: SNR + TTOTA BLER limit unchanged |
| 11.3.1 Performance requirements for PUCCH format 0 | See clause 11.3.1.1 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged |
| 11.3.2 Performance requirements for PUCCH format 1 | See clause 11.3.1.3 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged False NACK limit unchanged Correct ACK limit unchanged |
| 11.3.3 Performance requirements for PUCCH format 2 | See clause 11.3.1.4 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged UCI BLER limit unchanged |
| 11.3.4 Performance requirements for PUCCH format 3 | See clause 11.3.1.5 | [0.6] dB | Formula: SNR + TTOTA UCI BLER limit unchanged |
| 11.3.5 Performance requirements for PUCCH format 4 | See clause 11.3.1.6 | [0.6] dB | Formula: SNR + TTOTA UCI BLER limit unchanged |
| 11.4.1 PRACH false alarm probability and missed detection | See clause 11.4.1 | [0.6] dB for fading cases [0.3] dB for AWGN cases | Formula: SNR + TTOTA PRACH False detection limit unchanged PRACH detection limit unchanged |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

## -------- end of change ---------