3GPP TSG-RAN4 Meeting # 110-bis	R4-2405556

# Changsha, China,  15 – 19 April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.181 | CR | xxxx | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for TS 38.181: Introduction of FR2-NTN in Clause 4.7 Test Configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of FR2-NTN and SAN Type 2-O in TS 38.108 implies update of TS 38.181 with similar features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of test signal to be used to build NR TCs for SAN type 2-O |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specifications do not contain a test signal to be used to build NR TCs for SAN type 2-O |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## -------- start of change ---------

## 4.7	Test configurations

### 4.7.1	General

The test configurations shall be constructed using the methods defined below, subject to the parameters declared by the manufacturer for the supported RF configurations as listed in clause 4.6. The test configurations to use for conformance testing are defined for each supported RF configuration in clauses 4.8.3.

The applicable test models for generation of the carrier transmit test signal are defined in clause 4.9.

NOTE:	If required, carriers are shifted to align with the channel raster.

### 4.7.2	Test signal used to build Test Configurations

The signal's channel bandwidth and subcarrier spacing used to build NR Test Configurations shall be selected according to tables 4.7.2-1 and 4.7.2-2.

Table 4.7.2-1: Signal to be used to build NR TCs for SAN type 1-H and SAN type 1-O

| Operating Band characteristics |  | FDL_high – FDL_low <100 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 5 MHz (Note 1) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. |  |  |

Table 4.7.2-2: Signal to be used to build NR TCs for SAN type 2-O

| Operating Band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |
| NOTE 1: SAN vendor can decide to test with 50 MHz SAN channel bandwidth and smallest supported SCS declared per operating band (D.7) instead of 100 MHz SAN channel bandwidth in certain regions, where spectrum allocation and regulation require testing with 50 MHz. NOTE 2: If this SAN channel bandwidth is not supported, the narrowest supported SAN channel bandwidth shall be used. |  |  |

### 4.7.3	NRTC1: Contiguous spectrum operation

The purpose of test configuration NRTC1 is to test all SAN requirements excluding CA occupied bandwidth.

For NRTC1 used in receiver tests only the two outermost carriers within each supported operating band need to be generated by the test equipment;

#### 4.7.3.1	NRTC1 generation

NRTC1 shall be constructed on a per band basis using the following method:

-	Declared maximum SAN RF Bandwidth supported for contiguous spectrum operation (D.16) shall be used;

-	Select the carrier to be tested according to 4.7.2 and place it adjacent to the lower SAN RF Bandwidth edge. Place same signal adjacent to the upper SAN RF Bandwidth edge.

-	For transmitter tests, select as many carriers (according to 4.7.2) that the SAN supports within an operating band and fit in the rest of the declared maximum SAN RF Bandwidth (D.16). Place the carriers adjacent to each other starting from the upper SAN RF Bandwidth edge. The nominal channel spacing defined in TS 38.108 [2], clause 5.4.1 shall apply.

All configured component carriers are transmitted simultaneously in the tests where the transmitter should be ON.

#### 4.7.3.2	NRTC1 power allocation

For conducted test:

Set the power spectral density of each carrier to the same level so that the sum of the carrier powers equals the rated total output power (Prated,t,TABC, D.35) according to the manufacturer's declaration in clause 4.6.

For OTA test:

Set the number of carriers to the number of carriers at maximum TRP (D.15).

For EIRP accuracy requirements set each beam to rated beam EIRP (D.11) for the tested beam direction pair.

For all other requirements set the power of each carrier to the same level so that the sum of the carrier powers equals the rated transmitter TRP Prated,t,TRP (D.33).

## 4.8	Applicability of requirements

## -------- end of change ---------