3GPP TSG-RAN WG4 Meeting #110-bis	R4-2405568

Changsha, China, 15 – 19 April, 2024

Title: 	Discussion on remaining issues for BWP without restriction

Source: 	Huawei, HiSilicon

Agenda item:	6.6.1

Document for:	Approval

Introduction

Maintenance of RRM core requirements for BWP without restriction are discussed in RAN4#110, and the outcomes are captured in [1]. Based on [1], further discussions are needed for following remaining issues.

- Support of NCD-SSB on PSCell

- Configuration of SSB outside BWP for L1 measurement

- Requirements for combined options

In this paper, we will provide our views on remaining issues for BWP without restriction.

Discussion

Support of NCD-SSB on PSCell

#### Issue 1-4: Applicability of NCD-SSB based L1 and L3 intra-frequency measurement requirements - FFS: NCD-SSB related requirements also apply for PSCell when SCG deactivation is not enabled for the UE. - FFS requirements are specified for EN-DC only or for both EN-DC and NR-DC.

In RAN4#108-bis discussion, all companies agreed with the benefit to support NCD-SSB on PSCell, and the reason to not define BCD-SSB related requirements for the scenario is that PSCell can be deactivated, and it was considered beyond the WI scope to define how UE uses NCD-SSB during the PSCell activation process.

While it is true that PSCell can be deactivated, it is less likely than SCell. On the other hand, restricting use of NCD-SSB on PCell strongly limits the UE power saving potential, especially in EN-DC scenario, which is used in many NR deployments today and where PSCell is the only NR cell for the UE.

From UE perspective, we understand the use of NCD-SSB is same for PCell and PSCell. Therefore, we suggest to support NCD-SSB on PSCell for both EN-DC and NR-DC. If there is strong concern from UE side, we are also fine to support NCD-SSB on PSCell for EN-DC only.

Proposal 1: RAN4 to define NCD-SSB related requirements for PSCell when the PSCell is activated. The requirements apply at least for EN-DC and also preferably for NR-DC.

A draft LS to RAN2 is in the Annex.

Configuration of SSB outside BWP for L1 measurement

#### Issue 1-5: Configuration of SSB resources within CSI-ResourceConfig settings for option B-1-1 - LS RAN2 to ask whether SSB resources outside a BWP can be configured within CSI-ResourceConfig settings for the BWP if UE supports option B-1-1.

SSB for L1 measurement is configured via CSI-ResourceConfig, and the configuration includes an IE bwp-Id. Assuming this bwp-Id points to BWP-1, the question is whether SSB outside BWP-1 can be used for L1 measurement when BWP-1 is the active BWP.

In our view, the answer should be “yes” for UE supporting option B-1-1. Otherwise, we do not see how NW could configure UE to measure SSB outside active BWP for L1, and how option B-1-1 would work. In fact, in the field description for IE bwp-Id, only CSI-RS is limited to the pointed BWP.

If this is common understanding in RAN4, we believe there is no need to send LS to RAN1/2 as so far there is no restriction that the SSB is limited to the pointed BWP in RAN1 or RAN2 spec. Otherwise, we can send LS to confirm with RAN1/2.

Proposal 2: RAN4 to confirm that SSB outside a BWP can be configured within CSI-ResourceConfig for the BWP if UE supports option B-1-1.

Requirements for combined options

#### Issue 1-1: Requirements/UE behaviour for UE supporting both option B-1-1 and A - FFS following options - Option 1a: - For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/ UE behaviour. - Option 1b: - For UE supporting both option B-1-1 and A, if configured by NW, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario. - Option 2: - For UE supporting both option B-1-1 and A, if CSI-RS is configured within the active BWP for L1 measurements, then UE is NOT expected to perform RLM/BFD/BM based on CD-SSB outside active BWP. - It is clarified in the SSB based L1 measurement requirements that if UE supports both option B-1-1 and option A then the UE is not required to measure the SSB if it is outside active BWP.

We support option 1.

When UE supports both option B-1-1 and A, the relevant scenario is where UE is in an active BWP that does not contain CD- or NCD-SSB, but UE is configured to measure L1 both from SSB and CSI-RS. In this case, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP.

This is same to the R15 scenario where CD-SSB is within active BWP and UE is configured to measure L1 both from SSB and CSI-RS. We do not see additional UE behaviour or requirements are needed.

Proposal 3: For UE supporting both option B-1-1 and A, if configured by NW, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario.

#### Issue 1-3: Requirements/UE behaviour for UE supporting both option C and B-1-1 - FFS following options - Option 1: - For UE supporting both option B-1-1 and C, if both CD-SSB and NCD-SSB are configured outside active BWP, then UE is expected to perform RLM/BFD/BM following NW configuration based on TS 38.331. No additional requirements will be defined for such scenario. - Option 2: - For UE supporting both option B-1-1 and C, if the active BWP that does not contain CD- or NCD-SSB, UE should perform L1 measurement on CD-SSB. No additional requirements will be defined for such scenario.

We understand the two options are same, and we support option 2 for better clarity.

When UE supports both option B-1-1 and C, the relevant scenario is where UE is in an active BWP that does not contain CD- or NCD-SSB, and UE is configured to measure L1 from SSB. In this case, UE should always perform L1 measurement on CD-SSB (i.e. option 2).

This is according to the existing RAN2 spec. NCD-SSB is configured per BWP basis, and in 38.331 it is specified that NCD-SSB must be within the corresponding BWP. In 38.331 it is specified that when NCD-SSB is configured, it will be used for SSB related operations when it is in the active BWP.

absoluteFrequencySSB Frequency of the NCD-SSB. The network configures this field so that the SSB is within the bandwidth of the BWP configured in BWP-DownlinkCommon.

nonCellDefiningSSB If configured, the RedCap UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the CD-SSB of the serving cell (e.g. obtaining sync, measurements, RLM). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the “SSB” configured in the QCL-Info IE; the “ssb-Index” configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitily to this NCD-SSB. The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-periodicity, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE.

For example, if NCD-SSB is configured for BWP1, then the configured NCD-SSB must be within BWP1, and UE should use the NCD-SSB for L1 measurement when BWP1 is the active BWP. If active BWP is BWP2, then UE should not use the NCD-SSB in BWP1 because the NCD-SSB is only configured for BWP1. In this case, UE should use the CD-SSB, which is cell specific, for L1 measurement.

Since RAN2 spec is already clear, we do not see additional UE behaviour or requirements are needed.

It is noted that in RAN4#108, the behaviour is already agreed for UE supporting both option B-1-1 and C when the active BWP contains NCD-SSB.

Proposal 4: For UE supporting both option B-1-1 and C, if the active BWP that does not contain CD- or NCD-SSB, UE should perform L1 measurement on CD-SSB. No additional requirements will be defined for such scenario.

Conclusions

In this paper we provided our views on remaining issues for BWP without restriction.

Proposal 1: RAN4 to define NCD-SSB related requirements for PSCell when the PSCell is activated. The requirements apply at least for EN-DC and also preferably for NR-DC.

Proposal 2: RAN4 to confirm that SSB outside a BWP can be configured within CSI-ResourceConfig for the BWP if UE supports option B-1-1.

Proposal 3: For UE supporting both option B-1-1 and A, if configured by NW, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario.

Proposal 4: For UE supporting both option B-1-1 and C, if the active BWP that does not contain CD- or NCD-SSB, UE should perform L1 measurement on CD-SSB. No additional requirements will be defined for such scenario.

Reference

- R4-2403544, WF on core maintenance and performance part for BWP without restriction, vivo, Vodafone

Annex: draft Reply LS to RAN2 on BWP operation without restriction

3GPP TSG-RAN WG4 Meeting #110-bis R4-240xxxx Changsha, China, 15 – 19 April, 2024 Title: Reply LS to RAN2 on BWP operation without restriction Response to: R2-2313951 Release: Release 18 Work Item: NR_BWP_wor-Core Source: RAN4 To: RAN2 Cc: RAN1 Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 thanks RAN2 for the LS R2-2313951. RAN4 would like to inform RAN2 that based on further discussion in RAN4#110, RAN4 confirms that the NCD-SSB related requirements also apply to PSCell when the PSCell is activated. The requirements apply for [both EN-DC and NR-DC]. RAN4 kindly asks RAN2 to take this information into account. 2. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #111 May 20 – May 24, 2024 Fukuoka, Japan RAN WG4 Meeting #112 Aug 19 – Aug 23, 2024 Maastricht , NL