3GPP TSG-RAN WG4 Meeting #110-bis	R4-2405570

Changsha, China, 15 – 19 April, 2024

Title: 	Discussion on RRM test cases for BWP without restriction

Source: 	Huawei, HiSilicon

Agenda item:	6.6.2

Document for:	Discussion

Introduction

RRM test cases for BWP without restriction are discussed in RAN4#110, and the outcomes are captured in [1]. Based on [1], further discussions are needed for following remaining issues.

- TC for RLM/BFD/BM for option B-1-1

- TC for intra-frequency measurement for option B-1-1

- TC for HO for option C

- TC for RLM/BFD/BM for option A

In this paper, we will provide our views on RRM test cases for BWP without restriction.

Discussion

TC for RLM/BFD/BM for option B-1-1

#### Issue 2-1: Test cases list for RLM/BFD/BM requirements for option B-1-1 Take the following proposal as baseline, and further discuss the CR wording and applicability rule. - Add the following NOTE in a selected set of test cases from the existing test cases for RLM/BFD/L1-RSRP/L1-SINR measurements - NOTE: For testing the UE supporting FG 53-1, the starting PRB index of the SSB can be any possible PRB index of the RF channel BW occurring after the last PRB of the DL active BWP. - FFS: selected set of test cases from the existing test cases for RLM/BFD/L1-RSRP/L1-SINR measurements

Since the basic L1 measurements are already verified by existing TCs, the set of TCs for testing option B-1-1 should be minimized. We suggest to select 1 TC for each measurement and this leads to the following TCs.

Proposal 1: Select the following set of TCs for verifying L1 measurement with option B-1-1.

- FR1: A.6.5.1.2, A.6.5.5.1, A.6.6.4.1, A.6.6.8.2

- FR2: A.7.5.1.2, A.7.5.5.1, A.7.6.3.1, A.7.6.6.2

TC for intra-frequency measurement for option B-1-1

#### Issue 2-2: Test cases list for intra-frequency measurements without gaps for option B-1-1 - Add the following NOTE in a selected set of test cases from the existing test cases for intra-frequency measurements without gaps measurements - NOTE: For testing the UE supporting FG 53-1, the starting PRB index of the SSB can be any possible PRB index of the RF channel BW occurring after the last PRB of the DL active BWP. - FFS: selected set of test cases from the existing test cases for intra-frequency measurements without gaps measurements

Since the basic L3 measurements are already verified by existing TCs, the set of TCs for testing option B-1-1 should be minimized. We suggest to select 1 TC for FR1 and FR2 each and this leads to the following TCs.

Proposal 2: Select the following set of TCs for verifying L3 measurement with option B-1-1.

- FR1: A.6.6.1.1

- FR2: A.7.6.1.1

TC for HO for option C

Define new intra-frequency measurements and handover test cases for option C. As baseline, agree to introduce Test #2, 3, 5, 6, 7, 8. Further discuss whether Test #1 and 4 are needed. Test No.; Test cases; Test configuration; Section. [1; Intra-frequency handover from FR1 to FR1; known target cell configured with NCD-SSB; Baseline: A.16.3.1.2 (FR1); A.6.3.1.Z1]. 2; Inter-frequency handover from FR1 to FR1; known target cell configured with NCD-SSB; Baseline: A.16.3.1.6 (FR1, Measured SSB: CD-SSB, Target SSB: NCD-SSB); A.6.3.1.Z2. 3; Inter-frequency handover from FR1 to FR1; unknown target cell configured with NCD-SSB; Baseline: A.16.3.1.6 (FR1, Measured SSB: CD-SSB, Target SSB: NCD-SSB); A.6.3.1.Z3. [4; Intra-frequency handover from FR2 to FR2; known target cell configured with NCD-SSB; Baseline: A.16.3.1.2 (FR2); A.7.3.1.Z1]. 5; Inter-frequency handover from FR2 to FR2; known target cell configured with NCD-SSB; Baseline: A.17.3.1.2 (FR2, Measured SSB: CD-SSB, Target SSB: NCD-SSB); A.7.3.1.Z2. 6; Inter-frequency handover from FR2 to FR2; unknown target cell configured with NCD-SSB; Baseline: A.17.3.1.2 (FR2, Measured SSB: CD-SSB Target SSB: NCD-SSB); A.7.3.1.Z3. 7; SA event triggered reporting tests without gap configured with NCD-SSB; Baseline: A.6.6.1.1 (FR1, No DRX); A.6.6.1.Z1. 8; SA event triggered reporting tests without gap configured with NCD-SSB; Baseline: A.7.6.1.1 (FR2, No DRX); A.7.6.1.Z1. Note1: In all tests, CD-SSB is configured outside active BWP, but within UE CBW, and NCD-SSB is configured within active BWP..

We do not see strong need to define TC1 and TC4 for HO for option C.

The motivation to have these 2 TCs is to verify UE can do intra-frequency HO with NCD-SSB to NCD-SSB, i.e. UE active BWP in source cell contains NCD-SSB, and the first active BWP in the target cell contains NCD-SSB in the same SSB frequency.

As intra-frequency measurement based on NCD-SSB can be verified with TC 7 and 8, the main points of TC 1 and 4 are to verify UE can do fine time tracking on NCD-SSB and can use a BWP containing NCD-SSB as first active BWP in the target cell. We believe these requirements can be also verified with inter-frequency HO TCs, i.e. TC 2, 3,5 and 6.

Proposal 3: Do not define TC 1 and 4 for HO with option C.

TC for RLM/BFD/BM for option A

#### Issue 2-6: Test cases list for RLM/BFD/BM requirements for option A - Add the following NOTE in a selected set of test cases from the existing test cases for RLM/BFD/L1-RSRP/L1-SINR measurements - NOTE: For testing the UE supporting FG 53-4, the starting PRB index of the SSB can be any possible PRB index of the RF channel BW occurring after the last PRB of the DL active BWP. - FFS: selected set of test cases from the existing test cases for RLM/BFD/L1-RSRP/L1-SINR measurements

Similar to L1 measurement for option B-1-1, we suggest to select a minimum set of TCs for option A.

Proposal 4: Select the following set of TCs for verifying L1 measurement with option A.

- FR1: A.6.5.1.6, A.6.5.5.3, A.6.6.4.3, A.6.6.8.1

- FR2: A.7.5.1.6, A.7.5.5.3, A.7.6.3.3, A.7.6.6.1

Conclusions

In this paper we provided our views on RRM test cases for BWP without restriction.

Proposal 1: Select the following set of TCs for verifying L1 measurement with option B-1-1.

- FR1: A.6.5.1.2, A.6.5.5.1, A.6.6.4.1, A.6.6.8.2

- FR2: A.7.5.1.2, A.7.5.5.1, A.7.6.3.1, A.7.6.6.2

Proposal 2: Select the following set of TCs for verifying L3 measurement with option B-1-1.

- FR1: A.6.6.1.1

- FR2: A.7.6.1.1

Proposal 3: Do not define TC 1 and 4 for HO with option C.

Proposal 4: Select the following set of TCs for verifying L1 measurement with option A.

- FR1: A.6.5.1.6, A.6.5.5.3, A.6.6.4.3, A.6.6.8.1

- FR2: A.7.5.1.6, A.7.5.5.3, A.7.6.3.3, A.7.6.6.1

Reference

- R4-2403544, WF on core maintenance and performance part for BWP without restriction, vivo, Vodafone