3GPP TSG-RAN WG4 Meeting #110-bis	R4-2405584

Changsha, China, 15 – 19 April, 2024

Title: 	On performance requirements for LPHAP

Source: 	Huawei, HiSilicon

Agenda item:	6.12.3.2

Document for:	Discussion

Introduction

RRM performance requirements for LPHAP are discussed in RAN4#110, and the outcomes are captured in [1]. Based on [1], further discussions are needed for the following issues.

- RRM test case

In this paper, we will provide our views on RRM performance requirements for LPHAP.

Discussion

TCs for PRS measurement

Question 1: whether to define test cases for measurement accuracy - Wayforward - FFS whether to define test cases for measurement accuracy for LPHAP.

For PRS measurement, new requirements are defined for RRC_INACTIVE with eDRX, and for RRC_IDLE. For accuracy, it was agreed to re-use existing requirements. In last meeting, we proposed to define accuracy tests for RRC_IDLE, while most companies did not see the need.

Considering that UE supporting PRS measurement in RRC_IDLE must support same in RRC_INACTIVE, and that we already have accuracy tests for the latter in R17, we can compromise to not having accuracy tests for RRC_IDLE. In that case, there is no need to define any accuracy test for the objective.

Proposal 1: RAN4 to define test cases for measurement accuracy for LPHAP.

Issue 1-2-2: RRM testing for PRS measurement Question 2: for which measurement types to define RRC test cases - Wayforward - Option 1: - Define test cases for RSTD, PRS RSRP and PRS RSRPP measurement in RRC_IDLE state and for UE Rx-Tx time difference in RRC_INACTIVE state. - Option 2: - Define test cases for all applicable measurement in RRC_IDLE state and RRC_INACTIVE state, respectively.

For delay tests, one question is for which measurement types to define test cases. Since not all UEs will support PRS measurement in RRC_IDLE, we suggest to define test cases for all applicable measurement in RRC_IDLE state and RRC_INACTIVE state, respectively.

On the other hand, if a UE supports PRS measurement in RRC_IDLE, there is no need for this UE to pass the tests for both RRC_INACTIVE and RRC_IDLE. RAN4 should define test applicability such that this UE does not need to pass the tests for RRC_INACTIVE if the corresponding tests for RRC_IDLE are defined.

Proposal 2a: RAN4 to define test cases for all applicable measurement in RRC_IDLE state and RRC_INACTIVE state, respectively.

Proposal 2b: RAN4 to define test applicability such that a UE supporting PRS measurement in both RRC_IDLE and RRC_INACTIVE does not need to pass the tests for RRC_INACTIVE if the corresponding tests for RRC_IDLE are defined.

Issue 1-2-2: RRM testing for PRS measurement Question 3: what eDRX configuations are to be tested - Background - For RAN eDRX > 10.24s, we have 2 cases for PRS measurement - Case 1: eDRX cycle is smaller or equal to configured PRS measurement reporting periodicity. In this case, UE starts positioning measurement within PTW. - Case 2: eDRX cycle is longer than configured PRS measurement reporting periodicity. In this case, positioning measurement start is not limited to PTW. - Wayforward - Option 1: - Both RAN eDRX ≤ 10.24s and RAN eDRX ＞ 10.24s - For RAN eDRX <= 10.24s, both CN eDRX <= 10.24 s and CN eDRX >10.24 s - Option 2: - Case 2 for RAN eDRX ＞ 10.24 - Option 3: - Both Case 1 and Case 2 for RAN eDRX ＞ 10.24

For PRS measurement, the requirements are defined for both cases with RAN eDRX > 10.24s and with RAN eDRX <= 10.24s. In our view, the main use case for LPHAP is RAN eDRX > 10.24s, and RAN eDRX <= 10.24s is mainly a leftover from R17. We suggest to test only RAN eDRX > 10.24s case.

For RAN eDRX > 10.24s, split requirements are defined for 2 cases.

- Case 1: When eDRX cycle is smaller or equal to configured PRS measurement reporting periodicity, UE starts positioning measurement within PTW

- Case 2: When eDRX cycle is longer than configured PRS measurement reporting periodicity, positioning measurement start is not limited to PTW

Again, we believe Case 2 is more typical configuration for LPHAP. Also, in both cases the measurement cycle is same with Tavailable = LCM (TPRS,i, TDRX), and the difference is that in Case 1 UE is allowed to start the measurement within PTW. Comparing to Case 1, the requirements for Case 2 is more stringent, and we can safely assume a UE that can pass test for Case 2 would be able to meet Case 1 requirements. We suggest to test only Case 2.

Proposal 3: For eDRX in RRC_INACTIVE, RAN4 to define PRS measurement delay TCs for RAN eDRX > 10.24s for Case 2.

Question 4: what PRS measurement features are to be tested - Wayforward - Option 1: - Reduced number of PRS samples - parallelPRS-MeasRRC-Inactive-r17 capability - Lower Rx beam sweeping in FR2 - Option 2: - 4 sample baseline

For PRS measurement delay, we suggest to only test the baseline requirements for 4 samples. Otherwise, all or some of the delay tests needs two TCs, one for 4-sample and the other for 1-sample since side conditions for the two requirements are different. Considering that reduced sample number is not the point of the tests, we prefer to focus on the new requirements for eDRX and RRC_IDLE and take baseline for sample number.

It is noted that parallel measurement and lower Rx beam sweeping factor can be tested as they do not require a separate TC with different side condition.

Proposal 4: For PRS measurement delay TCs, test baseline of 4 samples.

TCs for other requirements

REQ 2: Transmit timing - Wayforward - Option 1: - Test UE transmit timing for positioning measurements - Option 2: - Test UE autonomous TA adjustment REQ 3: TA validation - Wayforward - FFS whether to define test cases for TA validation

We support to define TCs for UE transmit timing requirements. Option 1 is more general than option 2 because the timing accuracy requirement (Te) needs to be met no matter there is autonomous TA adjustment or not. Of course, the test setup can be defined to enable autonomous TA adjustment, so for UE supporting autonomous TA adjustment, both the TA adjustment and timing accuracy can be tested in one go.

Proposal 5: RAN4 to define UE transmit timing TCs, to verify both timing accuracy and autonomous TA adjustment.

In last meeting, we proposed to also define TCs for TA validation. After further checking, it seems the test cannot directly re-use the setup from SDT because cell reselection should be involved for SRS transmission in positioning validity area. Considering the SDT TCs are already complex and there are already quite many TCs to be defined for R18 positioning, we suggest to skip this test.

Proposal 6: RAN4 not to define TCs for TA validation.

TCs for RedCap and non-RedCap UEs

Issue 1-2-4: UE types to be tested - Wayforward - Option 1: - Define separate sets of TCs for normal (non-RedCap) and RedCap type of UEs. - Option 2: - Use same set of TCs for normal (non-RedCap) and RedCap type of UEs.

It is clear that the RRM requirements for LPHAP need to be verified for both RedCap and non-RedCap UEs. The question is whether to define separate TCs for the two UE types or the same TCs can be used for both. We do not have strong view, but slightly prefer to define separate TCs.

- In existing spec, RRM TCs for RedCap are defined separately.

- The spec will be complicated with same TCs, e.g. due to differences in Io for different BWs

Proposal 7: RAN4 to define separate sets of TCs for normal (non-RedCap) and RedCap type of UEs.

TC list

Issue 1-2-5: Test case list - Conclusion - Test case list will be defined in next meeting based on the outcome of open issues. Companies can bring proposals on the detailed test case list.

Based on discussions above, the following TC list is suggested for non-RedCap UE.

Proposal 8: RAN4 to consider the following TC list for LPHAP for non-RedCap UE.

| Set | LPHAP test case for non-RedCap UE | Impacted section in TS 38.133 |
| --- | --- | --- |
| PRS measurement delay tests in RRC_INACTIVE |  |  |
| LP-1 | RSTD reporting delay tests with eDRX in RRC_INACTIVE in FR1 |  |
| LP-2 | RSTD reporting delay tests with eDRX in RRC_INACTIVE in FR2 |  |
| LP-3 | PRS-RSRP reporting delay tests with eDRX in RRC_INACTIVE in FR1 |  |
| LP-4 | PRS-RSRP reporting delay tests with eDRX in RRC_INACTIVE in FR2 |  |
| LP-5 | UE Rx-Tx reporting delay tests with eDRX in RRC_INACTIVE in FR1 |  |
| LP-6 | UE Rx-Tx reporting delay tests with eDRX in RRC_INACTIVE in FR2 |  |
| RRM measurement delay tests in RRC_INACTIVE |  |  |
| LP-7 | Cell reselection delay tests for positioning with eDRX in RRC_INACTIVE in FR1 |  |
| LP-8 | Cell reselection delay tests for positioning with eDRX in RRC_INACTIVE in FR2 |  |
| UL timing tests in RRC_INACTIVE |  |  |
| LP-9 | UE transmit timing for positioning in RRC_INACTIVE in FR1 |  |
| LP-10 | UE transmit timing for positioning in RRC_INACTIVE in FR2 |  |
| PRS measurement delay tests in RRC_IDLE |  |  |
| LP-11 | RSTD reporting delay tests without eDRX in RRC_IDLE in FR1 |  |
| LP-12 | RSTD reporting delay tests with eDRX in RRC_IDLE in FR2 |  |
| LP-13 | PRS-RSRP reporting delay tests with eDRX in RRC_IDLE in FR1 |  |
| LP-14 | PRS-RSRP reporting delay tests without eDRX in RRC_IDLE in FR2 |  |

Conclusions

In this paper we provided our views on RRM performance requirements for LPHAP.

Proposal 1: RAN4 to define test cases for measurement accuracy for LPHAP.

Proposal 2a: RAN4 to define test cases for all applicable measurement in RRC_IDLE state and RRC_INACTIVE state, respectively.

Proposal 2b: RAN4 to define test applicability such that a UE supporting PRS measurement in both RRC_IDLE and RRC_INACTIVE does not need to pass the tests for RRC_INACTIVE if the corresponding tests for RRC_IDLE are defined.

Proposal 3: For eDRX in RRC_INACTIVE, RAN4 to define PRS measurement delay TCs for RAN eDRX > 10.24s for Case 2.

Proposal 4: For PRS measurement delay TCs, test baseline of 4 samples.

Proposal 5: RAN4 to define UE transmit timing TCs, to verify both timing accuracy and autonomous TA adjustment.

Proposal 6: RAN4 not to define TCs for TA validation.

Proposal 7: RAN4 to define separate sets of TCs for normal (non-RedCap) and RedCap type of UEs.

Proposal 8: RAN4 to consider the following TC list for LPHAP for non-RedCap UE.

| Set | LPHAP test case for non-RedCap UE | Impacted section in TS 38.133 |
| --- | --- | --- |
| PRS measurement delay tests in RRC_INACTIVE |  |  |
| LP-1 | RSTD reporting delay tests with eDRX in RRC_INACTIVE in FR1 |  |
| LP-2 | RSTD reporting delay tests with eDRX in RRC_INACTIVE in FR2 |  |
| LP-3 | PRS-RSRP reporting delay tests with eDRX in RRC_INACTIVE in FR1 |  |
| LP-4 | PRS-RSRP reporting delay tests with eDRX in RRC_INACTIVE in FR2 |  |
| LP-5 | UE Rx-Tx reporting delay tests with eDRX in RRC_INACTIVE in FR1 |  |
| LP-6 | UE Rx-Tx reporting delay tests with eDRX in RRC_INACTIVE in FR2 |  |
| RRM measurement delay tests in RRC_INACTIVE |  |  |
| LP-7 | Cell reselection delay tests for positioning with eDRX in RRC_INACTIVE in FR1 |  |
| LP-8 | Cell reselection delay tests for positioning with eDRX in RRC_INACTIVE in FR2 |  |
| UL timing tests in RRC_INACTIVE |  |  |
| LP-9 | UE transmit timing for positioning in RRC_INACTIVE in FR1 |  |
| LP-10 | UE transmit timing for positioning in RRC_INACTIVE in FR2 |  |
| PRS measurement delay tests in RRC_IDLE |  |  |
| LP-11 | RSTD reporting delay tests without eDRX in RRC_IDLE in FR1 |  |
| LP-12 | RSTD reporting delay tests with eDRX in RRC_IDLE in FR2 |  |
| LP-13 | PRS-RSRP reporting delay tests with eDRX in RRC_IDLE in FR1 |  |
| LP-14 | PRS-RSRP reporting delay tests without eDRX in RRC_IDLE in FR2 |  |

Reference

- R4-2403475, WF on R18 NR positioning – LPHAP, Huawei, HiSilicon