3GPP TSG-RAN WG4 Meeting #110-bis	R4-2405586

Changsha, China, 15 – 19 April, 2024

Title: 	Additional simulation results for RedCap 1RX

Source: 	Huawei, HiSilicon

Agenda item:	6.12.3.3

Document for:	Discussion

Introduction

Simulation assumption for evaluating the link level performance of 1RX RedCap UE without FH is updated in RAN4#107 [1]. Our initial simulation results are provided in [2] for RSTD and Rx-Tx. In this paper, we will provide additional simulation results for RSRP and RSRPP based on agreed conditions in RAN4#110.

Discussion

RSRP

Legacy RSRP accuracy are defined under following side conditions

- 4-sample, Es/Iot of -3dB and -13dB, TDL channel (for simulation)

- 1-sample, Es/Iot of 0dB and -6dB, AWGN channel

For RedCap 1RX UE, the Es/Iot for TDL channel is changed to -10dB. The simulation results are shown in Table 1-4.

Table 1: RSRP error for 4-sample, Es/Iot of -3dB, TDL-A

| Parameter | Value |  |  | RSRP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Max(5%,95%) |
|  | 15 | 24 | 1 | 1.7 |
|  |  | 52 | 1 | 1 |
|  |  | 104 | 1 | 1 |
|  | 30 | 24 | 1 | 1.4 |
|  |  | 52 | 1 | 1 |
|  |  | 104 | 1 | 0.9 |
|  | 60 | 24 | 1 | 1.4 |
|  |  | 52 | 1 | 1.4 |
|  |  | 104 | 1 | 1.4 |

Table 2: RSRP error for 4-sample, Es/Iot of -10dB, TDL-A

| Parameter | Value |  |  | RSRP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Max(5%,95%) |
|  | 15 | 24 | 1 | 3.8 |
|  |  | 52 | 1 | 2.1 |
|  |  | 104 | 1 | 1.8 |
|  | 30 | 24 | 1 | 3.3 |
|  |  | 52 | 1 | 2.1 |
|  |  | 104 | 1 | 1.7 |
|  | 60 | 24 | 1 |  |
|  |  | 52 | 1 |  |
|  |  | 104 | 1 |  |

Table 3: RSRP error for 1-sample, Es/Iot of 0dB, AWGN

| Parameter | Value |  |  | RSRP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Max(5%,95%) |
|  | 15 | 48 | 1 | 1 |
|  |  | 52 | 1 | 1 |
|  |  | 104 | 1 | 0.9 |
|  | 30 | 48 | 1 | 1 |
|  |  | 52 | 1 | 1 |
|  |  | 104 | 1 | 0.9 |
|  | 60 | 48 | 1 | 1.4 |
|  |  | 52 | 1 | 1.4 |
|  |  | 104 | 1 | 1.3 |

Table 4: RSRP error for 1-sample, Es/Iot of -6dB, AWGN

| Parameter | Value |  |  | RSRP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Max(5%,95%) |
|  | 15 | 48 | 1 | 1.7 |
|  |  | 52 | 1 | 1.6 |
|  |  | 104 | 1 | 1.4 |
|  | 30 | 48 | 1 | 2.0 |
|  |  | 52 | 1 | 2.0 |
|  |  | 104 | 1 | 1.6 |
|  | 60 | 48 | 1 |  |
|  |  | 52 | 1 |  |
|  |  | 104 | 1 |  |

RSRPP

Legacy RSRPP accuracy are defined under following side conditions

- 4-sample, Es/Iot of -3dB and -13dB, 2-tap channel

- 1-sample, Es/Iot of 0dB and -6dB, 2-tap channel

For RedCap 1RX UE, the Es/Iot for 2-tap channel is changed to -10dB for 4-sample. The simulation results are shown in Table 5-9.

Table 5: RSRPP error for 4-sample, Es/Iot of -3dB, 2-tap channel

| Parameter | Value |  |  | RSRPP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Max(5%,95%) |
|  | 15 | 24 | 1 | 1.4 |
|  |  | 52 | 1 | 1.2 |
|  |  | 104 | 1 | 1.0 |
|  | 30 | 24 | 1 | 1.3 |
|  |  | 52 | 1 | 1.1 |
|  |  | 104 | 1 | 0.9 |
|  | 60 | 24 | 1 | 1.3 |
|  |  | 52 | 1 | 1.1 |
|  |  | 104 | 1 | 1.0 |

Table 6: RSRPP error for 4-sample, Es/Iot of -10dB, 2-tap channel

| Parameter | Value |  |  | RSRPP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Max(5%,95%) |
|  | 15 | 24 | 1 | 2.7 |
|  |  | 52 | 1 | 1.9 |
|  |  | 104 | 1 | 1.3 |
|  | 30 | 24 | 1 | 2.6 |
|  |  | 52 | 1 | 1.6 |
|  |  | 104 | 1 | 1.3 |
|  | 60 | 24 | 1 |  |
|  |  | 52 | 1 |  |
|  |  | 104 | 1 |  |

Table 7: RSRPP error for 1-sample, Es/Iot of 0dB, 2-tap channel

| Parameter | Value |  |  | RSRPP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Max(5%,95%) |
|  | 15 | 48 | 1 | 1.2 |
|  |  | 52 | 1 | 1.4 |
|  |  | 104 | 1 | 1.1 |
|  | 30 | 48 | 1 | 1.2 |
|  |  | 52 | 1 | 1.2 |
|  |  | 104 | 1 | 1.0 |
|  | 60 | 48 | 1 | 1.3 |
|  |  | 52 | 1 | 1.3 |
|  |  | 104 | 1 | 1.1 |

Table 8: RSRPP error for 1-sample, Es/Iot of -6dB, 2-tap channel

| Parameter | Value |  |  | RSRPP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Max(5%,95%) |
|  | 15 | 48 | 1 | 1.8 |
|  |  | 52 | 1 | 1.8 |
|  |  | 104 | 1 | 1.5 |
|  | 30 | 48 | 1 | 1.8 |
|  |  | 52 | 1 | 1.8 |
|  |  | 104 | 1 | 1.5 |
|  | 60 | 48 | 1 |  |
|  |  | 52 | 1 |  |
|  |  | 104 | 1 |  |

Conclusions

In this paper we provided our additional simulation results for 1RX RedCap without FH. We suggest to take these results into account when defining the accuracy requirements for 1RX RedCap without FH.

Reference

- R4-2310075, Updated simulation assumptions for 1Rx RedCap positioning, Ericsson

- R4-2312840, Updated simulation results for PRS measurement with 1RX, Huawei, HiSilicon