3GPP TSG-RAN WG4 Meeting #110bis                                        R4-2405616

Changsha, China, April 15 – 19, 2024

Title:	LS on 4T6R antenna switching SRS

Response to:	-

Release:	Rel-19

Work Item:	NR_ENDC_RF_Ph4-Core

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:	TSG RAN WG2

Contact person:

- Name:	Xiang Gao

- E-mail Address:	gaoxiang74@huawei.com

Attachments: None

- 1		Overall description

RAN4 is tasked to specify RF requirements for 6 Rx UE. Specifically, 3T6R and 4T6R antenna switching SRS are included according to the Rel-19 WID RP-240828.

Core part - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

From UE capability indication perspective, it is noted that both 3T6R and 4T6R are not supported according to TS 38.306.

As for RAN1, RAN4 has no intention to revert following RAN1 conclusion that has been captured in R1-2401798.

Conclusion There is no consensus in RAN1 to support antenna switching for 3TX UE in Rel-19

It is also noticed that how to support 4T6R antenna switching SRS has been discussed specifically in Rel-17. However, as one conclusion achieved in RAN1#107e, 4T6R antenna switching SRS was not implemented in RAN1 specification eventually. Following contents are excerpted from R1-2200002 for information.

Therefore, RAN4 kindly asks RAN1 to restart the work on specifying 4T6R AS-SRS, while RAN2 can consider necessary signaling design afterwards.

- 2		Actions

To RAN1

ACTION:  RAN4 respectfully asks RAN1 to specify 4T6R AS-SRS.

To RAN2

ACTION: 	RAN4 respectfully asks RAN2 to consider necessary signaling design to support 4T6R AS-SRS depending on future inputs from RAN1 and RAN4.

- 3		Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #111	     May 20 – 24, 2024	           			Fukuoka, Japan

TSG RAN WG4 Meeting #112	     August 19 – 23, 2024	   		      Maastricht, Netherland