3GPP TSG-RAN WG4 Meeting #110bis	R4-2405669

Changsha, China, Apr 15 – 19, 2024

Agenda item:	6.12.3.5

Source: 	Qualcomm Incorporated

Title: 	Simulation results for carrier phase positioning

Document for:	Information

# Introduction

RAN4 agreed on updated simulation assumptions for carrier phase positioning measurements in [1]. This paper includes simulation results for the following measurements:

- RSCPD

# Simulation results for RSCPD

| FR | PRS BW (PRBs) | SCS (kHz) | sample rate(Tc) | Num. samples | Num. repetitions | Accuracy [5%, 95%] (deg) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -3 dB, SINRi = -6 dB | SINRref = -6 dB, SINRi = -13 dB |
| FR1 | 24 | 15 | 256 | 1 | 4 | ±17.3 | ±36 |
|  | 52 |  | 128 | 1 | 1 | ±11.5 | ±21.8 |
|  | 104 |  | 64 | 1 | 1 | ±15.2 | ±20.6 |
|  | 24 | 30 | 128 | 1 | 4 | ±17.7 | ±35.7 |
|  | 48 |  | 64 | 1 | 1 | ±11.8 | ±23.4 |
|  | 24 | 60 | 64 | 1 | 4 | ±17.3 | ±35.7 |
| FR2 | 24 | 60 | 64 | 1 | 4 | ±17.3 | ±35.7 |
|  | 64 |  | 32 | 1 | 1 | ±11 | ±21 |
|  | 132 |  | 16 | 1 | 1 | ±18.3 | ±23.9 |
|  | 32 | 120 | 32 | 1 | 4 | ±18 | ±32.6 |
|  | 64 |  | 16 | 1 | 1 | ±11.4 | ±21.1 |

Table 1: RSCPD simulation results in AWGN with 1 sample.

# References

[1] R4-2321459, Updated simulation assumptions for CPP measurement, RAN4#109