3GPP TSG-RAN WG4 Meeting #110bis	R4-2405673

Changsha, China, Apr 15 – 19, 2024

Title: 	Simulation assumptions for PRS measurement accuracy for PRS BW aggregation

Source: 	Qualcomm Incorporated

Agenda item:	6.12.3.4

Document for:	Approval

# Introduction

This paper provides simulation assumption for PRS measurement accuracy for PRS BW aggregation. Parameters for link level simulations are captured in Table 1 and Table 2. Companies are encouraged to provide simulation results based on the agreed performance metric in section 3.

# Simulation assumption

Table 1. General parameters.

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Cell layout | 3 TRPs at distinct locations: <cell 1, cell 2, cell 3>, where TRP 1 is the reference TRP |  |
| Network synchronization | • Synchronous with time shifts <0, 0, 3 us> • Asynchronous with time shifts: <0, 7 symbols, -7 symbols> |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TDD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) | - TDDConf.3.1 (120 kHz) |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |  |
| Data and CCH load in non-PRS symbols | - 50% utilization in time - 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 15 kHz - FDD, TDD - 4 GHz - 30 kHz, 60 kHz - FDD, TDD | - 40 GHz - 60 kHz, 120 kHz - TDD |
| Propagation conditions | AWGN TDL-A (30 ns delay spread, 5Hz) | AWGN TDL-C (60 ns delay spread, 300Hz) |
| Es/Iot for three TRPs (cell 1, cell 2, cell 3), [dB] | Same as Rel-17 side conditions for Nsample = 4 and Nsample = 1. |  |
| Number of samples ( Nsample) | 4 and 1 |  |
| Number of UE receive antennas | 2 |  |
| Number of transmit PRS antennas | 1 |  |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |  |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |  |
| PRS muting | No muting, muting (comb 2) |  |
| Power boosting | No power boosting |  |
| ExpectedRSTD-UncertaintyNOTE 1 | 5 us (15 kHz), 2.5 us (30 kHz), 0.625 us (120 kHz) |  |
| Number of PFLs NOTE 2 | 2 and 3 |  |
| Carrier BW | Smallest channel BW as defined in 38.101 that is larger than the PFL BW |  |
| PFL spacing | See Table 2 |  |
| NOTE 1: nr-DL-PRS-expectedRSTD-uncerainty is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). NOTE 2: Each PFL belongs to a carrier, and the 2 or 3 carriers to which the PFLs belong to are intra-band contiguous. |  |  |

Table 2: PRS transmission configuration parameters for each PFL

| Parameter | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Gap between PFLs (num subcarriers) | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 104 | 84 | 1 | 64 |
|  | 30 | 132 | 82 | 1 | 32 |
|  |  | 272 | 68 | 1 | 16 |
|  | 60, FR1 | 64 | 65 | 1 | 32 |
|  |  | 132 | 82 | 1 | 16 |
|  | 60, FR2 | 64 | 64 | 1 | 32 |
|  |  | 132 | 82 | 1 | 16 |
|  | 120 | 64 | 65 | 1 | 16 |
|  |  | 128 | 82 | 1 | 8 |
| PRS comb size | 4 |  |  |  |  |
| PRS symbol size | 4 |  |  |  |  |
| Number of samples | 2, 4 |  |  |  |  |
| PRS periodicity | 40ms, 200ms |  |  |  |  |
| TOA estimation | Realistic |  |  |  |  |
| Path # | First path |  |  |  |  |

# Performance metric

At least the following performance characteristics are to be provided for PRS RSTD:

- RSTD error CDFs for the 2 neighbour cells

- 5%-ile and 95%-ile of the RSTD errors

At least the following performance characteristics are to be provided for UE Rx-Tx:

- TUE-RX error CDFs for the 3 cells

- 90%-ile of the TUE-RX errors for each cell

In the above,

- RSTD error = estimated RSTD – ideal RSTD (based on perfect channel knowledge).

- UE Rx-Tx error = abs (estimated TUE-RX – ideal TUE-RX) (based on perfect channel and UE location knowledge)