3GPP TSG-RAN WG4 Meeting #110-bis 	R4-2405681

Changsha, China, April 15 – 19, 2024

Source:	Nokia

Title:	Discussion on expected EIRP mask above horizon

Agenda item:	9.2.2	Expected EIRP mask for upper 6GHz	[NR_BS_RF_req_evo-Core]

Document for:	Approval

Introduction

TSG RAN#103 meeting approved new Rel-19 work item “NR base station (BS) RF requirement evolution for FR1/FR2 and testing” [1]. This work item includes two objectives: one is related to expected EIRP mask for upper 6 GHz, and other is related to OTA test enhancement.

WRC-23 has identified the frequency range 6425 – 7125 MHz which is 3GPP band n104 for IMT as in the RR Footnotes 5.457D, 5.457E and 5.457F. The IMT identification comes with some technical conditions which includes the limits on the expected equivalent isotropically radiated power (e.i.r.p.) spectral density of IMT base stations for protecting Earth-to-space fixed satellite services (FSS) as contained in Resolution 220 (WRC-23) [2].

Core part of expected EIRP mask objective of work item [1] is as follow:

Expected EIRP mask for upper 6GHz - Introduce the concept of “limits on expected EIRP for angles above horizon” into the RAN4 BS specifications and specify related BS requirement for 6425-7125 MHz based on the WRC-23 outcome. - Note: target completion date of this objective is December 2024.

In this contribution we focus the discussion on the first objective which is the expected EIPR mask for the upper 6 GHz.

Discussion

Background of expected e.i.r.p.

The ITU-R Radio Regulation has introduced the expected e.i.r.p. technical condition for the protection of the FSS uplink (Earth-to-space) satellite receiver from the aggregate effect of IMT base stations. This is introduced through Resolution 220 (previously known as Resolution COM4/7) [2] and the relevant excerpt is found in the Annex. The expected e.i.r.p. values defined in [2] are as presented in Table 1.

Table 1. Expected e.i.r.p. values for vertical angle range defined in ITU Resolution 220 [2]

| Vertical angle range θL ≤ θ < θH (vertical angle θ above horizon) | Expected e.i.r.p. (dBm/MHz) (See NOTES 1, 2 and 3) |
| --- | --- |
| 0° ≤ θ < 5° | 27 |
| 5° ≤ θ < 10° | 23 |
| 10°≤ θ < 15° | 19 |
| 15°≤ θ < 20° | 18 |
| 20°≤ θ <30° | 16 |
| 30°≤ θ < 60° | 15 |
| 60°≤ θ ≤ 90° | 15 |
| NOTE 1: The expected e.i.r.p. is defined as the average value of the e.i.r.p., with the averaging being performed. ‒ over horizontal angles from −180º to +180º, with the IMT base station beamforming in a specific direction within its horizontal and vertical steering range, ‒ over different beamforming directions within the IMT base station horizontal and vertical steering range, and ‒ over the specified vertical angle range θL ≤ θ < θH. NOTE 2: An IMT base station shall comply with the specified limits on expected e.i.r.p. spectral density for all mechanical tilts with which it can be deployed). NOTE 3: See the Annex to this Resolution for additional details on how the expected e.i.r.p. can be calculated for this frequency band. |  |

The key requirements of ITU Resolution 220 [2] are:

- compliance with values of expected e.i.r.p above the horizon and

- providing a calculation methodology to the theoretical procedure given in the Annex to Resolution 220 [2]

This expected e.i.r.p. condition proposed for the Radio Regulation is only for the top half of the hemisphere, i.e. above the horizon, and 360° (or 2π) around the azimuth. The angles for the ITU-R (Radio Regulation) are defined as described on Figure 1.

Figure 1:  Definitions of the angles defined by the ITU-R (Radio Regulation) Resolution 220 for the expected e.i.r.p.

Observation 1: The key requirements of ITU Resolution Resolution 220 [2] are:

- compliance with values of expected e.i.r.p above the horizon and

- providing a calculation methodology to the theoretical procedure given in the Annex to Resolution 220 [2]

In next sections we provide the formulation of the expected e.i.r.p. with some examples.

The expected e.i.r.p. calculation

The expected e.i.r.p. calculation is based on two averaging processes:

- Averaging of beamforming directions

- Averaging over horizontal and vertical angles

Averaging over beamforming directions

The initial ITU suggestion was a simple averaging process over a set of beams equally within the electrical steering range. This average power can be written as:

Paveθ0,φ0=1Nn=1NPθ0,φ0 ; αn,βn

However, the sharing studies in ITU-R assumes uniformly distributed UEs over the coverage cell which makes the beams steer with a non-uniform distribution over the coverage cell. To take account of this non-uniform distribution in the averaging, we can introduce some ‘weights’, wn, which can provide the adjustments.

The beamforming directions αn,βn have a uniform angular distribution within the steering range of the IMT base station. For a given vertical angle θ0 and horizontal angle φ0 with a sampling of N beamforming directions(αn,βn) n=1…N, the equation can be written as:

Paveθ0,φ0=n=1NwnPθ0,φ0 ; αn,βn

where wn refers to the weight for the nth beamforming direction, i.e., the fraction of the steering range represented by the nth beamforming direction. The power measured is the sum of e.i.r.p. in both polarisations.

The following example illustrate the associated weights for different number of beamforming directions, and how the average e.i.r.p. for the first stage is calculated.

      - Example of average e.i.r.p. calculation

This example has 6 beamforming directions. The Figure 2 shows the electrical steering range, and Figure 3 the beamforming directions with its weight. The weight for each beamforming direction is a simple representation of the fractional area covered by the angular boundaries, and it does not account for the variation due to the elevation. It can be regarded that each of the fractional area represents the probability of the beam electrically steering within these angles.

Figure 2: Electrical steering range

Figure 3: Beamforming directions and its associated weight

Table 2: The weight for each beamforming direction point

| Point | Weight, wn | θ (°) | ϕ (°) |
| --- | --- | --- | --- |
| 1 | 0.125 | -2.5 | -45 |
| 2 | 0.125 | -7.5 | -45 |
| 3 | 0.25 | -2.5 | 0 |
| 4 | 0.25 | -7.5 | 0 |
| 5 | 0.125 | -2.5 | 45 |
| 6 | 0.125 | -7.5 | 45 |

Figure 4: Visualization of beamforming direction point

These beams are multiplied with the associated weights before being summed to form the average e.i.r.p.

Paveθ0,φ0=18Point1+18Point2+28Point3+28Point4+18Point5+18Point6

Observation 2: The number of beamforming directions defined will impact test effort and increasing the number of these beams will increase the time it takes to test.

Proposal 1: RAN4 to consider correct values of beamforming directions taking into account all aspects.

Averaging over horizontal and vertical angles

The second averaging process is over a spherical strip. This average e.i.r.p. (EIRP) for a spherical strip is therefore:

EIRPθL , θH=12πsinθH-sinθLθLθH-ππEIRPθ,φ cosθ dφ dθ

With the help of solid angles and spherical cap formulae, we can approximate this doule integral in discrete form. For an equal angle grid method where the grid spacing is uniform in both the elevation θ (over the range of [0 to π/2]) and azimuth ϕ (over the range of [-π to π]), and with N and M points respectively, this average e.i.r.p. can be approximated to

EIRPθL , θH=12πsinθH-sinθLθLθH-ππEIRPθ,φ cosθ ∆φ ∆θ  ,

where

dφ≈∆φ=2πM

dθ≈∆θ=π2N

Therefore, the overall equation is

EIRPθL , θH=π2MNsinθH-sinθLn=nLnHm=1MEIRPθn,φm cosθn  ,

where

θn	is the midpoint between the elevation intervals

φm	is the midpoint between the azimuth intervals

nL	is the lowest elevation point within the θL , θH bounding range

nH	is the highest elevation point within the θL , θH bounding range

m	is the azimuth point within the -π , π bounding range

Note: the equation is assuming the angles in radians and the elevation angle is from horizon rather than the coordinate system defined in 3GPP.

- Observation 3: Angular steps size both in horizontal and vertical will impact test effort.

Proposal 2: RAN4 to consider the number of measurement points when defining the calculation methodology.

Observation 4: Number of frequencies to measure in operating band will impact test effort.

Proposal 3: RAN4 to consider the number of frequencies to measure when defining the calculation methodology.

Conclusion

In this contribution we discuss expected EIPR mask for upper 6 GHz according ITU Resolution 220 (previously known as Resolution COM4/7). We presented example illustrate the associated weights for different number of beamforming directions, and how the average e.i.r.p. for the first stage is calculated. We have also included a basic methodology on the second stage of the averaging process. We have made following observations and proposals:

Observation 1: The key requirements of ITU Resolution 220 [2] are:

- compliance with values of expected e.i.r.p above the horizon and

- providing a calculation methodology to the theoretical procedure given in the Annex to Resolution 220 [2]

Observation 2: The number of beamforming directions defined will impact test effort and increasing the number of these beams will increase the time it takes to test.

Proposal 1: RAN4 to consider correct values of beamforming directions taking into account all aspects.

Observation 3: Angular steps size both in horizontal and vertical will impact test effort.

Proposal 2: RAN4 to consider the number of measurement points when defining the calculation methodology.

Observation 4: Number of frequencies to measure in operating band will impact test effort.

Proposal 3: RAN4 to consider the number of frequencies to measure when defining the calculation methodology.

- References

[1] RP-240829, New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, RAN4 Chair (Huawei)

[2] ITU WRC-23, Final Acts, Resolution 220

# Annex

# Excerpt from ITU WRC-23, Final Acts, Resolution 220: