3GPP TSG-RAN WG4 Meeting #110-bis 	R4-2405685

Changsha, China, April 15 – 19, 2024

Source:	Nokia

Title:	NCR conformance testing

Agenda item:       6.18.3 RF conformance testing	[NR_netcon_repeater-Perf]

Document for:	Approval

Introduction

Last RAN4#110 meeting continued discussion on NCR RF conformance testing. Agreements and open issues are captured in agreed WF [1].

In this contribution, we further discuss some issues for NCR RF conformance testing.

Discussion

The Way Forward [1] includes following agreements:

For Agreement Issue 1-1: New Spec needed? - Agreements: - To use the TS 38.115-1/2 as baseline spec to specify the RF conformance testing requirement for NCR Issue 1-2: Spec structure - Agreements: - To follow the same principle as agreed for core requirement of NCR TS 38.106 to draft the conformance testing specification. Issue 2-2: Manufacture declaration table - Agreements: - To consider the manufacture declaration table format for NCR as shown in Table 2.3.1 and Table 2.3.2 from R4-2402514 as starting point. Issue 3-1: testing on edge PRB - Agreements: - Regarding the edge 1PRB testing for SEM and transmitter spurious emission requirement for Local area NCR-MT, its transmission power is up to the declaration. Issue 4-1: MU & TT for NCR-Fwd - Agreements: - Consider Rel-17 repeater MU &TT for NCR-Fwd type 1-C/H and type 2-O as starting point. Issue 4-2: MU & TT for NCR-MT - Agreements: - Consider BS MU &TT for NCR-MT type 1-C/H and type 2-O as starting point. Way forward Issue 2-1: Relationship mapping between input connectors and output connectors for Type 1-H NCR-Fwd - Way forward: - The manufacturer shall declare a set of the input/output pairs and/or groups to indicate the mapping between input-side and output-side TAB connectors. The set of declared input/output pairs and/or groups should include all TAB connectors. All TAB connectors need to be tested. - For declared pairs/groups, the applicability of requirements is FFS

Measurements uncertainties and test tolerances

NCR-Fwd type 1-C and type 1-H

In agreed way forward it was stated to consider Rel-17 repeater MU &TT for NCR-Fwd type 1-C/H as starting point. Overall, after checking Maximum Test System uncertainty for conducted characteristics tests in table 4.1.2.2-1 in conducted conformance testing repeater specification [2] for NCR-Fwd type 1-C/H. These values are also presented in table 1 below.

Proposal 1: It is proposed to use defined in table 4.1.2.2-1 in TS 38.115-1 measurement uncertainties for NCR-Fwd type 1-C and type 1-H.

Table 1. Maximum Test System uncertainty for conducted characteristics tests for NCR-Fwd type 1-C and type 1-H.

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Repeater output power | ±0.7 dB, f ≤ 3 GHz ±1.0 dB, 3 GHz < f ≤ 7.125 GHz |  |
| 6.3 Frequency stability | ±12 Hz Measurement results of 500 Hz |  |
| 6.4 Out of band gain | ±0,5 dB, f ≤ 3.0 GHz ±0,8 dB, 3.0 GHz < f ≤ 4.2 GHz Calibration of test set-up shall be made without DUT in order to achieve the accuracy |  |
| 6.5.2 ACLR/ CACLR | BW ≤ 20MHz: ±0.8 dB BW > 20MHz: ±1.2 dB Absolute power ±2.0 dB, f ≤ 3 GHz Absolute power ±2.5 dB, 3 GHz < f ≤ 7.125 GHz |  |
| 6.5.3 Operating band unwanted emission | 1,5 dB, f ≤ 3.0 GHz ±1,8 dB, 3.0 GHz < f ≤ 4.2 GHz The interference from the signal generator ACLR shall be minimum 10 dB below that of a NR repeater according to clause 6.5.2. |  |
| 6.5.4 Spurious emissions | In NR and coexistence receive bands: for results > -60 dBm ±2,0 dB for results < -60 dBm ±3,0 dB Outside above range: emission power; 9 kHz < f 4 GHz ±2,0 dB 4 GHz < f  19 GHz ±4,0 dB. The interference from the signal generator ACLR shall be minimum 10 dB below that of a NR repeater according to clause 6.5.2. |  |
| 6.6 EVM | 1% signal analyser 2% stimulus signal |  |
| 6.7 Input intermodulation | ±1,2 dB | Formula: RSS CW1 level error, 2 x CW2 level error, and measurement error (using all errors = ±0,5 dB) |
| 6.8 Output intermodulation | For operating band unwanted emission: 2,1 dB The interference from the signal generator ACLR shall be minimum 10 dB below that of a NR repeater according to clause 6.5.2. For spurious emission: In NR and coexistence receive bands: for results > -60 dBm ±2,0 dB for results < -60 dBm ±3,0 dB Outside above range: emission power; 9 kHz < f 4 GHz ±2,0 dB; 4GHz < f 19 GHz ±4,0 dB. The interference signal must have a spurious emission level at least 10 dB below the spurious levels required in 6.5.4 and 6.5.5. | Formula: RSS 2x Interference signal level error and operating band unwanted emission measurement level error. (1 dB interference signal level error is assumed). |
| 6.9 ACRR | 0,7 dB |  |
| 6.10.1 Transmit ON/OFF power | ±2.0 dB , f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 7.125 GHz |  |
| 6.10.1 Transmit OFF power | N/A |  |

NCR-Fwd type 2-O

In agreed way forward it was stated to consider Rel-17 repeater MU &TT for type 2-O as starting point. Overall, after checking Maximum Test System uncertainty for radiated characteristics tests in table 4.1.2.2-1 in conducted conformance testing repeater specification [3] for NCR-Fwd type type 2-O. These values are also presented in table 2 below.

Proposal 2: It is proposed to use defined in table 4.1.2.2-1 in TS 38.115-2 measurement uncertainties for NCR-Fwd type 2-O.

Table 2: Maximum Test System uncertainty for radiated characteristics tests for NCR-Fwd type 2-O

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Radiated transmit power (EIRP) | Normal condition: ±1.7 dB (24.25 – 29.5 GHz) ±2.0 dB (37 – 43.5 GHz) ±2.2 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
|  | Extreme condition: ±3.1 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.5 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.3 OTA repeater output power (TRP) | ±2.1 dB (24.25 – 29.5 GHz) ±2.4 dB (37 – 43.5 GHz) ±2.6 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.4 OTA frequency stability | ±1] Hz Measurement results of [5000] Hz |  |
| 6.5 OTA out of band gain | ±2.1 dB, 24.25GHz < f ≦ 29.5GHz ±2.4 dB, 37GHz < f ≦ 43,5GHz ±2.6 dB, 43.5GHz < f ≦ 48.2GHz |  |
| 6.6.2 OTA ACLR | Relative ACLR: ±2.3 dB (24.25 – 29.5 GHz) ±2.6 dB (37 – 43.5 GHz) ±2.8 dB (43.5 GHz < f ≤ 48.2 GHz) Absolute ACLR: ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.6.3 OTA operating band unwanted emission | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.6.4 OTA spurious emissions | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz |  |
| 6.7 OTA EVM | 1.25% signal analyser 2% stimulus signal |  |
| 6.8 OTA input intermodulation | ±2.0 dB, f ≤ 3.0 GHz ±2.6 dB, 3.0 GHz < f ≤ 4.2 GHz ±3.2 dB, 4.2 GHz < f ≤ 6.0 GHz |  |
| 6.9 OTA ACRR | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.10.1 OTA transmitter OFF power | ±2.9 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.6 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.10.2 OTA transient period | N/A |  |

NCR-MT type 1-C and type 1-H

In agreed way forward it was stated to consider BS MU &TT for NCR-MT type 1-C/H as starting point. Overall, after checking Maximum Test System uncertainty for conducted characteristics tests in table 4.1.2.2-1 in conducted BS conformance testing repeater specification [4]. These values are also presented in table 3 below.

Proposal 3: It is proposed to use defined in table 4.1.2.2-1 in TS 38.104 measurement uncertainties for NCR-MT type 1-C and type 1-H that are summarized in table 3.

Table 3. Maximum Test System uncertainty for conducted characteristics tests for NCR-MT type 1-C and type 1-H.

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Repeater output power | ±0.7 dB, f ≤ 3 GHz ±1.0 dB, 3 GHz < f ≤ 7.125 GHz |  |
| 6.3 Frequency stability | ±12 Hz Measurement results of 500 Hz |  |
| 6.4 Out of band gain | ±0,5 dB, f ≤ 3.0 GHz ±0,8 dB, 3.0 GHz < f ≤ 4.2 GHz Calibration of test set-up shall be made without DUT in order to achieve the accuracy |  |
| 6.5.2 ACLR/ CACLR | BW ≤ 20MHz: ±0.8 dB BW > 20MHz: ±1.2 dB Absolute power ±2.0 dB, f ≤ 3 GHz Absolute power ±2.5 dB, 3 GHz < f ≤ 7.125 GHz |  |
| 6.5.3 Operating band unwanted emission | 1,5 dB, f ≤ 3.0 GHz ±1,8 dB, 3.0 GHz < f ≤ 4.2 GHz The interference from the signal generator ACLR shall be minimum 10 dB below that of a NR repeater according to clause 6.5.2. |  |
| 6.5.4 Spurious emissions | In NR and coexistence receive bands: for results > -60 dBm ±2,0 dB for results < -60 dBm ±3,0 dB Outside above range: emission power; 9 kHz < f 4 GHz ±2,0 dB 4 GHz < f  19 GHz ±4,0 dB. The interference from the signal generator ACLR shall be minimum 10 dB below that of a NR repeater according to clause 6.5.2. |  |
| 6.6 EVM | 1% signal analyser 2% stimulus signal |  |
| 6.7 Input intermodulation | ±1,2 dB | Formula: RSS CW1 level error, 2 x CW2 level error, and measurement error (using all errors = ±0,5 dB) |
| 6.8 Output intermodulation | For operating band unwanted emission: 2,1 dB The interference from the signal generator ACLR shall be minimum 10 dB below that of a NR repeater according to clause 6.5.2. For spurious emission: In NR and coexistence receive bands: for results > -60 dBm ±2,0 dB for results < -60 dBm ±3,0 dB Outside above range: emission power; 9 kHz < f 4 GHz ±2,0 dB; 4GHz < f 19 GHz ±4,0 dB. The interference signal must have a spurious emission level at least 10 dB below the spurious levels required in 6.5.4 and 6.5.5. | Formula: RSS 2x Interference signal level error and operating band unwanted emission measurement level error. (1 dB interference signal level error is assumed). |
| 6.9 ACRR | 0,7 dB |  |
| 6.10.1 Transmit ON/OFF power | ±2.0 dB , f ≤ 3 GHz ±2.5 dB, 3 GHz < f ≤ 7.125 GHz |  |
| 6.10.1 Transmit OFF power | N/A |  |

NCR-MT type 2-O

In agreed way forward it was stated to consider BS MU &TT for NCR-MT type 2-O as starting point. Overall, after checking Maximum Test System uncertainty for conducted characteristics tests in table 4.1.2.2-1 in conducted BS conformance testing repeater specification [5]. These values are also presented in table 4 below.

Proposal 4: It is proposed to use defined in table 4.1.2.2-2 in TS 38.104 measurement uncertainties for NCR-MT type 2-O that are summarized in table 4.

Table 4: Maximum Test System uncertainty for radiated characteristics tests for NCR-MT type 2-O

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Radiated transmit power (EIRP) | Normal condition: ±1.7 dB (24.25 – 29.5 GHz) ±2.0 dB (37 – 43.5 GHz) ±2.2 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
|  | Extreme condition: ±3.1 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.5 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.3 OTA repeater output power (TRP) | ±2.1 dB (24.25 – 29.5 GHz) ±2.4 dB (37 – 43.5 GHz) ±2.6 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.4 OTA frequency stability | ±1] Hz Measurement results of [5000] Hz |  |
| 6.5 OTA out of band gain | ±2.1 dB, 24.25GHz < f ≦ 29.5GHz ±2.4 dB, 37GHz < f ≦ 43,5GHz ±2.6 dB, 43.5GHz < f ≦ 48.2GHz |  |
| 6.6.2 OTA ACLR | Relative ACLR: ±2.3 dB (24.25 – 29.5 GHz) ±2.6 dB (37 – 43.5 GHz) ±2.8 dB (43.5 GHz < f ≤ 48.2 GHz) Absolute ACLR: ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.6.3 OTA operating band unwanted emission | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.6.4 OTA spurious emissions | ±2.3 dB, 30 MHz ≤ f ≤ 6 GHz ±2.7 dB, 6 GHz < f ≤ 40 GHz ±5.0 dB, 40 GHz < f ≤ 60 GHz |  |
| 6.7 OTA EVM | 1.25% signal analyser 2% stimulus signal |  |
| 6.8 OTA input intermodulation | ±2.0 dB, f ≤ 3.0 GHz ±2.6 dB, 3.0 GHz < f ≤ 4.2 GHz ±3.2 dB, 4.2 GHz < f ≤ 6.0 GHz |  |
| 6.9 OTA ACRR | ±2.7 dB (24.25 – 29.5 GHz) ±2.7 dB (37 – 43.5 GHz) ±2.9 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.10.1 OTA transmitter OFF power | ±2.9 dB (24.25 – 29.5 GHz) ±3.3 dB (37 – 43.5 GHz) ±3.6 dB (43.5 GHz < f ≤ 48.2 GHz) |  |
| 6.10.2 OTA transient period | N/A |  |

Conclusion

In this contribution, we further discuss some issues for NCR RF conformance testing. We have made following proposals:

Proposal 1: It is proposed to use defined in table 4.1.2.2-1 in TS 38.115-1 measurement uncertainties for NCR-Fwd type 1-C and type 1-H.

Proposal 2: It is proposed to use defined in table 4.1.2.2-1 in TS 38.115-2 measurement uncertainties for NCR-Fwd type 2-O.

Proposal 3: It is proposed to use defined in table 4.1.2.2-1 in TS 38.104 measurement uncertainties for NCR-MT type 1-C and type 1-H that are summarized in table 3.

Proposal 4: It is proposed to use defined in table 4.1.2.2-2 in TS 38.104 measurement uncertainties for NCR-MT type 2-O that are summarized in table 4.

- References

[1] R4-2402960, Way Forward for [110][309] NCR RF conformance

[2] 3GPP TS 38.115-1, NR, Repeater conformance testing - Part 1: Conducted conformance testing

[3] 3GPP TS 38.115-2, NR, Repeater conformance testing - Part 2: Radiated conformance testing

[4] 3GPP TS 38.141-1, NR; Base Station (BS) conformance testing; Part 1: Conducted conformance testing

[5] 3GPP TS 38.141-2, NR; Base Station (BS) conformance testing; Part 2: Radiated conformance testing