3GPP TSG-WG4 Meeting #110bis	R4-2405700

# Changsha, China, 15 April - 19 April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to 38.106: Correction of FRC numbers for NCR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-4-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 agreed to align the FRC numbers among specifications in the past. However, FRC numbers for NCR are not aligned among specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | FRC numbers in annex B are modified. FRC numbers in performance requirement clauses are modifed to be aligned with numbers in annex B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unaligned FRC numbers among specifications may lead to unnecessary confusions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1, 8.2.2, 8.2.3, 9.3.1, Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.115-1, 38.115-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discussion is provided in R4-2405699. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of change-------------

8.2.1.1.1.1	Minimum requirements

Table 8.2.1.1.1.1-1: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex B) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex C) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | M-FR1-A.3.0-1 | 10/15 | TDLA30-10 | 2x2, ULA Low | 70 | -0.9 |
| 1-2 | M-FR1-A.3.1-4 | 10/15 | TDLA30-10 | 2x2, ULA Low | 70 | 6.8 |

Table 8.2.1.1.1.1-2: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex B) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex C) | Antenna configuration | Fraction of maximum BLER (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 2-1 | M-FR1-A.3.0-1 | 10/15 | TDLA30-10 | 2x2, ULA Low | 1 | 5.0 |
| 2-2 | M-FR1-A.3.1-4 | 10/15 | TDLA30-10 | 2x2, ULA Low | 1 | 13.0 |

--------------next change-------------

8.2.1.1.2.1	Minimum requirements

Table 8.2.1.1.2.1-1: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex B) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex C) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | M-FR1-A.3.0-2 | 40/30 | TDLA30-10 | 2x2, ULA Low | 70 | -1.0 |
| 1-2 | M-FR1-A.3.1-1 | 40/30 | TDLA30-10 | 2x2, ULA Low | 70 | 6.8 |

Table 8.2.1.1.2.1-2: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex B) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex C) | Antenna configuration | Fraction of maximum BLER (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 2-1 | M-FR1-A.3.0-2 | 40/30 | TDLA30-10 | 2x2, ULA Low | 1 | 4.3 |
| 2-2 | M-FR1-A.3.1-1 | 40/30 | TDLA30-10 | 2x2, ULA Low | 1 | 12 |

--------------next change-------------

8.2.1.2.1.1	Minimum requirements

Table 8.2.1.2.1.1-1: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex B) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex C) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | M-FR1-A.3.0-1 | 10/15 | TDLA30-10 | 2x4, ULA Low | 70 | -3.9 |
| 1-2 | M-FR1-A.3.1-4 | 10/15 | TDLA30-10 | 2x4, ULA Low | 70 | 3.5 |

Table 8.2.1.2.1.1-2: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex B) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex C) | Antenna configuration | Fraction of maximum BLER (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 2-1 | M-FR1-A.3.0-1 | 10/15 | TDLA30-10 | 2x4, ULA Low | 1 | 0.2 |
| 2-2 | M-FR1-A.3.1-4 | 10/15 | TDLA30-10 | 2x4, ULA Low | 1 | 7.7 |

--------------next change-------------

8.2.1.2.2.1	Minimum requirements

Table 8.2.1.2.2.1-1: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex B) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex C) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | M-FR1-A.3.0-2 | 40/30 | TDLA30-10 | 2x4, ULA Low | 70 | -3.9 |
| 1-2 | M-FR1-A.3.1-1 | 40/30 | TDLA30-10 | 2x4, ULA Low | 70 | 3.6 |

Table 8.2.1.2.2.1-2: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex B) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex C) | Antenna configuration | Fraction of maximum BLER (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 2-1 | M-FR1-A.3.0-2 | 40/30 | TDLA30-10 | 2x4, ULA Low | 1 | -0.5 |
| 2-2 | M-FR1-A.3.1-1 | 40/30 | TDLA30-10 | 2x4, ULA Low | 1 | 7.1 |

--------------next change-------------

8.2.2.1.1.1	1Tx requirements

For the parameters specified in Table 8.2.2.1.1-1 , the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.2.2.1.1.1-1.

Table 8.2.2.1.1.1-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Annex B) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 10 | 24 | 2 | 2 | M-FR1-A.3.4-4 | TDLA30-10 | 1x2 Low | 1 | 8.1 |
| 1-2 | 10 | 48 | 2 | 4 | M-FR1-A.3.4-5 | TDLA30-10 | 1x2 Low | 1 | 5.5 |

8.2.2.1.1.2 	2Tx requirements

For the parameters specified in Table 8.2.2.1.1-1 , the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.2.2.1.1.2-1.

Table 8.2.2.1.1.2-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Annex B) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 10 | 48 | 1 | 8 | M-FR1-A.3.4-6 | TDLA30-10 | 2x2 Low | 1 | -0.2 |

--------------next change-------------

8.2.2.1.2.1	1Tx requirements

For the parameters specified in Table 8.2.2.1.2-1 , the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.2.2.1.2.1-1.

Table 8.2.2.1.2.1-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Annex B) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 40 | 102 | 1 | 2 | M-FR1-A.3.4-1 | TDLA30-10 | 1x2 Low | 1 | 7.0 |
| 1-2 | 40 | 102 | 1 | 4 | M-FR1-A.3.4-2 | TDLA30-10 | 1x2 Low | 1 | 4.9 |

8.2.2.1.2.2	2Tx requirements

For the parameters specified in Table 8.2.2.1.2-1 , the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.2.2.1.2.2-1.

Table 8.2.2.1.2.2-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Annex B) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 40 | 90 | 1 | 8 | M-FR1-A.3.4-3 | TDLA30-10 | 2x2 Low | 1 | -0.7 |

--------------next change-------------

8.2.2.2.1.1	1Tx requirements

For the parameters specified in Table 8.2.2.2.1-1 , the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.2.2.2.1.1-1.

Table 8.2.2.2.1.1-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Annex B) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 10 | 24 | 2 | 2 | M-FR1-A.3.4-4 | TDLA30-10 | 1x4 Low | 1 | 2.2 |
| 1-2 | 10 | 48 | 2 | 4 | M-FR1-A.3.4-5 | TDLA30-10 | 1x4 Low | 1 | 0.2 |

8.2.2.2.1.2	2Tx requirements

For the parameters specified in Table 8.2.2.2.1-1 , the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.2.2.2.1.2-1.

Table 8.2.2.2.1.2-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Annex B) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 10 | 48 | 2 | 8 | M-FR1-A.3.4-6 | TDLA30-10 | 2x4 Low | 1 | -2.5 |

--------------next change-------------

8.2.2.2.2.1	1Tx requirements

For the parameters specified in Table 8.2.2.2.2-1 , the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.2.2.2.2.1-1.

Table 8.2.2.2.2.1-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Annex B) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 40 | 102 | 1 | 2 | M-FR1-A.3.4-1 | TDLA30-10 | 1x4 Low | 1 | 2.1 |
| 1-2 | 40 | 102 | 1 | 4 | M-FR1-A.3.4-2 | TDLA30-10 | 1x4 Low | 1 | 0.7 |

8.2.2.2.2.2	2Tx requirements

For the parameters specified in Table 8.2.2.2.2-1 , the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.2.2.2.2.2-1.

Table 8.2.2.2.2.2-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Annex B) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 40 | 90 | 1 | 8 | M-FR1-A.3.4-3 | TDLA30-10 | 2x4 Low | 1 | -4.1 |

--------------next change-------------

8.2.3.2.1.1	Test parameters

Parameters specified in Table 8.2.3.2.1.1-1 are applied for all test cases in clause 8.2.3.2.1 unless otherwise stated.

Table 8.2.3.2.1.1-1: Test parameters for testing CQI reporting

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |  |  |
| Duplex Mode |  |  |  | FDD |  |  |  |
| Subcarrier spacing |  |  | kHz | 15 |  |  |  |
| SNR |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex C |  |  |  |
| Beamforming Model |  |  |  | TBD |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3,(6) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel (Annex B) |  |  |  | M-FR1-A.3.5-5 |  |  |  |

--------------next change-------------

8.2.3.2.2.1	Test parameters

Parameters specified in Table 8.2.3.2.2.1-1 are applied for all test cases in clause 8.2.3.2.2.1  unless otherwise stated.

Table 8.2.3.2.2.1-1: Test parameters for testing CQI reporting

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern (Note 1) |  |  |  | 7D1S2U, S=6D:4G:4U |  |  |  |
| SNR |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex C |  |  |  |
| Beamforming Model |  |  |  | TBD |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel (Annex B) |  |  |  | [M-FR1-A.3.5-6] |  |  |  |
| Note 1: The same requirements are applicable for TDD with different UL-DL pattern. Note 2: SSB, TRS, CSI-RS, and/or other unspecified test parameters with respect to TS 38.101-4 [28] are left up to test implementation, if transmitted or needed. |  |  |  |  |  |  |  |

--------------next change-------------

8.2.3.3.1.1	Test parameters

Parameters specified in Table 8.2.3.3.1.1-1 are applied for all test cases in clause 8.2.3.3.1.1  unless otherwise stated.

Table 8.2.3.3.1.1-1: Test parameters for testing CQI reporting

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |  |  |
| Subcarrier spacing |  |  | kHz | 15 |  |  |  |
| Duplex Mode |  |  |  | FDD |  |  |  |
| SNR |  |  | dB | 5 | 6 | 11 | 12 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×4 with static channel specified in Annex C |  |  |  |
| Beamforming Model |  |  |  | TBD |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3,(6) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel (Annex B) |  |  |  | M-FR1-A.3.5-5 |  |  |  |

--------------next change-------------

8.2.3.3.2.1	Test parameters

Parameters specified in Table 8.2.3.3.2.1 -1 are applied for all test cases in clause 8.2.3.3.2.1 unless otherwise stated.

Table 8.2.3.3.2.1 -1: Test parameters for testing CQI reporting

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern (Note 1) |  |  |  | 7D1S2U, S=6D:4G:4U |  |  |  |
| SNR |  |  | dB | 5 | 6 | 11 | 12 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×4 with static channel specified in Annex C |  |  |  |
| Beamforming Model |  |  |  | TBD |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3,(6,-) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel (Annex B) |  |  |  | [M-FR1-A.3.5-6] |  |  |  |
| Note 1: The same requirements are applicable for TDD with different UL-DL pattern. Note 2: SSB, TRS, CSI-RS, and/or other unspecified test parameters with respect to TS 38.101-4 [28] are left up to test implementation, if transmitted or needed. |  |  |  |  |  |  |  |

--------------next change-------------

9.3.1.1.2	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput, and also meet the 1st transmission BLER 1% for the FRCs stated in table 9.3.1.1.2-1 at the given respective SNR with the test parameters stated in Tabl9.3.1.1.1-1.

Table 9.3.1.2-1: Minimum requirements for PDSCH with 100 MHz Channel Bandwidth, 120 kHz SCS

| Case number | Bandwidth (MHz) | FRC (Annex B) | SCS (kHz) | Propagation condition | Antenna configuration | 70% throughput (dB) | 1% BLER (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100 | M-FR2-A.3.0-1 | 120 | TDLA30-75 | 2x2 | -0.9 | 2.8 |

--------------next change-------------

9.3.1.2.2	Minimum requirements

The Pm-dsg shall be equal to or smaller than 1%, for the cases stated in Table 9.3.1.2.2-1 at the given SNR with the test parameters stated in Table 9.3.1.2.1-1.

Table 9.3.1.2.2-1: Minimum requirements for PDCCH, 100 MHz Channel Bandwidth, 120 kHz SCS

| Antenna configuration | CORESET RB | CORESET duration | Aggregation level | FRC (Annex B) | Propagation conditions and correlation matrix (Annex I) | Pm-dsg (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1x2 | 60 | 1 | 2 | M-FR2-A.3.4-1 | TDLA30-75, ULA Low | 1 | 6.4 |
| 1x2 | 60 | 1 | 4 | M-FR2-A.3.4-2 | TDLA30-75, ULA Low | 1 | 2.9 |
| 2x2 | 60 | 1 | 8 | M-FR2-A.3.4-3 | TDLA30-75, ULA Low | 1 | 0.1 |

--------------next change-------------

9.4.1.3	Reporting of Channel Quality Indicator (CQI)

9.4.1.3.1	General

The reporting accuracy of the channel quality indicator (CQI) under frequency non-selective conditions is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI median. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [11]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

Table 9.4.1.3.1-1: Test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |  |  |  |
| Subcarrier spacing |  |  | kHz | 120 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| Default TDD UL-DL pattern (Note 1) |  |  |  | 3D1S1U |  |  |  |
| Special Slot Configuration |  |  |  | 10D+2G+2U |  |  |  |
| SNRBB |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex I.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex I.3.1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/4 |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 1.75 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel (Annex B) |  |  |  | M-FR2-A.3.5-1 |  |  |  |
| Note 1: The same requirements are applicable to with different UL-DL patterns. Note 2: SSB, TRS, CSI-RS, and/or other unspecified test parameters with respect to TS 38.101-4 [28] are left up to test implementation, if transmitted or needed. Note 3: If the NCR-MT reports in an available uplink reporting instance at slot #n based on CQI estimation at a downlink slot not later than slot#(n-4), this reported CQI cannot be applied at the gNB downlink before slot#(n+4). |  |  |  |  |  |  |  |

--------------next of change-------------

Annex B (normative):
NCR-MT Reference measurement channels

B.1	NCR-MT Fixed Reference Channels

B.1.1	Fixed Reference Channels for PDSCH performance requirements (QPSK)

The parameters for the reference measurement channels are specified in table B.1.1-1 for FR1 PDSCH performance requirements.

The parameters for the reference measurement channels are specified in table B.1.1-2 for FR2-1 PDSCH performance requirements.

Table B.1.1-1: FRC parameters for FR1 PDSCH performance requirements, 1 transmission layer, QPSK

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | M-FR1-A.3.0-1 | M-FR1-A.3.0-2 |
| Channel bandwidth | MHz | 10 | 40 |
| Subcarrier spacing | kHz | 15 | 30 |
| Allocated resource blocks | PRBs | 52 | 106 |
| Number of consecutive PDSCH symbols |  | 12 | 12 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 4 | 4 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 0.30 | 0.30 |
| Number of MIMO layers |  | 1 | 1 |
| Number of DMRS REs |  | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 |
| Information Bit Payload per Slot |  | 4096 | 8456 |
| Transport block CRC per Slot |  | 24 | 24 |
| Number of Code Blocks per Slot |  | 1 | 2 |
| Binary Channel Bits Per Slot |  | 13728 | 27984 |

Table B.1.1-2: FRC parameters for FR2-1 PDSCH performance requirements, 1 transmission layer, QPSK

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | M-FR2-A.3.0-1 |
| Channel bandwidth | MHz | 100 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  | 13 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.30 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 6 |
| Information Bit Payload per Slot |  | 5504 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 1 |
| Binary Channel Bits Per Slot |  | 18282 |

B.1.2	Fixed Reference Channels for PDSCH performance requirements (16QAM)

The parameters for the reference measurement channels are specified in table B.1.2-1 for FR1 PDSCH performance requirements.

Table B.1.2-1: FRC parameters for FR1 PDSCH performance requirements, 1 transmission layer, 16QAM

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | M-FR1-A.3.1-4 | M-FR1-A.3.1-1 |
| Channel bandwidth | MHz | 10 | 40 |
| Subcarrier spacing | kHz | 15 | 30 |
| Allocated resource blocks | PRBs | 52 | 106 |
| Number of consecutive PDSCH symbols |  | 12 | 12 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 13 | 13 |
| Modulation |  | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 |
| Number of MIMO layers |  | 1 | 1 |
| Number of DMRS REs |  | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 |
| Information Bit Payload per Slot |  | 13064 | 26632 |
| Transport block CRC per Slot |  | 24 | 24 |
| Number of Code Blocks per Slot |  | 2 | 4 |
| Binary Channel Bits Per Slot |  | 27456 | 55968 |

B.1.3	Fixed Reference Channels for PDCCH performance requirements

The parameters for the reference measurement channels are specified in table B.1.3-1 for FR1 PDCCH performance requirements.

The parameters for the reference measurement channels are specified in table B.1.3-2 for FR2-1 PDCCH performance requirements.

Table B.1.3-1: FR1 PDCCH Reference Channels

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | M-FR1-A.3.4-4 | M-FR1-A.3.4-5 | M-FR1-A.3.4-6 | M-FR1-A.3.4-1 | M-FR1-A.3.4-2 | M-FR1-A.3.4-3 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 30 | 30 | 30 |
| CORESET frequency domain allocation |  | 24 | 48 | 48 | 102 | 102 | 90 |
| CORESET time domain allocation |  | 2 | 2 | 1 | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 39 | 52 | 52 | 41 | 53 | 53 |

Table B.1.3-2: FR2-1 PDCCH Reference Channels

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | M-FR2-A.3.4-1 | M-FR2-A.3.4-2 | M-FR2-A.3.4-3 |
| Subcarrier spacing | kHz | 120 | 120 | 120 |
| CORESET frequency domain allocation |  | 60 | 60 | 60 |
| CORESET time domain allocation |  | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 40 | 56 | 56 |

B.1.4	Fixed Reference Channels for CSI reporting performance

This clause defines the DL signal applicable to the reporting of channel status information.

Tables in this clause specifies the mapping of CQI index to Information Bit payload, which complies with the CQI definition specified in clause 5.2.2.1 of TS 38.214 [11] and with MCS definition specified in clause 5.1.3 of TS 38.214 [11].

Table B.1.4-1: Fixed Reference Channels for FR1 CSI reporting with CQI table 2 and MCS table 2

| Reference channel |  |  |  | M-FR1-A.3.5-5 | [M-FR1-A.3.5-6] |
| --- | --- | --- | --- | --- | --- |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 106 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 |
| Available RE-s for PDSCH |  |  |  | 6240 | 12720 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |
| 0 | OOR | OOR | OOR | N/A | N/A |
| 1 | 0.1523 | 0 | QPSK | 1480 | 2976 |
| 2 | 0.3770 | 1 |  | 2408 | 4744 |
| 3 | 0.8770 | 3 |  | 5504 | 11016 |
| 4 | 1.4766 | 5 | 16QAM | 9224 | 18960 |
| 5 | 1.9141 | 7 |  | 12040 | 24576 |
| 6 | 2.4063 | 9 |  | 15112 | 30728 |
| 7 | 2.7305 | 11 | 64QAM | 16896 | 34816 |
| 8 | 3.3223 | 13 |  | 20496 | 42016 |
| 9 | 3.9023 | 15 |  | 24576 | 49176 |
| 10 | 4.5234 | 17 |  | 28168 | 57376 |
| 11 | 5.1152 | 19 |  | 31752 | 65576 |
| 12 | 5.5547 | 21 | 256QAM | 34816 | 69672 |
| 13 | 6.2266 | 23 |  | 38936 | 79896 |
| 14 | 6.9141 | 25 |  | 43032 | 88064 |
| 15 | 7.4063 | 27 |  | 46104 | 94248 |
| NOTE 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data NOTE 2: PDSCH is only scheduled on slots which are full DL |  |  |  |  |  |

Table B.1.4-2: Fixed Reference Channels for FR2-1 CSI reporting with CQI table 1 and MCS table 1

| Reference channel |  |  |  | M-FR2-A.3.5-1 |
| --- | --- | --- | --- | --- |
| Number of allocated PDSCH resource blocks |  |  |  | 66 |
| Number of consecutive PDSCH symbols |  |  |  | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 |
| Number of DMRS REs (Note 1) |  |  |  | 24 |
| Overhead for TBS determination |  |  |  | 6 |
| Available RE-s for PDSCH |  |  |  | 7590 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |
| 0 | OOR | OOR | OOR | N/A |
| 1 | 0.1523 | 0 | QPSK | 1800 |
| 2 | 0.2344 | 0 |  | 1800 |
| 3 | 0.3770 | 2 |  | 2856 |
| 4 | 0.6016 | 4 |  | 4480 |
| 5 | 0.8770 | 6 |  | 6528 |
| 6 | 1.1758 | 8 |  | 8712 |
| 7 | 1.4766 | 11 | 16QAM | 11016 |
| 8 | 1.9141 | 13 |  | 14343 |
| 9 | 2.4063 | 15 |  | 17928 |
| 10 | 2.7305 | 18 | 64QAM | 20496 |
| 11 | 3.3223 | 20 |  | 25104 |
| 12 | 3.9023 | 22 |  | 29192 |
| 13 | 4.5234 | 24 |  | 33816 |
| 14 | 5.1152 | 26 |  | 38936 |
| 15 | 5.5547 | 28 |  | 42016 |
| NOTE 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data NOTE 2: PDSCH is only scheduled on slots which are full DL |  |  |  |  |

B.1.5	Fixed Reference Channels for reference sensitivity level, ACS, in-band blocking, out-of-band blocking and receiver intermodulation (QPSK, R=1/3)

The parameters for the FR1 WA NCR-MT reference measurement channels are specified in tables B.1.5-1 for FR1 reference sensitivity level, ACS, in-band blocking, out-of-band blocking, receiver intermodulation.

Table B.1.5-1: FRC parameters for FR1 reference sensitivity level for FR1 WA NCR-MT.

| Reference channel | G-FR1-A1-22 | G-FR1-A1-23 | G-FR1-A1-24 | G-FR1-A1-25 | G-FR1-A1-26 | G-FR1-A1-27 |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 30 | 60 | 15 | 30 | 60 | 15 |
| Allocated resource blocks | 11 | 11 | 106 | 51 | 24 | 25 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 | 9 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| NOTE 1: DL-DMRS-config-type = 1 with DL-DMRS-max-len = 1, DL-DMRS-add-pos = pos2 with l0 = 2, l = 6 and 9 as per Table 7.4.1.1.2-3 of TS 38.211 [3]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity |  |  |  |  |  |  |

--------------End of change-------------