3GPP TSG-RAN WG4 Meeting #110-bis	R4-2405765

Changsha, China, 15th April – 19th April 2024

Source:	Ericsson

Title:	Adding CA_n100-n101

Agenda item:			6.8.2

Document for:	Approval

# 1. Introduction

In the WID RP-240832 NR channel BW less than 5MHz for FR1 Phase 2 it was agreed to specify the example band combination CA_n100-n101.

# 2. Text Proposal

## 1		CA_n100-n101

### 1.1	Common for 1 band UL and 2 bands UL CA

#### 1.1.1 Operating bands for CA

Table 1.1.1-1: CA band combination of band n100+n101

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n100 | 874.4 MHz – 880 MHz | 919.4 MHz – 925 MHz | FDD |
| n101 | 1900 MHz – 1910 MHz | 1900 MHz – 1910 MHz | TDD |

#### 1.1.2	Channel bandwidths per operating band for CA

Table 1.1.2-1: Supported bandwidths per CA band combination of band n100+n101

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n100A-n101A | CA_n100A-n101A | n100 | 3, 5 | 0 |
|  |  | n101 | 5, 10 |  |

#### 1.1.3	Co-existence studies

Table 1.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n3-n102.

Table 1.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n100 | 874.4 | 880 | 919.4 | 925 | 1748,8 | 1760 | 2623,2 | 2640 | 3497,6 | 3520 | 4372 | 4400 |
| n101 | 1900 | 1910 | 1900 | 1910 | 3800 | 3820 | 5700 | 5730 | 7600 | 7640 | 9500 | 9550 |

Based on the table above there is no UL harmonic issues of this combination.

Table 1.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n100 | 874.4 | 880 | 919.4 | 925 | 1838,8 | 1850 | 2758,2 | 2775 | 3677,6 | 3700 | 4597 | 4625 |
| n101 | 1900 | 1910 | 1900 | 1910 | 3800 | 3820 | 5700 | 5730 | 7600 | 7640 | 9500 | 9550 |

Based on the table above there is no Harmonic mixing issues of this combination.

#### 1.1.4	∆TIB and ∆RIB values

For CA_n100-n101 TIB,c and RIB,c values are reused from CA_n8-n39.

Table 1.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n100-n101 | 0.3 | 0.3 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 1.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n100-n101 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 1.1.5	REFSENs requirements

As can be seen in the co-existence studies in 1.1.3 there are no harmonics issues.

#### 1.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

### 1.2		Specific for 2 bands UL CA

#### 1.2.1	Maximum output power for inter-band CA

Table 1.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n100A-n101A | 23 | +2/-3 |

#### 1.2.2		UE co-existence

Table 1.2.2-1 gives IMD interference analysis for CA_ n100-n101 with uplink CA.

Table 1.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 874.4 | 880 | 1900 | 1910 |
| DL Frequency [MHz] | 919.4 | 925 | 1900 | 1910 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1035,6 | 1020 | 2774,4 | 2790 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 161,2 | 140 | 2920 | 2945,6 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3648,8 | 3670 | 4674,4 | 4700 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2071,2 | 2040 | 5548,8 | 5580 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 713,2 | 740 | 4820 | 4855,6 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4523,2 | 4550 | 6574,4 | 6610 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6765,6 | 6720 | 1620 | 1587,6 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8474,4 | 8520 | 5397,6 | 5430 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3981,2 | 3940 | 1160 | 1196,8 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7448,8 | 7490 | 6423,2 | 6460 |

Based on the table 1.2.2-1 there are no intermodulation products falling inside the RX bands.

#### 1.2.3		REFSENS requirements

There is no need to define additional REFSENS requirements.

# Reference

[1] 	RP-240832, NR channel BW less than 5MHz for FR1 Phase 2