3GPP TSG-RAN4 Meeting #110bis	R4-2405775

# Changsha, China, 15 Apr. 2024 – 19 Apr. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR for 38.133 on RRM core requirements for NR support for dedicated spectrum less than 5MHz for FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RRM requiremnts maintenance for the core part of NR less than 5 MHz |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction HO requirements and RLM/BFD requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements are not correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2, 8.1.1, 8.1.2, 8.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 2>

#### 6.1.1.2	NR FR1 - NR FR1 Handover

##### 6.1.1.2.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

Where:

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

Editor’s Note: P1 is to be updated based on simulation results.

-	If the target cell is a known intra-frequency cell, then Tsearch = 0ms.

-	If the target cell is a known inter-frequency cell, then

-	if the measured SSB is the target SSB for handover of the target cell, Tsearch = 0ms;

-	if the measured SSB and the target SSB for handover belong to the same NR target cell and if the UE supports [UE capability IE for FG 53-3], Tsearch = Trs ms provided any one of the following conditions is satisfied:

-	CD-SSB in initial DL BWP is the measured SSB and NCD-SSB in first active DL BWP is the target SSB for handover

-	NCD-SSB in a DL BWP is the measured SSB and CD-SSB in initial DL BWP is the target SSB for handover

-	Both measured SSB and the target SSB for handover are NCD-SSB within different DL BWPs

-	If the target cell is an unknown inter-frequency cell and Tsearch = 3* Trs ms if the target cell Es/Iot≥-2 dB.

-	If the target cell is an unknown cell operating with 12 PRB SSB bandwidth, then Tinterrupt is extended by [5] *Trs ms

<End of change 2>

# <Start of change 3>

## 8.1	Radio Link Monitoring

### 8.1.1	Introduction

The requirements in clause 8.1 apply for radio link monitoring on:

-	PCell in SA NR, NR-DC and NE-DC operation mode,

-	PCell operating with less than 5MHz BW in SA NR (single carrier) operation mode,

-	PSCell in NR-DC and EN-DC operation mode,

-	Deactivated PSCell in NR-DC and EN-DC operation mode, when configured.

<End of change 3>

# <Start of change 4>

### 8.1.2	Requirements for SSB based radio link monitoring

#### 8.1.2.1	Introduction

Table 8.1.2.1-3: PDCCH transmission parameters for out-of-sync evaluation for a UE operating on a cell with less than 5MHz BW

| Attribute | Value for BLER Configuration #0 |  |  |  |
| --- | --- | --- | --- | --- |
| Channel BW | 3MHz |  |  | 5MHz |
| DL Transmission BW | 12 PRBs |  | 15 PRBs | 20 PRBs |
| DCI format | 1-0 |  |  |  |
| Number of control OFDM symbols | [2] |  | [3] | [3] |
| Aggregation level (CCE) | [4] |  | [8] | [8] |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |  |  |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |  |  |  |
| Bandwidth (PRBs) | 12 |  | 15 | 20 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |  |  |
| DMRS precoder granularity | REG bundle size |  |  |  |
| REG bundle size | 6 |  |  |  |
| CP length | Normal |  |  |  |
| Mapping from REG to CCE | Distributed |  | Non-DistributedNote 1 |  | Distributed |  |
| NOTE 1: For 15 PRB Transmission BW, distributed mapping may lead to unexpected out-of-sync indication. |

Table 8.1.2.1-4: PDCCH transmission parameters for in-sync evaluation for a UE operating on a cell with less than 5MHz BW

| Attribute | Value for BLER Configuration #0 |  |  |
| --- | --- | --- | --- |
| Channel BW | 3MHz |  | 5MHz |
| DL Transmission BW | 12 PRBs | 15 PRBs | 20 PRBs |
| DCI format | 1-0 |  |  |
| Number of control OFDM symbols | [2] | [3] | [3] |
| Aggregation level (CCE) | [2] | [4] | [4] |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |  |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |  |  |
| Bandwidth (PRBs) | 12 | 15 | 20 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |  |
| DMRS precoder granularity | REG bundle size |  |  |
| REG bundle size | 6 |  |  |
| CP length | Normal |  |  |
| Mapping from REG to CCE | Distributed | Non-Distributed | Distributed |

<End of change 4>

# <Start of change 5>

### 8.5.2	Requirements for SSB based beam failure detection

#### 8.5.2.1	Introduction

Table 8.5.2.1-2: PDCCH transmission parameters for beam failure instance for a UE operating on a cell with less than 5MHz BW

| Attribute | Value for BLER |  |  |
| --- | --- | --- | --- |
| Channel BW | 3MHz |  | 5MHz |
| DL Transmission BW | 12 PRBs | 15 PRBs | 20 PRBs |
| DCI format | 1-0 |  |  |
| Number of control OFDM symbols | [2] | [3] | [3] |
| Aggregation level (CCE) | [4] | [8 Note 1] | [8] |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |  |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |  |  |
| Bandwidth (PRBs) | 12 | 15 | 20 |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |  |  |
| DMRS precoder granularity | REG bundle size |  |  |
| REG bundle size | 6 |  |  |
| CP length | Normal |  |  |
| Mapping from REG to CCE | Distributed | Non-DistributedNote2 | [Distributed] |
| NOTE 1: PDCCH puncturing as defined in 38.214 [26] applies. NOTE 2: For 15PRB Transmission BW, distributed mapping may lead to unexpected beam failure detection. |  |  |  |

<End of change 5>