3GPP TSG-RAN WG4 Meeting#110bis	R4-2405790

Changsha, China, April 15 - 19, 2024

Source:	Nokia

Title:	NR Sidelink Evolution: UE Demodulation Performance Requirements

Agenda item:	6.20.4

Document for:	Discussion

Introduction

The Way Forward (WF) of Rel-18 NR Sidelink (SL) Evolution from RAN4#110 is documented in [1]. Several issues have reached agreements, while few issues were left as open issues to be addressed in RAN4#110bis.

For SL Carrier Aggregation (CA), the issues with agreements are summarized below [1]:

#### Issue 1-1-1: NR sidelink CA Bandwidth combination #### Issue 1-1-2: Test setup for PSCCH decoding capability test #### Issue 1-1-3: Test setup for PSFCH decoding capability test

The following issues in SL-Unlicensed (SL-U) were treated in RAN4#110 and led to agreements [1]:

#### Issue 1-2-1: Whether to introduce new requirement for PSSCH/PSCCH in SL-U #### Issue 1-2-2: Whether to introduce new requirement for PSFCH in SL-U #### Issue 1-2-3: Starting point for LBT model #### Issue 1-2-4: Test configurations for PSSCH of SL-U #### Issue 1-2-5: Test configurations for PSCCH of SL-U #### Issue 1-2-6: Test configurations for PSFCH of SL-U

The following WFs [1] still needs to be discussed during RAN4#110bis:

According to the agreements, companies need to check the band combinations and if necessary RAN1 procedures also for UE capability. And each companies are encouraged to prepare CR according to the CR work split. From next meeting we can discuss based on CR document. Additionally for SL-U PSSCH demodulation requirement, companies are encouraged to prepare simulation results according to the agreed simulation assumption. (Just for check simulation can be run at two DMRS patterns of {2,3} and {2,2}) Regarding the LBT model, it can be refined by e-mail discussion after RAN4#110 meeting.

Discussion

In this contribution, we provide our views on the remaining open issues. We first discuss the open issue on SL CA, followed by the open issues on SL-U performance requirements.

SL CA Performance Requirements

While most issues on SL CA were resulting in agreements, nonetheless, there was one issue that needs to be discussed in RAN4#110bis, namely, the bandwidth combination to be specified in the requirements. SL CA related issues and their corresponding agreements from RAN4#110 [1] are summarized below:

#### Issue 1-1-1: NR sidelink CA Bandwidth combination - Define single carrier requirements for each of bandwidth size (10, 20, 30 and 40 MHz) to allow all possible bandwidth combination - Concerning the bandwidth combinations to be specified in the requirements, it is subject to discussion. Candidates would be: 10+10 and 30+40. #### Issue 1-1-2: Test setup for PSCCH decoding capability test - Support every single carrier bandwidth Rel16 test setup/procedure - Check the band combination by next meeting RAN4#110b #### Issue 1-1-3: Test setup for PSFCH decoding capability test - Support every single carrier bandwidth Rel16 test setup/procedure - Check the band combination by next meeting RAN4#110b

As shown in the highlighted agreements above, companies need to check and discuss the bandwidth combination for SL CA in RAN4#110bis. There were already discussions in RAN4#110, in which 10 MHz + 10 MHz and 30 MHz + 40 MHz are considered as possible candidates for the band combinations for demodulation requirements of Rel-18 SL CA.

The following bandwidth combinations are defined in [3]:

### 5.2.3 Channel bandwidth for NR SL CA operation For NR SL CA operation, the SL CA channel bandwidths for each operating band is specified in Table 5.2.3-1. Table 5.2.3-1: Intra-band contiguous CA operating bands for SL CA in FR1 Sidelink CA configuration / Bandwidth combination set. Sidelink CA configuration; Sidelink CA configuration for TX; Component carriers in order of increasing carrier frequency; Maximum aggregated bandwidth (MHz); Bandwidth combination set. Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz). SL_n47B; SL_n47B; 10; 10, 20,30; 70; 0. 20; 20,30. 30; 30,40.

It was agreed in RAN4#110 to define single carrier requirements for each bandwidth size from the possible bandwidth combinations, namely, 10 MHz, 20 MHz, 30 MHz and 40 MHz.

In RAN4#110, 10 MHz + 10 MHz and 30 MHz + 40 MHz were considered as possible candidates for the bandwidth combination used in Rel-18 specification for SL-CA.

If RAN4 is not mandated to define requirements for all possible bandwidth combinations, RAN4 shall consider bandwidth combinations of 10 MHz + 10 MHz and 30 MHz + 40 MHz for Rel-18 SL-CA requirements.

SL Unlicensed Performance Requirements

The discussions on SL-U during RAN4#110 led to agreements that RAN4 will define requirements for only SL-U PSSCH (and no requirements for PSCCH and PSFCH), as shown below [1]:

#### Issue 1-2-1: Whether to introduce new requirement for PSSCH/PSCCH in SL-U - For SL-U only define PSSCH demodulation performance requirements. #### Issue 1-2-2: Whether to introduce new requirement for PSFCH in SL-U - Not support PSFCH in SL-U #### Issue 1-2-3: Starting point for LBT model - This issue can be discussed at issue 1-2-4 together. #### Issue 1-2-4: Test configurations for PSSCH of SL-U Table 11.1.2B.1.1-1: Test parameters Parameter; Unit; Value. Active cell(s); None. SL transmission model; As specified in B.X. SL model parameters; SL transmission duration values; Slots; {2,4,8}. Channel access procedure; Type 1. Channel Access Priority Class (CAPC); 1. LBT failure probability (pLBT); [0.25]. Sidelink UE 1; Sidelink transmissions; PSCCH + PSSCH. PSSCH DMRS pattern (Note 1); {2,3}. Number of interlace; 1 with RB index 0, 5, 10, …, 50. Timing offset (Note 2); s; CP/2-12*64*Tc. Frequency offset (Note 3); Hz; +650. Synchronization; GNSS or GNSS-equivalent. Antenna configuration; 1x2 Low. PSFCH resource period; Slot; 4. MinTimeGapPSFCH; Slot; 3. Note 1: {x, y}: x and y means the number of DMRS symbols for slot with PSFCH transmission and without PSFCH transmission, respectively. Note 2: Time offset of transmitted Sidelink UE signal with respect to GNSS referring timing. Note 3: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency.. Table 11.1.2B.1.1-2: Minimum performance for SL-U Test num.; Reference channel; Bandwidth (MHz)/ Subcarrier spacing(kHz); Modulation format and code rate; Propagation condition; Reference value. PSSCH BLER (%); SNR(dB) of PSSCH. 1; R.PSSCH.2-2.1; 20 / 30; 16QAM, 0.37; TDLA30-195; 10 %; TBD. - Table A.6.2.2-2: PSSCH Reference Channel for SL-U Parameters; Unit; Value. Reference channel; R.PSSCH.2-2.1. Channel bandwidth; MHz; 20. Subcarrier spacing; kHz; 30. Allocated resource blocks; RB; 11. CP-OFDM symbols for slot with PSFCH (Note 1); 9. CP-OFDM symbols for slot without PSFCH (Note 2); 12. Modulation order; 16QAM. MCS index; 11. Number of MIMO layers; 1. Number of DMRS REs; 15. Number of REs for SCI format 1-A; 240. 2nd stage SCI format 2-A configuration; Payloads; Bits; 68. α; 1. βoffset; 2. Overhead for TBS determination; 0. Transport Block Size for slot with PSFCH; Bits; 808. Transport Block Size for slot without PSFCH; Bits; 1416. Transport block CRC; 24. Maximum number of HARQ transmissions; 1. Binary Channel Bits for slots with PSFCH; 2232. Binary Channel Bits for slots without PSFCH; Bits; 3792. Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC), PSFCH symbols, and guard symbols. Note 2: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and guard symbols.. #### Issue 1-2-5: Test configurations for PSCCH of SL-U - From issue 1-2-1, PSCCH channel for SL-U is decided not to support. So, the PSCCH test configuration is not necessary. #### Issue 1-2-6: Test configurations for PSFCH of SL-U - Not support PSFCH. So PSFCH test configuration is not necessary.

The open issues on SL-U that still needs to be discussed during RAN4#110bis are related to the simulation results and the LBT Model as below [1]:

Additionally for SL-U PSSCH demodulation requirement, companies are encouraged to prepare simulation results according to the agreed simulation assumption. (Just for check simulation can be run at two DMRS patterns of {2,3} and {2,2}) Regarding the LBT model, it can be refined by e-mail discussion after RAN4#110 meeting.

DMRS Pattern

As described above, two possible DMRS patterns are considered, namely, {2,3} and {2,2}. Referring to the agreed test parameters for SL-U (cf. Issue 1-2-4) [1], the selected propagation condition has considerably low delay spread and doppler shift.

The selected propagation condition for simulation has considerably low delay spread and doppler shift.

Based on companies’ simulation results in RAN4#110bis, if there is no issue found, the DMRS pattern {2,2} can be considered instead of {2,3}.

LBT Model

During email offline discussions on LBT model prior to RAN4#110bis, the possibility of using the LBT model of LAA instead of the LBT model of NR-U was discussed. In relation to LBT model, there is one parameter that needs to be set for test configuration parameter, namely, p_LBT, which defines the probability of when to transmit or to mute. Referring to [2], the probability p_LBT is set to be p_LBT = 0.75.

The LBT model of LAA could be used for Rel-18 SL-U.

The value for p_LBT in 38.133 is 0.75.

If NR-U LBT model in 38.101-4 B.5 is assumed to be not suitable for SL-U, RAN4 could consider the LBT model of LAA.

If the LBT model of LAA is used, the value for p_LBT shall be defined. For alignment with 38.133, the value of 0.75 can be considered.

Conclusion

In this contribution, we provide our views on issues in Rel-18 NR SL Evolution demodulation performance requirements for discussion in RAN4#110bis. The following Observations and Proposals were made:

Observation 1: It was agreed in RAN4#110 to define single carrier requirements for each bandwidth size from the possible bandwidth combinations, namely, 10 MHz, 20 MHz, 30 MHz and 40 MHz.

Observation 2: In RAN4#110, 10 MHz + 10 MHz and 30 MHz + 40 MHz were considered as possible candidates for the bandwidth combination used in Rel-18 specification for SL-CA.

Proposal 1: If RAN4 is not mandated to define requirements for all possible bandwidth combinations, RAN4 shall consider bandwidth combinations of 10 MHz + 10 MHz and 30 MHz + 40 MHz for Rel-18 SL-CA requirements.

Observation 3: The selected propagation condition for simulation has considerably low delay spread and doppler shift.

Proposal 2: Based on companies’ simulation results in RAN4#110bis, if there is no issue found, the DMRS pattern {2,2} can be considered instead of {2,3}.

Observation 4: The LBT model of LAA could be used for Rel-18 SL-U.

Observation 5: The value for p_LBT in 38.133 is 0.75.

Proposal 3: If NR-U LBT model in 38.101-4 B.5 is assumed to be not suitable for SL-U, RAN4 could consider the LBT model of LAA.

Proposal 4: If the LBT model of LAA is used, the value for p_LBT shall be defined. For alignment with 38.133, the value of 0.75 can be considered.

- References

- R4-2402869, WF on UE demodulation performance for NR SL evolution, LG Electronics, RAN4#110, Athens, Greece, February 26 – March 01, 2024

- 3GPP TS 38.133, Requirements for support of radio resource management, v18.4.0, 12-2023

- 3GPP TR38.786, NR Sidelink evolution; User Equipment (UE) radio transmission and reception (Release 18), V18.1.0, 04-2024