3GPP TSG-RAN WG4 Meeting #110bis	R4-2405791

Changsha, China, April 15 – 19, 2024

Source:	Nokia

Title:	Enhancement of NR dynamic spectrum sharing: UE Demodulation Performance Requirements

Agenda item:	6.25.1

Document for:	Discussion

Introduction

In RAN4#110, several agreements and way forwards (WFs) from the discussions on Enhancement of NR Dynamic Spectrum Sharing (eDSS) have been made and documented in [1]. They are summarized below:

Antenna configuration Agreement: - 2x4, 4x2 for both FDD and TDD Channel bandwidth Agreement: - For TDD, consider 10MHz/15kHz Aggregation level Agreement: - 4, 8 Channel model Agreement: - TDLA30-10 and TDLC300-100 Payload Agreement: - Remain 39 bits PDCCH test cases for requirement Agreement: Test number; Duplex; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; FDD; 10; 48; 2; 4; TDLA30-10; 2x4 Low; 1; TBD. 2; FDD; 10; 48; 2; 8; TDLC300-100; 4x2 Low; 1; TBD. 3; TDD; 10; 48; 2; 4; TDLA30-10; 2x4 Low; 1; TBD. 4; TDD; 10; 48; 2; 8; TDLC300-100; 4x2 Low; 1; TBD. How to align the results Way Forward: - Companies are encouraged to double check their submitted results and update in the next meeting if necessary.

In addition to the agreements and WFs as mentioned above, the work split among companies was also documented in [1].

Discussion

In RAN4#110, interested companies only provided simulation results on FDD cases, as there was still an open issue related to the channel bandwidth for TDD. In the WF [1], companies are encouraged to double check the simulation results and provide an update to the FDD simulation results in RAN4#110bis, if any update is necessary:

How to align the results Way Forward: - Companies are encouraged to double check their submitted results and update in the next meeting if necessary.

Our simulation results can be found in [2] for further information, in which we have updated the results on the agreed FDD cases.

We note here that in [2], we have not yet provided our simulation results for TDD. As the test parameters for both FDD and TDD cases are the same, one may expect that the TDD performance would be similar to FDD.

The test parameters for FDD and TDD are the same, which may lead to similar SNR requirements for both FDD and TDD cases.

RAN4 to discuss whether to change the test parameters for FDD and/or TDD cases.

There is one issue which might be revisited in RAN4#110bis concerning the payload for AL8, which can also be related to Proposal 1 above.

The payload of 39 bits was firstly agreed in RAN4#109 [6]:

Payload bits Agreement: - 39 bits

In RAN4#110, the possibility of considering a higher payload than 39 bits for Aggregation Level (AL) 8 was raised in [4]. The discussion on the payload for AL8 in the online session during RAN4#110, is captured in [3], in which it was also noticed by some companies from the observations on the collected simulation results that the SNR of AL8 is relatively quite low.

In RAN4#110 offline discussion, as new TDD cases were being agreed, there was a down-selection discussion for the test cases for FDD and TDD. As the complete set of simulation results for all FDD and TDD cases were not available in RAN4#110, the payload is first kept being 39 bits for both AL4 andAL8 [1]:

Payload Agreement: - Remain 39 bits

In RAN4#110bis, it is expected that there would be some simulation results from interested companies for both FDD and TDD cases, which may help the evaluation on the payload for AL8. Referring to 38.101-4 [5] section A.3.3, one may observe that for both FDD and TDD higher payloads than 39 bits is mostly used for the case of AL8.

From the collected FDD simulation results in RAN4#110, it was observed that with a payload of 39 bits, Aggregation Level 8 leads to a relatively low SNR value.

In 38.101-4 section A.3.3, the payload of Aggregation Level 8 is mostly set to be higher than 39 bits for both FDD and TDD cases.

RAN4 to further evaluate the collected simulation results in RAN4#110bis, and if agreed, to consider a higher payload than 39 bits for Aggregation Level 8 for Rel-18 eDSS PDCCH.

Conclusion

In this contribution, we provide our views on UE demodulation performance requirements of Rel-18 NR eDSS. The following Observations and Proposals were made:

Observation 1: The test parameters for FDD and TDD are the same, which may lead to similar SNR requirements for both FDD and TDD cases.

Proposal 1: RAN4 to discuss whether to change the test parameters for FDD and/or TDD cases.

Observation 2: From the collected FDD simulation results in RAN4#110, it was observed that with a payload of 39 bits, Aggregation Level 8 leads to a relatively low SNR value.

Observation 3: In 38.101-4 section A.3.3, the payload of Aggregation Level 8 is mostly set to be higher than 39 bits for both FDD and TDD cases.

Proposal 2: RAN4 to further evaluate the collected simulation results in RAN4#110bis, and if agreed, to consider a higher payload than 39 bits for Aggregation Level 8 for Rel-18 eDSS PDCCH.

- References

- R4-2402873, Way forward on [110][328] NR_DSS_enh, 3GPP RAN4#110, Ericsson, Athens, Greece, February 26 ‒ March 01, 2024

- R4-2405792, On PDCCH of Enhancement of NR Dynamic Spectrum Sharing: Simulation Results, Nokia, RAN4#110bis, Changsha, China, April 15 – 19, 2024

- R4-24xxxxx, DRAFT Meeting Report for TSG RAN WG4 meeting: 110, Third Generation Partnership Project (3GPP™), Athens, Greece, February 26 ‒ March 01, 2024 (RAN4_110_BDaT_Session_report_v13_PostMeeting.docx)

- R4-2402720, Enhancement of NR dynamic spectrum sharing: UE Demodulation Performance Requirements, 3GPP RAN4#110, Nokia, Nokia Shanghai Bell, Athens, Greece, February 26 ‒ March 01, 2024

- 3GPP TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements, V18.2.0, 12-2023

- R4-2321139, WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements, 3GPP RAN4#109, Ericsson, Chicago, USA, November 13 ‒ 17, 2023