3GPP TSG-RAN WG4 Meeting #110bis	R4-2405792

Changsha, China, April 15 – 19, 2024

Source:	Nokia

Title:	On PDCCH of Enhancement of NR Dynamic Spectrum Sharing: Simulation Results

Agenda item:	6.25.1

Document for:	Information

Introduction

In [1], the Way Forward (WF) from the discussions on Enhancement of NR Dynamic Spectrum Sharing (eDSS) during RAN4#110 is documented. Interested companies were suggested to bring the updated simulation results on FDD and the preliminary simulation results on TDD for evaluation in RAN4#110bis. The simulation assumptions were also agreed in RAN4#110 for both FDD and TDD and tabulated in [1].

Simulation Results

In this contribution, we provide our simulation results based on the agreements in [1]. The corresponding simulation assumptions [1] are given in the Appendix below for a direct reference.

The simulation results of PDCCH eDSS based on the simulation assumptions are tabulated in Table 1 below:

Table 1. Simulation Results of PDCCH eDSS

| Test number | Duplex | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | FDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2x4 Low | 1 | -2.3 |
| 2 | FDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4x2 Low | 1 | -3.2 |
| 3 | TDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2x4 Low | 1 | TBA |
| 4 | TDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4x2 Low | 1 | TBA |

The simulation results on Table 1 are only for FDD cases. We note here, however, that as the test parameters for FDD and TDD cases are the same, similar performance could be expected.

Our Observations and Proposals related to the simulation results and to other open issues can be found in [2].

Conclusion

In this contribution, we provide our simulation results on PDCCH of eDSS. More on our Observations and Proposals related to the simulation results and to other open issues can be found in [2].

- References

- R4-2402873, Way forward on [110][328] NR_DSS_enh, 3GPP RAN4#110, Ericsson, Athens, Greece, February 26 ‒ March 01, 2024

- R4-2405791, Enhancement of NR dynamic spectrum sharing: UE Demodulation Performance Requirements, 3GPP RAN4#110bis, Nokia, Changsha, China, April 15 ‒ 19, 2024

- Appendix

## Simulation Assumptions

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD/TDD |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLA30-10 and TDLC300-100 |
| Antenna configuration | 2x4, 4x2 |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 39 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 |
| Shift index | 0 |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |
| CORSET RB | 48 |
| AL | 4, 8 |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |