3GPP TSG-WG4 Meeting #110bis 	R4-2405796

# Changsha, China, April 15 - April 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR general aspects for multi Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clarification of scenario for multi Rx operation, and activation |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Removal of restriction os multi Rx operation only on PCell -New applicability session created reflecting agreement that multi-Rx applies for single cell in FR2-1. -Definition of abreviations used for multi Rx |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi Rx can not be used if UE is configured with FR1 + FR2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 3.6.x (new), 3.6.y (new), 8.5, 8.18, 9.5, 9.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revision of R4-2402008 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change #1>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [11] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [11].

AoA	Angle of Arrival

AoD	Angle of Departure

ATG	Air to Ground

BFD	Beam Failure Detection

BFD-RS	BFD Reference Signal

BLER	Block Error Rate

BM-RS	Beam Management Reference Signal

BWP	Bandwidth Part

CA	Carrier Aggregation

CBD	Candidate Beam Detection

CBW	Channel Bandwidth

CC	Component Carrier

CCA	Clear Channel Assessment

CG-SDT	Configured Grant Small Data Transmisison

CLI	Cross Link Interference

CMR	Channel Measurement Resource

CORESET	Control Resource Set

CP	Cyclic Prefix

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

CSI-RSRP	CSI Reference Signal based Reference Signal Received Power

CSI-RSRQ	CSI Reference Signal based Reference Signal Received Quality

CSI-SINR	CSI Reference Signal based Signal to Noise and Interference Ratio

CSI_RP	Received (linear) average power of the resource elements that carry NR CSI-RS signals and channels, measured at the UE antenna connector

DBT	Discovery Burst Transmission

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DL-AoD	Downlink Angle-of-Departure

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell ID

eDRX	Extended DRX

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EMW	Effective measurement window

EMWRP	Effective measurement window repetition period

EN-DC	E-UTRA-NR Dual Connectivity

FDD	Frequency Division Duplex

FH	Frequency Hopping

FR	Frequency Range

GEO	Geostationary Earth Orbit

HARQ	Hybrid Automatic Repeat Request

HO	Handover

GAP	Refers to any of Measurement Gap, activated Pre-MG and NCSG

IMR	Interference Measurement Resource

L1-RSRP	Layer 1 RSRP

L1 SL-RSRP	Layer 1 Sidelink RSRP which corresponds to PSCCH-RSRP and/or PSSCH-RSRP

LEO	Low Earth Orbit

LMF	Location Management Function

LPP	LTE Positioning Protocol

LTM	L1/L2 triggered mobility

MAC	Medium Access Control

MCG	Master Cell Group

mDCI	Multiple Downlink Control Information

MDT	Minimization of Drive Tests

MG	Measurement Gap

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MIB	Master Information Block

ML	Measurement Length

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

MUSIM	Multi-Universal Subscriber Identity Module

NCSG	Network Controlled Small Gap

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NTN	Non-Terrestrial Network

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OTDOA	Observed Time Difference Of Arrival

PBCH	Physical Broadcast Channel

PCC	Primary Component Carrier

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PRACH	Physical RACH

Pre-MG	Pre-configured Measurement Gap

ProSe	Proximity-based Service

PRP	PRS Received Power

PRS	Positioning Reference Signal

PRS-RSRP	Positioning Reference Signal based Reference Signal Received Power

PPW	PRS Processing Window

PSBCH	Physical Sidelink Broadcast Channel

PSBCH-RSRP	Physical Sidelink Broadcast Channel DMRS based Reference Signal Received Power

PSCCH	Physical Sidelink Control Channel

PSCCH-RSRP	Physical Sidelink Control Channel DMRS based Reference Signal Received Power

PSCell	Primary SCell

PSS	Primary Synchronization Signal

PSSCH	Physical Sidelink Shared Channel

PSSCH-RSRP	Physical Sidelink Shared Channel DMRS based Reference Signal Received Power

pTAG	Primary Timing Advance Group

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

QCL	Quasi Co-Location

RACH	Random Access Channel

RAT	Radio Access Technology

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RMSI	Remaining Minimum System Information

RRC	Radio Resource Control

RRH	Remote Radio Head

RRM	Radio Resource Management

RSCP	Reference Signal Carrier Phase

RSCPD	Reference Signal Carrier Phase Difference

RSSI	Received Signal Strength Indicator

RSRP	Reference Signal Received Power

RSRPP	Reference Signal Received Path Power

RSRQ	Reference Signal Received Quality

RSTD	Reference Signal Time Difference

RTOA	Relative Time Of Arrival

RTT	Round Trip Time

sDCI	Single Downlink Control Information

S-SSB	Sidelink Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector or radiated interface boundary.

SA	Standalone operation mode

SAB	Satellite access band

SAN	Satellite Access Node

SCC	Secondary Component Carrier

SCCH	Sidelink Control Channel

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing

SDL	Supplementary Downlink

SDT	Small Data Transmission

SFN	System Frame Number

SFTD	SFN and Frame Timing DifferenceSI	System Information

SIB	System Information Block

SL	Sidelink

SL AoA	Sidelink AoA

SL PRS-RSRP	Sidelink PRS-based RSRP

SL PRS-RSRPP	Sidelink PRS-based RSRPP

SL RSTD	Sidelink RSTD

SL RTOA	Sidelink RTOA

SL Rx-Tx	Sidelink Receive-Transmit time difference

SL-PRS	Sidelink PRS

SL-RSSI	Sidelink Received Signal Strength Indicator

SLPP	Sidelink Positioning Protocol

SLSS	Sidelink Synchronization Signal

SMTC	SSB-based Measurement Timing configuration

SpCell	Special Cell

SRS	Sounding Reference Signal

SRS-RSRP	Sounding Reference Signal based Reference Signal Received Power

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

sTAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TA	Timing Advance

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TDD	Time Division Duplex

TDOA	Time Difference Of Arrival

TN	Terrestrial Network

TRP	Transmission-Reception Point

TTI	Transmission Time Interval

U2N	UE-to-Network

U2U	UE-to-UE

UE	User Equipment

UL	Uplink

V2X	Vehicle-to-Everything service

VIL	Visible Interruption Length

VIRP	Visible Interruption Repetition Period

VSAT	Very Small Aperture Terminal

## <End of Change #1>

## <Start of Change #2>

### 3.6.16	Applicability of requirements for ATG

The requirements in ATG specific clauses apply to an ATG UE operating in FR1 NR SA operation mode at an altitude of at-least 3km.

### 3.6.17	Applicability of requirements for MUSIM gaps

No requirements are defineed in this version of specification when MUSIM gaps collide with (activated) Pre-MG and/or NCSG.

### 3.6.x	Applicability of requirements for FR2 L1 fast beam sweeping

The requirements related to the support of [reducedRxBeamNum] are applicable to a PCell, PSCell, or SCell, provided the cell is the only serving cell in the single FR2-1 band and UE is configured with one FR2-1 band.

### 3.6.y	Applicability of requirements for FR2 L1 scheduling/measurement restrictions relaxation

The requirements related to the support of [30-1] are applicable to a PCell, PSCell, or SCell, provided the cell is the only serving cell in the single FR2-1 band and UE is configured with one FR2-1 band.

## <End of Change #2>

## <Start of Change #3>

#### 8.5.2.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set  estimated over the last TEvaluate_BFD_SSB ms period becomes worse than the threshold Qout_LR_SSB within TEvaluate_BFD_SSB ms period.

The value of TEvaluate_BFD_SSB is defined in Table 8.5.2.2-1 or Table 8.5.2.2-4 (deactivated PSCell) for FR1.

The value of TEvaluate_BFD_SSB is defined in Table 8.5.2.2-2 or Table 8.5.2.2-5 (deactivated PSCell) for FR2 with scaling factor N, where

N = [reducedRxBeamNum] in FR2-1 for UE supporting [fast beam sweeping capability] [and activated with multi-Rx operation] according to the conditions described in clause 3.6.x,

Editor’s note: FFS how to capture UE is activated with multi-Rx operation

N=8 for other cases in FR2-1, and

N=12 for FR2-2,

for FR2 power classes other than power class 6 or for FR2 power class 6 when highSpeedMeasFlagFR2-r17 is not configured.

## <End of Change #3>

## <Start of Change #4>

#### 8.5.3.3	Measurement restrictions for CSI-RS beam failure detection

The SSB mentioned in this clause can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

The UE is required to be capable of measuring CSI-RS for BFD without measurement gaps. The UE is required to perform the CSI-RS measurements with measurement restrictions as described in the following scenarios.

For both FR1 and FR2, when the CSI-RS for BFD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for BFD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for BFD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for BFD measurement without any restriction.

For FR2, when the CSI-RS for BFD measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP],

-	For both FR1 and FR2, when the CSI-RS for BFD measurement is in the same or adjacent OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, UE is not required to receive CSI-RS for BFD measurement in the PRBs that overlap with an SSB.

-	For FR1, when the CSI-RS for BFD measurement is in the same or adjacent OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, if CSI-RS and SSB have different SCS and UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD is expected, and no requirements are defined.

-	For FR2, when the CSI-RS for BFD measurement on one CC is in the same or adjacent OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD is expected, and no requirements are defined.

For FR2, when the CSI-RS for BFD measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band,

-	In the following cases, UE is required to measure one of but not both CSI-RS for BFD measurement and the other CSI-RS. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

-	The CSI-RS for BFD measurement or the other CSI-RS in a resource set configured with repetition ON, or

-	The other CSI-RS is configured in set  and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE,

-	Otherwise, UE shall be able to measure the CSI-RS for BFD measurement without any restriction.

For FR2-1, when the first CSI-RS for BFD measurement is in the same OFDM symbol as the second CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same cell, the UE supporting [multi-Rx capability] according to the conditions described in clause 3.6.y is required to measure both the first and the second CSI-RSs without measurement restrictions, provided the following conditions are met:

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON, and

-	One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH, and

-	the CSI-RS configured for BFD and [at least one of the PDSCHs] are on the same OFDM symbols, and

-	Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

-	[UE is activated with multi-Rx operation]

Editor’s note 1: FFS the condition CSI-RS(s) and PDSCH(s) are overlapped on the same OFDM symbol(s).

Editor’s note 2: FFS how to capture UE is activated with multi-Rx operation.

## <End of Change #4>

## <Start of Change #5>

#### 8.5.5.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured SSB resource in set  estimated over the last TEvaluate_CBD_SSB ms period becomes better than the threshold Qin_LR provided SSB_RP and SSB Ês/Iot are according to Annex Table B.2.4.1 for a corresponding band.

The UE shall monitor the configured SSB resources using the evaluation period in table 8.5.5.2-1 and 8.5.5.2-2 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320ms.

The value of TEvaluate_CBD_SSB is defined in Table 8.5.5.2-1 for FR1.

The value of TEvaluate_CBD_SSB is defined in Table 8.5.5.2-2 for FR2 with scaling factor N, where

N = [TBD] in FR2-1 if the UE supports [fast beam sweeping capability] according to the conditions described in clause 3.6.x,

N=8 for other cases in FR2-1, and

N=12 for FR2-2.

## <End of Change #5>

## <Start of Change #6>

#### 8.5.6.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in set  estimated over the last TEvaluate_CBD_CSI-RS [ms] period becomes better than the threshold Qin_LR within TEvaluate_CBD_CSI-RS [ms] period provided CSI-RS Ês/Iot is according to Annex Table B.2.4.2 for a corresponding band.

The UE shall monitor the configured CSI-RS resources using the evaluation period in table 8.5.6.2-1 and 8.5.6.2-2 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320ms.

The value of TEvaluate_CBD_CSI-RS is defined in Table 8.5.6.2-1 for FR1.

The value of TEvaluate_CBD_CSI-RS is defined in Table 8.5.6.2-2 for FR2 with scaling factor N, where

N = [TBD] in FR2-1 if the UE supports [Fast beam sweeping for layer 1 measurement] according to the conditions described in clause 3.6.x,

N=8 for other cases in FR2-1, and

N=12 for FR2-2.

## <End of Change #6>

## <Start of Change #7>

#### 8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection

The SSB mentioned in this clause can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

For both FR1 and FR2, when the CSI-RS for CBD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for CBD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for CBD measurement, the UE shall be able to perform CSI-RS based CBD measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for CBD measurement, the UE shall be able to perform CSI-RS based CBD measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS based CBD measurement for without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based CBD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for CBD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for CBD measurement without any restriction.

For FR2, when the CSI-RS for CBD measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer evaluation period for CSI-RS based CBD measurement is expected, and no requirements are defined.

For UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP],

-	For both FR1 and FR2, when the CSI-RS for CBD measurement is in the same or adjacent OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, UE is not required to receive CSI-RS for CBD measurement in the PRBs that overlap with an SSB.

-	For FR1, when the CSI-RS for CBD measurement is in the same or adjacent OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, if CSI-RS and SSB have different SCS and UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based CBD is expected, and no requirements are defined.

-	For FR2, when the CSI-RS for CBD measurement on one CC is in the same or adjacent OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based CBD is expected, and no requirements are defined.

For FR2, when the CSI-RS for CBD measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and the other CSI-RS. Longer evaluation period for CSI-RS based CBD measurement is expected, and no requirements are defined.

For FR2-1, when the first CSI-RS for CBD measurement on a cell is in the same OFDM symbol as the second CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same cell, the UE supporting [multi-Rx capability] according to the conditions described in clause 3.6.y is required to measure both the first and the second CSI-RSs without measurement restrictions, while meeting requirements in clause 8.5.6.2, provided the following conditions are met,

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON, and

-	One CSI-RS has same QCL source as the active TCI state of one PDCCH or PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDCCH or PDSCH, and

-	[The CSI-RS configured for CBD and both PDSCHs are on the same OFDM symbols, and]

-	Resources of the active TCI states for the two PDCCHs, or two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

Editor’s note 1: FFS the condition CSI-RS(s) and PDSCH(s) are overlapped on the same OFDM symbol(s).

Editor’s note 2: FFS how to capture UE is activated with multi-Rx operation.

## <End of Change #7>

## <Start of Change #8>

#### 8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2

The following scheduling restriction applies due to beam failure detection.

-	For the case where no RSs are provided for BFD, or when CSI-RS is configured for BFD is explicitly configured and is type-D QCLed with active TCI state for PDCCH or PDSCH, and the CSI-RS is not in a CSI-RS resource set with repetition ON

-	There are no scheduling restrictions due to beam failure detection performed based on the CSI-RS.

-	For the case when UE supporting [multi-Rx capability] in FR2-1, according to the conditions described in clause 3.6.y, is configured to receive two PDSCH transmission occasions from two different QCL sources on a single cell, there are no scheduling restrictions for the PDSCHs due to beam failure detection performed based on the CSI-RS, when following conditions are met:

-	the CSI-RS configured for BFD is not in a CSI-RS resource set with repetition ON, and

-	the CSI-RS configured for BFD has same QCL source as the active TCI state of one of PDSCHs and has different QCL-TypeD from the other PDSCH, and

-	the CSI-RS configured for BFD and [at least one of the PDSCHs] are on the same OFDM symbols, and

-	Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report

-	[UE is activated with multi-Rx operation]

Editor’s Note 1: FFS the CSI-RS and only one of the PDSCHs with different QCLed typeD are on the same OFDM symbol(s)

Editor’s Note 2: FFS how to capture UE is activated with multi-Rx operation.

-	Otherwise

-	For FR2-1 or the BFD-RS is not using 480 kHz SCS or 960 kHz SCS on FR2-2, the UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on BFD-RS resource symbols to be measured for beam failure detection.

-	For FR2-2 and the BFD-RS is using 480 kHz SCS or 960 kHz SCS, the UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on BFD-RS resource symbols to be measured for beam failure detection, and on one data symbol before each BFD-RS symbol to be measured and one data symbol after each BFD-RS symbol to be measured.

When intra-band carrier aggregation in FR2 is performed, the scheduling restrictions on FR2 serving PCell or PSCell apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When inter-band carrier aggregation in FR2 is performed, there are no scheduling restrictions on FR2 serving cells in the bands due to beam failure detection performed on FR2 serving cell(s) in different band(s), provided that UE is capable of independent beam management on this FR2 band pair. Additionally, there is no scheduling restriction if the UE is configured with different numerology between SSB on one FR2 band and data on the other FR2 band provided the UE is configured for IBM operation for the band pair.

For FR2, if following conditions are met,

-	UE has been notified about system information update through paging,

-	The gap between UE’s reception of PDCCH that UE monitors in the Type2-PDCCH CSS set and that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, UE is expected to receive the PDCCH that UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for BFD mesurement; and

For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, UE is expected to receive PDSCH that corresponds to the PDCCH that UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for BFD mesurement.

## <End of Change #8>

## <Start of Change #9>

#### 8.18.2.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in two sets q0,0 and q0,1 estimated over the last TEvaluate_BFD_SSB ms period becomes worse than the threshold Qout_LR_SSB within TEvaluate_BFD_SSB ms period.

The value of TEvaluate_BFD_SSB is defined in Table 8.18.2.2-1 for FR1.

The value of TEvaluate_BFD_SSB is defined in Table 8.18.2.2-2 for FR2 with scaling factor N, where

N = [TBD] in FR2-1 for UE supporting [TBD - multi-rx faster beam switching capability] [additional conditions FFS], according to the conditions described in clause 3.6.x, and

N=8 for other cases in FR2.

## <End of Change #9>

## <Start of Change #10>

#### 8.18.3.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the CSI-RS resource in two sets q0,0 and q0,1 estimated over the last TEvaluate_BFD_CSI-RS ms period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS ms period.

The value of TEvaluate_BFD_CSI-RS is defined in Table 8.18.3.2-1 for FR1.

The value of TEvaluate_BFD_CSI-RS is defined in Table 8.18.3.2-2 for FR2 with N=1. The requirements of TEvaluate_BFD_CSI-RS apply provided that the CSI-RS for BFD is not in a resource set configured with repetition ON. The requirements shall not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for BFD and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

For FR1,

-	P=11-TCSI-RSMGRP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS.

-	P = 1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

For FR2,

-	P = 1, when the BFD-RS resource is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSMGRP, when the BFD-RS resource is partially overlapped with measurement gap and the BFD-RS resource is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=11-TCSI-RSTSMTCperiod, when the BFD-RS resource is not overlapped with measurement gap and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = Psharing factor, when the BFD-RS resource is not overlapped with measurement gap and the BFD-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=11-TCSI-RSMGRP - TCSI-RSTSMTCperiod, when the BFD-RS resource is partially overlapped with measurement gap and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5 × TSMTCperiod

-	P=Psharing factor1-TCSI-RSMGRP, when the BFD-RS resource is partially overlapped with measurement gap and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TCSI-RS = 0.5 × TSMTCperiod

-	P=11-TCSI-RSTSMTCperiod, when the BFD-RS resource is partially overlapped with measurement gap (TCSI-RS < MGRP) and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P=Psharing factor1-TCSI-RSMGRP, when the BFD-RS resource is partially overlapped with measurement gap and the BFD-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor = 1, if the BFD-RS resource outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and;

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

where,

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Note:	The overlap between CSI-RS for BFD and SMTC means that CSI-RS for BFD is within the SMTC window duration.

Longer evaluation period would be expected if the combination of the BFD-RS resource, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For either an FR1 or FR2 serving cell, longer BFD evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer BFD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of MBFD used in Table 8.18.3.2-1 and Table 8.18.3.2-2 are defined as

-	MBFD = 10, if the CSI-RS resource(s) in the two sets q0,0 and q0,1 used for BFD is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

The values of PBFD used in Table 8.18.3.2-1 and Table 8.18.3.2-2 are defined as

For each CSI-RS resource in the two sets q0,0 and q0,1 configured for PCell or PSCell in EN-DC or NE-DC or SA; or PCell in NR-DC

-	PBFD = 1.

For each CSI-RS resource in the two sets q0,0 and q0,1 configured for PSCell in NR-DC

PBFD = 2 if UE is configured for beam failure detection on SCell, 1 otherwise.

For each CSI-RS resource in the two sets q0,0 and q0,1 configured for a SCell

-	PBFD = Z in EN-DC or NE-DC or SA.

-	PBFD = 2* Z in NR-DC.

Where Z is the number of band(s) on which UE is performing beam failure detection only for SCell.

For UE not supporting [TBD - multi-rx capability], the values of PTRP in table 8.18.3.2-2 is defined as 2, if SSB/CSI-RS resources in the two sets q0,0 and q0,1  are overlapped, else it is 1.

For FR2-1, for UE supporting [TBD - multi-rx capability], according to the conditions described in clause 3.6.x, the value of PTRP in table 8.18.3.2-2 is defined as 1, when:

-	CSI-RS resources in the two sets q0,0 and q0,1  are not overlapped, or

-	CSI-RS resources in the two sets q0,0 and q0,1  are overlapped and the following conditions are met:

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON

-	The two CSI-RS resources in the two sets q0,0 and q0,1  for beam failure detection [and both PDSCH] are overlapped on the same OFDM symbol.

-	[The CSI-RS in set q0,0 has same QCL source as the active TCI state of one PDSCH, and the CSI-RS in set q0,1 has same QCL source as the active TCI state of the other PDSCH]

-	Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

-	[FFS how to capture UE is activated with multi-Rx operation]

-	else, the value of PTRP is 2.

## <End of Change #10>

## <Start of Change #11>

#### 8.18.3.3	Measurement restrictions for CSI-RS beam failure detection

The SSB mentioned in this clause can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

The UE is required to be capable of measuring CSI-RS for BFD without measurement gaps. The UE is required to perform the CSI-RS measurements with measurement restrictions as described in the following scenarios.

For both FR1 and FR2, when the CSI-RS for BFD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for BFD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for BFD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for BFD measurement without any restriction.

For FR2, when the CSI-RS for BFD measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For FR2, when the CSI-RS for BFD measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band,

-	In the following cases, UE is required to measure one of but not both CSI-RS for BFD measurement and the other CSI-RS. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

-	The CSI-RS for BFD measurement or the other CSI-RS in a resource set configured with repetition ON, or

-	The other CSI-RS is configured in two sets q1,0 and q1,1 and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE,

-	Otherwise, UE shall be able to measure the CSI-RS for BFD measurement without any restriction.

For FR2-1, there is no measurement restriction allowed for UE supporting [TBD - multi-rx capability] according to the conditions described in clause 3.6.y, and the UE is required to measure both the CSI-RS for BFD and the other CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, provided the following conditions are met:

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON

-	[FFS: The two CSI-RS resources are overlapped on the same OFDM symbol.]

-	[The two CSI-RS resources and both PDSCH are overlapped on the same OFDM symbol.]

Editor’s note: FFS whether CSI-RS need to overlap with PDSCH for measurement restriction

-	The CSI-RS and both of the PDSCHs are on the same OFDM symbol(s).

-	One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH

-	Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

-	[FFS how to capture UE is activated with multi-Rx operation]

When two CSI-RSs for BFD measurements are from different sets q0,0 and q0,1, UE shall be able to perform measure both CSI-RSs for BFD measurements.

## <End of Change #11>

## <Start of Change #12>

#### 8.18.3.3	Measurement restrictions for CSI-RS beam failure detection

The SSB mentioned in this clause can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

The UE is required to be capable of measuring CSI-RS for BFD without measurement gaps. The UE is required to perform the CSI-RS measurements with measurement restrictions as described in the following scenarios.

For both FR1 and FR2, when the CSI-RS for BFD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for BFD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for BFD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for BFD measurement without any restriction.

For FR2, when the CSI-RS for BFD measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For FR2, when the CSI-RS for BFD measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band,

-	In the following cases, UE is required to measure one of but not both CSI-RS for BFD measurement and the other CSI-RS. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

-	The CSI-RS for BFD measurement or the other CSI-RS in a resource set configured with repetition ON, or

-	The other CSI-RS is configured in two sets q1,0 and q1,1 and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE,

-	Otherwise, UE shall be able to measure the CSI-RS for BFD measurement without any restriction.

For FR2-1,  there is no measurement restriction allowed for UE supporting [TBD - multi-rx capability] according to the conditions described in clause 3.6.y, and the UE is required to measure both the CSI-RS for BFD and the other CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, provided the following conditions are met:

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON

-	[FFS: The two CSI-RS resources are overlapped on the same OFDM symbol.]

-	[The two CSI-RS resources and both PDSCH are overlapped on the same OFDM symbol.]

Editor’s note: FFS whether CSI-RS need to overlap with PDSCH for measurement restriction

-	The CSI-RS and both of the PDSCHs are on the same OFDM symbol(s).

-	One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH

-	Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

-	[FFS how to capture UE is activated with multi-Rx operation]

When two CSI-RSs for BFD measurements are from different sets q0,0 and q0,1, UE shall be able to perform measure both CSI-RSs for BFD measurements.

## <End of Change #12>

## <Start of Change #13>

#### 8.18.5.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured SSB resource in the two sets q1,0 and q1,1,estimated over the last TEvaluate_CBD_SSB ms period becomes better than the threshold Qin_LR provided SSB_RP and SSB Ês/Iot are according to Annex Table B.2.4.1 for a corresponding band.

The UE shall monitor the configured SSB resources using the evaluation period in table 8.18.5.2-1 and 8.18.5.2-2 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320ms.

The value of TEvaluate_CBD_SSB is defined in Table 8.18.5.2-1 for FR1.

The value of TEvaluate_CBD_SSB is defined in Table 8.18.5.2-2 for FR2 with scaling factor N, where

N = [TBD] in FR2-1 if the UE supports [fast beam sweeping capability]  [additional conditions FFS], according to the conditions described in clause 3.6.x, and

N=8 for other cases in FR2.

## <End of Change #13>

## <Start of Change #14>

#### 8.18.6.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in sets q1,0 and q1,1 estimated over the last TEvaluate_CBD_CSI-RS [ms] period becomes better than the threshold Qin_LR within TEvaluate_CBD_CSI-RS [ms] period provided CSI-RS Ês/Iot is according to Annex Table B.2.4.2 for a corresponding band.

The UE shall monitor the configured CSI-RS resources using the evaluation period in table 8.18.6.2-1 and 8.18.6.2-2 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320ms.

The value of TEvaluate_CBD_CSI-RS is defined in Table 8.18.6.2-1 for FR1.

The value of TEvaluate_CBD_CSI-RS is defined in Table 8.18.6.2-2 for FR2 with scaling factor N, where

N = [TBD] in FR2-1 if the UE supports [fast beam sweeping capability]  [additional conditions FFS], according to the conditions described in clause 3.6.x, and

N=8 for other cases in FR2.

## <End of Change #14>

## <Start of Change #15>

#### 8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection

The SSB mentioned in this clause can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

For both FR1 and FR2, when the CSI-RS for CBD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for CBD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for CBD measurement, the UE shall be able to perform CSI-RS based CBD measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for CBD measurement, the UE shall be able to perform CSI-RS based CBD measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS based CBD measurement for without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based CBD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for CBD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for CBD measurement without any restriction.

For FR2, when the CSI-RS for CBD measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer evaluation period for CSI-RS based CBD measurement is expected, and no requirements are defined.

For FR2, when the CSI-RS for CBD measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and the other CSI-RS. Longer evaluation period for CSI-RS based CBD measurement is expected, and no requirements are defined. When two CSI-RSs for CBD measurements are from different sets q1,0 and q1,1, UE shall be able to measure both CSI-RSs for CBD measurements.

For FR2-1, there is no measurement restriction allowed for UE supporting [TBD - multi-rx capability] according to the conditions described in clause 3.6.y, and the UE is required to measure both the CSI-RS for CBD and the other CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, provided the following conditions are met:

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON, and

-	[FFS: The two CSI-RS resources are overlapped on the same OFDM symbol, and]

-	[The two CSI-RS resources and both PDSCH are overlapped on the same OFDM symbol, and]

-	The CSI-RS and both of the PDSCHs are on the same OFDM symbol(s), and

-	One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH, and

-	Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

Editor’s note 1: FFS whether CSI-RS need to overlap with PDSCH for measurement restriction.

Editor’s note 2: [FFS how to capture UE is activated with multi-Rx operation.]

## <End of Change #15>

## <Start of Change #16>

#### 8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2

The following scheduling restriction applies due to TRP specific beam failure detection.

-	For the case where no RSs are provided for BFD, or when CSI-RS is configured for BFD is explicitly configured and is type-D QCLed with active TCI state for PDCCH or PDSCH, and the CSI-RS is not in a CSI-RS resource set with repetition ON

-	For FR2-1, for UE supporting [TBD - multi-rx capability] according to the conditions described in clause 3.6.y, if CSI-RS for BFD and the other CSI-RS for tracking or for CQI in the same or overlapping OFDM symbol are configured with different QCL-TypeD in the cell and the following conditions apply:

-	The CSI-RS is not in a CSI-RS resource set with repetition ON.

-	The CSI-RS has same QCL source as the active TCI state of one of the PDSCHs and has different QCL-TypeD from the other PDSCH.

-	The CSI-RS and both of the PDSCHs are on the same OFDM symbol(s).

-	[FFS: The CSI-RS and only one of the PDSCHs with different QCLed typeD are on the same OFDM symbol(s)]

-	Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

-	[FFS how to capture UE is activated with multi-Rx operation]

-	There are no scheduling restrictions due to TRP specific beam failure detection performed based on the CSI-RS.

-	Otherwise

-	The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on BFD-RS resource symbols to be measured for TRP specific beam failure detection.

When intra-band carrier aggregation in FR2 is performed, the scheduling restrictions on FR2 serving PCell or PSCell apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When inter-band carrier aggregation in FR2 is performed, there are no scheduling restrictions on FR2 serving cells in the bands due to beam failure detection performed on FR2 serving cell(s) in different band(s), provided that UE is capable of independent beam management on this FR2 band pair. Additionally, there is no scheduling restriction if the UE is configured with different numerology between SSB on one FR2 band and data on the other FR2 band provided the UE is configured for IBM operation for the band pair.

For FR2, if following conditions are met,

-	UE has been notified about system information update through paging,

-	The gap between UE’s reception of PDCCH that UE monitors in the Type2-PDCCH CSS set and that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, UE is expected to receive the PDCCH that UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for BFD mesurement; and

For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, UE is expected to receive PDSCH that corresponds to the PDCCH that UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for BFD mesurement.

## <End of Change #16>

## <Start of Change #17>

### 9.5.2	Requirements applicability

The requirements in clause 9.5 apply, provided:

-	The CSI-RS or SSB or CSI-RS and SSB resources configured for L1-RSRP measurements are measurable.

An SSB resource configured for L1-RSRP shall be considered measurable when for each relevant SSB the following conditions are met:

-	L1-RSRP related side conditions given in clauses 10.1.19.1 and 10.1.20.1 for FR1 and FR2, respectively, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.4.1 for a corresponding band.

A CSI-RS resource configured for L1-RSRP shall be considered measurable when for each relevant CSI-RS the following conditions are met:

-	L1-RSRP related side conditions given in clauses 10.1.19.2 and 10.1.20.2 for FR1 and FR2, respectively, for a corresponding band,

-	CSI-RS_RP and CSI-RS Ês/Iot according to Annex B.2.4.2 for a corresponding band.

A CSI-RS and SSB resource configured for L1-RSRP shall be considered measurable when the measurable resource conditions are met for both CSI-RS resource and SSB resource.

Requirements are defined for periodic, semi-persistent and aperiodic resources.

In case the SSB resources configured for L1-RSRP measurements outside SMTCs are overlapped with the PDSCH/PDCCH that is associated to a PCI different from serving cell, L1-RSRP measurement performance degradation is expected.

If groupBasedBeamReporting-r17 is configured, the requirements in clause 9.5 apply when all SSB resources of csi-SSB-ResourceSet or CSI-RS resources of NZP-CSI-RS-ResourceSet within the CSI-ResourceConfig settings are configured for L1-RSRP for the active BWP are from a serving cell, including a single PCell, PSCell, or SCell configured for L1-RSRP measurements, when the UE is configured with a single serving cell (PCell, PSCell, or SCell) in FR2-1.

For a UE supports [two TAs and RTD > CP capability], the requirements apply when Rx timing difference is up to MRTD as specified in clause [7.X1.X].

For a UE which do not support [two TAs and RTD > CP capability], the requirements apply when Rx timing difference is less than CP.

## <End of Change #17>

## <Start of Change #18>

#### 9.5.4.1	SSB based L1-RSRP Reporting

When groupBasedBeamReporting-r17 is not configured, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

When groupBasedBeamReporting-r17 is configured, the UE shall be capable of performing L1-RSRP measurements based on the two configured SSB resource sets for L1-RSRP, and the UE physical layer shall be capable of reporting group-based L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

When there is no intra-frequency L1-RSRP measurement on LTM neighbor cell(s) to measure, the value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-1 for FR1, the value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is not configured, and defined in Table 9.5.4.1-2A for FR2-1 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is configured, and defined in Table 9.5.4.1-3 for FR2 power class 6 UE when highSpeedMeasFlagFR2-r17 is configured, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	The value of N in Table 9.5.4.1-2 and Table 9.5.4.1-3 is [reducedRxBeamNum] in FR2-1 for UE supporting [faster beam switching capability], according to the conditions described in clause 3.6.x, and activated with multi-Rx operation, otherwise the value of N in Table 9.5.4.1-2 is 8.

Editor note: FFS how to capture UE is activated with multi-Rx operation.

-	The value of N in Table 9.5.4.1-2A is 8.

## <End of Change #18>

## <Start of Change #19>

#### 9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP

The SSB mentioned in this clause can be associated with either the serving cell PCI or a PCI different from serving cell PCI or intra-frequency neighbor cell(s) configured with L1-RSRP measurement.

For both FR1 and FR2, when the CSI-RS for L1-RSRP measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for L1-RSRP measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for L1-RSRP measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for L1-RSRP measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and SSB. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for L1-RSRP measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for L1-RSRP measurement without any restriction.

For FR2, when the CSI-RS for L1-RSRP measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and SSB. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined.

For FR2, when the CSI-RS for L1-RSRP measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band,

-	In the following cases, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and the other CSI-RS. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined.

-	The CSI-RS for L1-RSRP measurement or the other CSI-RS in a resource set configured with repetition ON, or

-	The other CSI-RS is configured in q1 and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE,

-	Otherwise, UE shall be able to measure the CSI-RS for L1-RSRP measurement without any restriction.

For FR2-1, when the CSI-RS for L1-RSRP measurement on the one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC, UE supporting [multi-Rx capability] according to the conditions described in clause 3.6.y shall be able to measure the CSI-RS for L1-RSRP measurement without restriction when the following conditions are met:

-	Both the CSI-RS for L1-RSRP and the other CSI-RS are not in any CSI-RS resource set configured with repetition ON, and

-	Resources of the active TCI states for two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report, and

-	[The two CSI-RSs and both PDSCHs are overlapped on the same OFDM symbol, and]

-	The CSI-RS for L1-RSRP has same QCL source as the active TCI state of one [PDSCH], and the other CSI-RS has same QCL source as the active TCI state of the other [PDSCH], and

Editor’s note 1: FFS the condition CSI-RS(s) and PDSCH(s) are overlapped on the same OFDM symbol(s).

Editor’s note 2: FFS how to capture UE is activated with multi-Rx operation.

## <End of Change #19>

## <Start of Change #20>

#### 9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2

The following scheduling restriction applies due to L1-RSRP measurement.

-	For the case where RS for L1-RSRP measurement is CSI-RS which is QCLed with active TCI state for PDCCH/PDSCH and not in a CSI-RS resource set with repetition ON, and N=1 applies as specified in clause 9.5.4.2

-	There are no scheduling restrictions due to L1-RSRP measurement performed based on the CSI-RS.

-	For FR2-1 and the case where UE supporting [multi-rx capability] according to the conditions described in clause 3.6.y is configured to receive two PDSCH transmission occasions from two different QCL sources, there are no scheduling restrictions for the PDSCHs due to L1-RSRP measurement performed based on the CSI-RS, when following conditions are met:

-	The CSI-RS configured for L1-RSRP is not in a CSI-RS resource set with repetition ON, and

-	The CSI-RS configured for L1-RSRP has same QCL source as the active TCI state of one of PDSCHs and has different QCL-TypeD from the other PDSCH, and

-	The CSI-RS configured for L1-RSRP and both PDSCHs are on the same OFDM symbol(s), and

-	Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

Editor’s Note 1: FFS the CSI-RS and only one of the PDSCHs with different QCLed typeD are on the same OFDM symbol(s)

Editor’s Note 2: FFS how to capture UE is activated with multi-Rx operation.

## <End of Change #20>

## <Start of Change #21>

#### 9.8.4.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured

The UE shall be capable of performing L1-SINR measurements with the SSB configured as CMR and dedicated resource configured as IMR for L1-SINR computation, in which the NZP-CSI-RS or CSI-IM resource configured as dedicated IMR shall be 1-to-1 mapped to SSB configured as CMR, with the same periodicity. The UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_SSB_CMR_IMR.

The requirements in this clause are not applicable if NZP-CSI-RS or CSI-IM resource configured as dedicated IMR is scheduled with different periodicity as SSB configured as CMR.

The value of TL1-SINR_Measurement_Period_SSB_CMR_IMR is defined in Table 9.8.4.2-1 for FR1 and in Table 9.8.4.2-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured, and defined in Table 9.8.4.2-3 for FR2 power class 6 UE when highSpeedMeasFlagFR2-r17 is configured, where

For the value of M

-	For periodic or semi-persistent NZP CSI-RS or CSI-IM resource as dedicated IMR, M=1 if the higher layer parameters timeRestrictionForChannelMeasurements and/or timeRestrictionForInterferenceMeasurements are configured, and M=3 otherwise;

For the value of N in FR2

-	N = [reducedRxBeamNum] in FR2-1 for UE supporting [faster beam switching capability], according to the conditions described in clause 3.6.x,

-	N = 8 otherwise.

## <End of Change #21>

## <Start of Change #22>

#### 9.8.6.3	Scheduling availability of UE performing L1-SINR measurement on FR2

The following scheduling restriction applies due to L1-SINR measurement.

-	For UE not supporting [TBD – multi-rx capability], for the cases of CSI-RS used for L1-SINR measurement of CSI-RS based CMR only case and CSI-RS based CMR plus CSI-RS based ZP-IMR/NZP-IMR case and CSI-RS based CMR plus ZP-IMR case, where CSI-RS is QCLed with active TCI state for PDCCH/PDSCH and not in a CSI-RS resource set with repetition ON, and N=1 applies as specified in clause 9.8.4, or

-	For FR2-1 for UE supporting [TBD - multi-rx capability] according to the conditions described in clause 3.6.y, for the cases of CSI-RS used for L1-SINR measurement of CSI-RS based CMR only case and CSI-RS based CMR plus CSI-RS based ZP-IMR/NZP-IMR case and CSI-RS based CMR plus ZP-IMR case, where N=1 applies as specified in clause 9.8.4, and where the following additional conditions are met:

-	The CSI-RS is not in a CSI-RS resource set with repetition ON.

-	The CSI-RS has same QCL source as the active TCI state of one of the PDSCHs and has different QCL-TypeD from the other PDSCH.

-	The CSI-RS and both of the PDSCHs are on the same OFDM symbol(s).

-	[FFS: The CSI-RS and only one of the PDSCHs with different QCLed typeD are on the same OFDM symbol(s)]

-	Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

-	[FFS how to capture UE is activated with multi-Rx operation]

-	There are no scheduling restrictions due to L1-SINR measurement performed based on the CSI-RS.

-	Otherwise

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the CSI-RS for L1-RSRP measurement symbols to be measured for L1-SINR for FR2 power class 6 UE which is not configured with highSpeedMeasFlagFR2-r17, and for the UE not supporting FR2 power class 6;

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the symbols to be measured for L1-SINR, and on 1 data symbol before symbols to be measured for L1-SINR and 1 data symbol after symbols to be measured for L1-SINR for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17.

When intra-band carrier aggregation is performed, the scheduling restrictions on serving cell where L1-SINR measurement is performed apply to all serving cells in the band on the symbols that fully or partially overlap with restricted symbols.

If following conditions are met,

-	UE has been notified about system information update through paging,

-	The gap between UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set and that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

for the SSB and CORESET for RMSI scheduling multiplexing patterns 3, UE is expected to receive the PDCCH that UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for L1-SINR measurement; and

for the SSB and CORESET for RMSI scheduling multiplexing patterns 2, UE is expected to receive PDSCH that corresponds to the PDCCH that UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for L1-SINR measurement.

## <End of Change #22>

## <Start of Change #23>

#### 9.5.4.2	CSI-RS based L1-RSRP Reporting

When groupBasedBeamReporting-r17 is not configured, the UE shall be capable of performing L1-RSRP measurements based on the configured CSI-RS resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_CSI-RS.

When groupBasedBeamReporting-r17 is configured, the UE shall be capable of performing L1-RSRP measurements based on the two configured CSI-RS resource sets for L1-RSRP, and the UE physical layer shall be capable of reporting group-based L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_CSI-RS.

## <End of Change #23>