3GPP TSG-RAN WG4 Meeting #110bis	R4-2405797

Changsha, China, April 15-19, 2024

Source:	Nokia

Title:	Multi Rx RRM measurement accuracy requirements

Agenda item:	6.3.3.2

Document for:	Discussion

Introduction

Proposals for the performance part based on RAN4#110 meeting contributions were captured in WF in [1]. In this contribution we discuss the accuracy requirements for m-Rx.

Accuracy requirements

The following options regarding the accuracy requirements for multi-Rx L1-RSRP measurements were FFS in the last meeting:

Issue 2-10: Accuracy requirements for multi-Rx in Rel-18 - FFS following options - Option 1a: - The legacy accuracy requirements for L1-RSRP measurement apply for L1-RSRP measurements with group-based beam reporting. - Option 1b: - The legacy accuracy requirements in section 10.1.20 of TS 38.133 apply for L1-RSRP measurements under multi-rx operation, with a clarification that multi-rx chain L1-RSRP accuracy requirements apply for FR2-1. - No new accuracy requirements section is created for L1-RSRP measurements under multi-rx operation. - Option 1c: - Under multi-Rx chain DL reception scenario, the legacy accuracy requirements for L1-RSRP measurement should be satisfied for both Rx chains. - In this sense, no new accuracy requirements section is created for L1-RSRP measurements under multi-rx operation - Option 2: - Do not define group-based L1-RSRP measurement accuracy requirements unless the requirements are applicable only when there are negligible mutual interferences across different pair of Tx-Rx beams.

We see no need to define any new accuracy requirements for group-based or non-group-based L1-RSRP measurements. Both shall fulfil the measurement accuracy requirements in section 10.1.20. We also agree that for the sake for specification completeness Option 1b has been proposed by some companies. We are open to both Option 1b and Option 1c.

Reuse the existing L1-RSRP measurement accuracy requirements for group-based and non-group-based L1-RSRP measurements for a multi-Rx capable UE.

Conclusion

The following observations and proposals were made in this contribution:

Proposal 1: Reuse the existing L1-RSRP measurement accuracy requirements for group-based and non-group-based L1-RSRP measurements for a multi-Rx capable UE.

- References

- R4- 2403560, WF on core maintenance and performance part for multi-Rx WI, Vivo, Ericsson, RAN4#110 meeting, Athens, Greece, 26 February – 01 March, 2024.