3GPP TSG-RAN WG4 Meeting #110bis	R4-2405816

# Changsha, China, April 15th – 19th, 2024

Source:	Nokia

Title:	RRM Performance Requirements for NR Sidelink Evolution

Agenda item:	6.20.3

Document for:	Discussion

Introduction

In RAN4#110, the RRM performance requirements are discussed to identify if any new performance requirement is needed. WF [1] from RAN4#110 captures RRM performance requirements and test cases.

- Sub-topic 2-2: RRM performance requirement for SL-U Accuracy Issue 2-5: Accuracy requirements – RSSI Agreement: - Reuse SL-RSSI accuracy requirements for SL-U congestion control - FFS: Define RSSI accuracy requirements for SL-U LBT measurement, e.g., reuse NR-U RSSI accuracy - FFS how to capture the requirements in the specification (e.g., add it in existing table or introduce new clause) Issue 2-6: Accuracy requirements – RSRP Agreement: - Reuse SL measurement accuracy requirements, such as PSBCH-RSRP, and L1 SL-RSRP for SL-U. Test configuration Issue 2-7: Whether to define new CCA models for SL-U test cases Agreement: - Define new CCA models for SL-U using the NR-U CCA models as baseline. Issue 2-8: Whether to define new RMCs for PSCCH and PSSCH due to CCA. Way forward: - Come back next meeting Test case list Issue 2-12: Test case list for SL-U RRM Agreement: - Introduce the TC for Initiation/Cease of SLSS transmission. - Do not introduce the TC for Selection/Reselection of synchronization Reference source - Do not introduce the TC for UE Transmit timing - Do not introduce the TC for L1 SL-RSRP measurement - Do not introduce the TC for Congestion control measurement - Do not introduce the TC for Interruption Test case; Note; Volunteer Company. Initiation/Cease of SLSS transmission; with SyncRef UE as synchronization reference source under SL-U operation; Huawei.

In this paper, we discuss potential RRM performance requirements and test cases as listed in the WF from RAN4#110.

Discussion

The remaining issues on NR sidelink unlicensed are discussed in this section.

Accuracy

Issue 2-5: Accuracy requirements – RSSI Agreement: - Reuse SL-RSSI accuracy requirements for SL-U congestion control - FFS: Define RSSI accuracy requirements for SL-U LBT measurement, e.g., reuse NR-U RSSI accuracy - FFS how to capture the requirements in the specification (e.g., add it in existing table or introduce new clause)

When SL is operating on an unlicensed carrier, the accuracy requirements need to be specified. The remaining issue on accuracy requirements on SL-U is whether to define RSSI accuracy requirements for SL-U LBT measurement. Since SL-U can measure RSSI, the accuracy requirements should be specified for SL-U. RSSI measurements under CCA are defined in Clause 10.1.34, Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1 are defined in clause 10.4.3. Since the existing clauses do not define RSSI accuracy requirements for SL-U, NR-U RSSI accuracy requirements can be reused as baseline and new RSSI accuracy requirements shall be captured in the clause 10.4.3A

RAN4 to reuse NR-U RSSI accuracy as a baseline for RSSI accuracy requirements for SL-U LBT measurement.

RSSI accuracy requirements shall be captured in the clause 10.4.3A of TS 38.133

Test configuration

Issue 2-8: Whether to define new RMCs for PSCCH and PSSCH due to CCA. Way forward: - Come back next meeting

The reference measurement channels (RMCs) under CCA are defined under clause A.3.1A. Also, in clause A.3.21.3, the RRM test configurations of RMCs in sidelink are defined for PSCCH and PSSCH, respectively. Since the existing test cases do not defined in the case of operating with CCA, a new test considering the RMCs for SL-U may be needed.

RAN4 to define new RMCs for PSCCH and PSSCH due to CCA.

Conclusion

This paper has discussed the RRM performance requirements and test cases for the R18 SL unlicensed. In the paper, the following proposals were made.

- RAN4 to reuse NR-U RSSI accuracy as a baseline for RSSI accuracy requirements for SL-U LBT measurement.

- RSSI accuracy requirements shall be captured in the clause 10.4.3A of TS 38.133

RAN4 to define new RMCs for PSCCH and PSSCH due to CCA.

- References

- R4-2403472, “WF for [110][228] NR_SL_enh2_part2,” OPPO, RAN4#110.