3GPP TSG-RAN WG4 Meeting #110bis	R4-2405849

Changsha, China, 15th April 2024 ‒ 19th April 2024

Agenda item:			6.21.4

Source:	Moderator (Ericsson)

Title:	Topic summary for [110bis][330] NR_redcap_enh_demod

Document for:	Information

# Introduction

This topic summary lists open issues on Demodulation performance part for Rel-18 WI Enhanced support of reduced capability NR devices.

- 6.21.3.1: UE demodulation performance and CSI requirements [NR_redcap_enh-Perf]

# Topic #1: UE demodulation and CSI reporting requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2404329 | Apple | Observation 1: Rel-17 RedCap defines both Rank 1 and Rank 2 PDSCH requirements for 1Rx/2Rx UEs Proposal 1: Define Rank 1 and Rank 2 requirements for PDSCH for 64QAM modulation Observation 2: Use of 256 QAM modulation is an optional feature, as recognized in Rel-17 RedCap Observation 3: Use of 256 QAM modulation would exceed the maximum TBS for Rel-18 RedCap at meaningful PRB allocations Proposal 2: Do not define new requirements for PDSCH involving 256 QAM Proposal 3: RAN4 to discuss whether CQI and RI requirements can be reused by analysing what configurations might need to be adapted. |
| R4-2404330 | Apple | Simulation results |
| R4-2405097 | Ericsson | Proposal 1: Define PDSCH demodulation requirements with 64QAM with rank 1 for both 1Rx/2Rx. Proposal 2: Not to define PDSCH demodulation requirements with 256QAM for eRedCap UE. Proposal 3: Revise the simulation parameters for PDSCH test cases for UE supporting eRedCapNotReducedBB-BW-r18 (TDD SCS=30kHz) as follows: - QPSK: PRB size from 12 to 52, and TBS from 928 to 3840 - 16QAM: PRB size from 25 to 20. Proposal 4: For SDR requirements, add the following sentence in TS 38.101-4 5.5.1: - For eRedCap with reduced peak date rate and reduced baseband bandwidth in FR1 (not supporting supporting eRedCapNotReducedBB-BW-r18), only 25 PRBs for 15kHz SCS and 12RBs for 30kHz SCS are allocated for PDSCH transmission. Proposal 5: Add the definition of eRedCap in TS 38.101-4 3.1: - eRedCap: A UE with enhanced reduced capabilities as specified in clause 4.2 in TS 38.306 [14]. Observation 1: The smallest configurable PRB size for CSI-RS for CSI acquisition is 24 for eRedCap. Proposal 6: Configure the following CSI-RS frequency occupation parameters for CSI reporting requirements for eRedCap. - Frequency occupation for NZP CSI-RS for CSI acquisition: Start PRB 0 and number of PRB = 24 - Frequency occupation for ZP CSI-RS for CSI acquisition: Start PRB 0 and number of PRB = 24 Proposal 7: Configure the following PDSCH PRB allocation parameters for CSI reporting requirements for eRedCap to align with PRBs used for CSI-RS transmission. - For CQI reporting test, add the following note in the CQI-TBS table. Number of allocated PDSCH resource blocks (Note 1); 15; 7. Note 1: PDSCH are allocated from PRB 0. - For PMI reporting test, add the following note in the FRC table. Reference channel; R.PDSCH.x-x.x. Channel bandwidth; MHz; 10. Subcarrier spacing; kHz; 15. Number of allocated resource blocks (Note 1); PRBs; 25. Note 1: PDSCH is allocated from PRB 0.. Proposal 8: Set SNR=5/6dB and SNR=11/12dB for CQI reporting test in static condition for FDD/HD-FDD/TDD with 1Rx, same as Rel-17 RedCap CQI reporting test in static condition. Proposal 9: Set SNR=8/9dB and SNR=14/15dB for CQI reporting test in static condition for FDD/HD-FDD/TDD with 2Rx, same as Rel-17 RedCap CQI reporting test in static condition. Proposal 10: Set SNR=9/10dB and SNR=15/16dB for CQI reporting test in fading condition for FDD/HD-FDD/TDD with 1Rx, same as Rel-17 RedCap CQI reporting test in fading condition. Proposal 11: Set SNR=6/7dB for CQI reporting test in fading condition for FDD/HD-FDD/TDD with 2Rx, same as Rel-17 RedCap CQI reporting test in fading condition. Proposal 12: Apply the same requirements as Rel-17 RedCap UE CQI reporting test in fading condition: - A CQI index not in the set {median CQI - 1, median CQI, median CQI + 1} shall be reported at least α% of the time where α = 20%. - Throughput ratio of follow CQI and fixed median CQI shall be ≥ γ, where γ = 1.05 - Average BLER with follow CQI shall be greater than or equal to 0.02. Proposal 13: Apply the same requirements as Rel-17 RedCap UE PMI reporting test for FDD/HD-FDD/TDD with 1Rx/2Rx: - Define the test metric as γ=tuetrnd , where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue with random precoding based on type I Single Panel codebook. - Set γ=1.3 Proposal 14: RAN4 should discuss the specification structure for eRedCap PDSCH demodulation and CQI/PMI reporting requirements. Proposal 15: Introduce applicability of demodulation requirements for eRedCap UE as follows in TS 38.101-4 5.1.1. Proposal 16: Introduce applicability of CSI reporting requirements for eRedCap UE as follows in TS 38.101-4 6.1.1. |
| R4-2405098 | Ericsson | Simulation results |
| R4-2405099 | Ericsson | Simulation results summary |
| R4-2405160 | Huawei, HiSilicon | Proposal 1: RAN4 to introduce 64QAM and 256QAM requirements for UE supporting or not supporting eRedCapNotReducedBB-BW-r18 Proposal 2: Use tests listed in Table 2-1 for UE supporting or not supporting eRedCapNotReducedBB-BW-r18 Proposal 3: Use CSI-RS configuration in Table 2-2 for CSI test |
| R4-2405161 | Huawei, HiSilicon | Simulation results |
| R4-2405941 | Qualcomm Incorporated | Proposal 1: RAN4 to consider the initial simulation results in the above tables for defining PDSCH demod performance. Proposal 2: Consider deprioritizing 256QAM modulation scheme for PDSCH demod tests. Proposal 3: Specify 64QAM test with rank1 configuration. Proposal 4: RAN4 to discuss CSI-RS BW allocation in CQI tests for eRedCap UEs. |

## Open issues summary

### Sub-topic 1-1	PDSCH demodulation requirements

Background: RAN4#110 agreed to define PDSCH demodulation requirements with QPSK/16QAM for FDD/HD-FDD/TDD with 1Rx/2Rx, however it was FFS whether to define 64QAM/256QAM requirements.

- Additional test case options for FDD/HD-FDD 1Rx/2Rx

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 64QAM 1Rx | 1Rx | MCS19 in Table 1 | 0.50 | Rank 1 | 25 | 9992 | TDLA30-10 |
| 64QAM 2Rx Option 1 | 2Rx | MCS19 in Table 1 | 0.50 | Rank 1 | 25 | 9992 | TDLA30-10 |
| 64QAM 2Rx Option 2 | 2Rx | MCS19 in Table 1 | 0.50 | Rank 2 | 12 | 9480 | TDLA30-10 |
| 256QAM | 1Rx | MCS20 in Table 2 | 0.67 | Rank 1 | 14 | 9736 | TDLA30-10 |
| 256QAM | 2Rx | MCS24 in Table 2 | 0.82 | Rank 1 | 11 | 9480 | TDLA30-10 |

- Additional test case options for TDD 1Rx/2Rx

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 64QAM 1Rx | 1Rx | MCS19 in Table 1 | 0.50 | Rank 1 | 12 | 4736 | TDLA30-10 |
| 64QAM 2Rx Option 1 | 2Rx | MCS19 in Table 1 | 0.50 | Rank 1 | 12 | 4736 | TDLA30-10 |
| 64QAM 2Rx Option 2 | 2Rx | MCS19 in Table 1 | 0.50 | Rank 2 | 6 | 4736 | TDLA30-10 |
| 256QAM | 1Rx | MCS20 in Table 2 | 0.67 | Rank 1 | 7 | 4864 | TDLA30-10 |
| 256QAM | 2Rx | MCS24 in Table 2 | 0.82 | Rank 1 | 5 | 4352 | TDLA30-10 |

Issue 1-1-1: PDSCH test cases for 64QAM

- Proposals

  - Option 1 (Apple, Huawei): 64QAM Rank 1 for 1Rx, and 64QAM Rank 2 for 2Rx (Same as Rel-17).

  - Option 2 (Ericsson, Qualcomm): 64QAM Rank 1 for both 1Rx and 2Rx.

- Recommended WF

  - Define 64QAM UE demodulation requirements.

    - For 1Rx UE, define 64QAM rank 1.

    - For 2Rx UE, discuss 64QAM rank 1 or rank 2.

Issue 1-1-2: PDSCH test cases for 256QAM

- Proposals

  - Option 1 (Apple, Ericsson, Qualcomm): Not define 256QAM demodulation requirements.

  - Option 2 (Huawei): Define 256QAM demodulation requirements.

- Recommended WF

  - Discuss the options.

Issue 1-1-3:  Correction of test setup

Background: RAN4#110 agreed to set the following parameters for PDSCH demodulation requirements for UE supporting eRedCapNotReducedBB-BW-r18 (TDD SCS=30kHz).

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx/2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 12 | 928 | TDLB100-400 |
| 16QAM | 1Rx/2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 25 | 4992 | TDLC300-100 |

- Proposal (Ericsson)

  - Correct the parameters as follows.

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 1Rx/2Rx | MCS4 in Table 1 | 1/3 | Rank 1 | 51 | 3840 | TDLB100-400 |
| 16QAM | 1Rx/2Rx | MCS13 in Table 1 | 0.47 | Rank 1 | 20 | 4992 | TDLC300-100 |

- Recommended WF

  - Moderator expects this is agreeable without discussion.

  - Capture the correction in the simulation assumption.

### Sub-topic 1-2	SDR requirements

Issue 1-2-1: How to capture the applicability

- Proposals (Ericsson)

  - Add the following sentence in TS 38.101-4 5.5.1

For eRedCap with reduced peak date rate and reduced baseband bandwidth in FR1 (not supporting eRedCapNotReducedBB-BW-r18), only 25 PRBs for 15kHz SCS and 12RBs for 30kHz SCS are allocated for PDSCH transmission.

- Recommended WF

  - Discuss the detailed wording during the review of the draft CR.

### Sub-topic 1-3 CSI reporting requirements

Issue 1-3-1:  CSI-RS configuration for CSI reporting requirements

Background: RAN4#110 agreed to adjust the PDSCH PRB size so that it does not exceed eRedCap capability, 10Mbps, however RAN4 has not decided the PRB size for CSI-RS configuration for CSI acquisition.

- Proposals

  - Option 1 (Huawei): Configure BWP size, CSI-RS PRBs and PDSCH PRBs as follows:

|  | CQI tests (static/fading) | PMI test |
| --- | --- | --- |
| FDD SCS=15kHz | BWP:52 RBs CSI-RS: 24RBs PDSCH:15 RBs | BWP:52 RBs CSI-RS: 28RBs PDSCH: 25RBs |
| TDD SCS=30kHZ | BWP:52 RBs CSI-RS: 24RBs PDSCH:7RBs | BWP:52 RBs CSI-RS: 24RBs PDSCH: 15RBs |

  - Option 2 (Ericsson): Start PRB 0 and number of PRB = 24 for NZP-CSI-RS and ZP CSI-RS

    - Specify PDSCH are allocated from PRB 0

  - Option 3 (Qualcomm): Discuss CSI-RS BW allocation in CQI tests for eRedCap UEs.

- Recommended WF

  - Moderator: All the companies agree to align CSI-RS PRB size and PDSCH PRB size as much as possible.

  - Configure the following parameters for CSI reporting tests:

|  | CQI tests (static/fading) | PMI test |
| --- | --- | --- |
| FDD CBW/SCS=10MHz/15kHz | BWP:52 RBs CSI-RS: 24RBs (PRB 0 to 23) PDSCH:15 RBs (PRB 0 to 14)) | BWP:52 RBs CSI-RS: 28RBs (PRB 0 to 27) PDSCH: 25RBs (PRB 0 to 24) |
| TDD CBW/SCS=20MHz/30kHz | BWP:51 RBs CSI-RS: 24RBs (PRB 0 to 23) PDSCH:7RBs (PRB 0 to 6) | BWP:51 RBs CSI-RS: 24RBs (PRB 0 to 23) PDSCH: 15RBs (PRB 0 to 14) |
| Note: BWP size is same as the transmission bandwidth in TS38.101-1 5.3.2. |  |  |

Issue 1-3-2:  CQI reporting test in static condition

- Proposal (Ericsson):

  - Set SNR=5/6dB and SNR=11/12dB for CQI reporting test in static condition for FDD/HD-FDD/TDD with 1Rx, same as Rel-17 RedCap CQI reporting test in static condition.

  - Set SNR=8/9dB and SNR=14/15dB for CQI reporting test in static condition for FDD/HD-FDD/TDD with 2Rx, same as Rel-17 RedCap CQI reporting test in static condition.

- Recommended WF

  - Discuss whether to apply the same test points as Rel-17 RedCap CQI reporting requirements in static condition.

Issue 1-3-3:  CQI reporting test in fading condition

- Proposals (Ericsson):

  - Set SNR=9/10dB and SNR=15/16dB for CQI reporting test in fading condition for FDD/HD-FDD/TDD with 1Rx, same as Rel-17 RedCap CQI reporting test in fading condition

  - Set SNR=6/7dB for CQI reporting test in fading condition for FDD/HD-FDD/TDD with 2Rx, same as Rel-17 RedCap CQI reporting test in fading condition.

  - Apply the same requirements as Rel-17 RedCap UE CQI reporting test in fading condition:

    - A CQI index not in the set {median CQI - 1, median CQI, median CQI + 1} shall be reported at least α% of the time where α = 20%.

    - Throughput ratio of follow CQI and fixed median CQI shall be ≥ γ, where γ = 1.05

    - Average BLER with follow CQI shall be greater than or equal to 0.02.

- Recommended WF

  - Discuss whether to apply the same test points and requirements as Rel-17 RedCap CQI reporting requirements in fading condition.

Issue 1-3-4:  PMI reporting tests

- Proposals (Ericsson)

  - Apply the same requirements as Rel-17 RedCap UE PMI reporting test for FDD/HD-FDD/TDD with 1Rx/2Rx:

    - Define the test metric as γ=tuetrnd , where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue with random precoding based on type I Single Panel codebook.

    - Set γ=1.3.

- Recommended WF

  - Discuss whether to apply the same test points and requirements as Rel-17 RedCap PMI reporting requirements.

### Sub-topic 1-4	Applicability of requirements

Issue 1-4-1: Test applicability of UE demodulation requirements

- Proposals (Ericsson)

  - Add the definition of eRedCap in TS 38.101-4 3.1:

  - Introduce applicability of demodulation requirements for eRedCap UE as follows in TS 38.101-4 5.1.1.

- Recommended WF

  - Discuss whether to add the definition of eRedCap in TS 38.101-4 3.1 and whether to add the following table in TS 38.101-4 5.1.1.

- eRedCap: A UE with enhanced reduced capabilities as specified in clause 4.2 in TS 38.306 [14].

| UE capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| eRedCap with 1RX | FR1 FDD and HD-FDD (Note 1) | PDSCH | Clause 5.2.1.1.x |  |
|  |  | PDCCH | All tests in Clause 5.3.1.1.1 |  |
|  |  | PBCH | All tests in Clause 5.4.1.1 |  |
|  |  | SDR | Clause 5.5.1 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.1.2.x |  |
|  |  | PDCCH | All tests in Clause 5.3.1.2.1 |  |
|  |  | PBCH | All tests in Clause 5.4.1.2 |  |
|  |  | SDR | Clause 5.5.1 |  |
| eRedCap with 2RX | FR1 FDD and HD-FDD (Note 1) | PDSCH | Clause 5.2.2.1.x |  |
|  |  | PDCCH | All tests in Clause 5.3.2.1.4 |  |
|  |  | PBCH | Clause 5.4.2.1 (Table 5.4.2.1-2 Test 1) Clause 5.4.2.1 (Table 5.4.2.1-3 Test 1) |  |
|  |  | SDR | Clause 5.5.1 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.x |  |
|  |  | PDCCH | All tests in Clause 5.3.2.2.4 |  |
|  |  | PBCH | Clause 5.4.2.2 (Table 5.4.2.2-4 Test 1) Clause 5.4.2.2 (Table 5.4.2.2-5 Test 1) |  |
|  |  | SDR | Clause 5.5.1 |  |
| Note 1: If UE support only HD-FDD in a FDD band, this UE is tested with HD-FDD mode otherwise UE is tested with full-duplex FDD mode |  |  |  |  |

Issue 1-4-2: Test applicability of CSI reporting requirements

- Proposals (Ericsson)

  - Introduce applicability of demodulation requirements for eRedCap UE as follows in TS 38.101-4 6.1.1

- Recommended WF

  - Discuss whether to add the following table in TS 38.101-4 6.1.1.

  - Note it depends on the conclusion on the issues 1-4-1, 1-4-2, and 1-5-1 whether to apply the same test points and criteria as Rel-17 RedCap CQI/PMI reporting requirements. If new requirements are specified, may need to add new sections.

| UE capability | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| eRedCap with 1RX | FR1 FDD and HD-FDD (Note 1) | CQI | All tests in Clause 6.2.1.1.1.1 All tests in Clause 6.2.1.1.2.1 |  |
|  |  | PMI | All tests in Clause 6.3.1.1.1 |  |
|  | FR1 TDD | CQI | All tests in Clause 6.2.1.2.1.1 All tests in Clause 6.2.1.2.2.1 |  |
|  |  | PMI | All tests in Clause 6.3.1.2.1 |  |
| eRedCap with 2RX | FR1 FDD and HD-FDD (Note 1) | CQI | All tests in Clause 6.2.2.1.1.4 All tests in Clause 6.2.2.1.2.4 |  |
|  |  | PMI | Clause 6.3.2.1.1 (Test 1) |  |
|  | FR1 TDD | CQI | All tests in Clause 6.2.2.2.1.5 All tests in Clause 6.2.2.2.2.4 |  |
|  |  | PMI | Clause 6.3.2.2.7 (Test 1) |  |
| Note 1: If UE support only HD-FDD in a FDD band, this UE is tested with HD-FDD mode otherwise UE is tested with full-duplex FDD mode |  |  |  |  |

### Sub-topic 1-5	CR work split

Issue 1-5-1: CR work split

Moderator proposes to discuss the CR work split for Rel-18 eRedCap UE demodulation and CSI reporting requirements according to the workplan agreed in RAN4#110.

# Recommendation for Tdocs

| T-doc number | Suggested status | Comments |
| --- | --- | --- |
| R4-2404329 | Noted |  |
| R4-2404330 | Noted |  |
| R4-2405097 | Noted |  |
| R4-2405098 | Noted |  |
| R4-2405160 | Noted |  |
| R4-2405161 | Noted |  |
| R4-2405941 | Noted |  |
| R4-2405099 | Noted | Simulation results summary |