3GPP TSG-RAN WG4 Meeting #110bis	R4-2405886

# Changsha, China, April 15 – 19, 2024

Source:	Nokia

Title:	Simulation results for PRS Bandwidth Aggregation

Agenda item:	6.12.3.4

Document for:	Information

Introduction

The Rel-18 WID [1] for NR positioning contains several enhancements, including PRS/SRS bandwidth aggregation:

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4].

PRS/SRS bandwidth aggregation has been studied in the FS_NR_pos_enh2 study item by RAN4 with company contributions and outcome of the study being captured in TR 38.859 [2].  Based on Rel-16 and Rel-17 link level simulation assumptions for RSTD and PRS-RSRP, respectively, in [3] and [4], RAN4 has agreed link level simulation assumptions for PRS bandwidth aggregation in [5].

The initial preliminary simulation results on PRS RSTD with bandwidth aggregation are provided in [6].  In this contribution we provide updated simulation results for PRS RSTD, and preliminary results on UE Rx-Tx time difference. It is noted that these results do not contain any RF/BB impairments.

Simulation Assumptions

Simulation assumptions for PRS RSTD captured in [5] are considered. The general parameters and PRS configurations as listed in Table 1 and Table 2 were used in the simulation study.

| Parameter | Value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Cell layout | 3 TRPs at distinct locations: <cell 1, cell 2, cell 3>, where TRP 1 is the reference TRP |  |
| Network synchronization | • Synchronous with time shifts <0, 0, 3 us> • Asynchronous with time shifts: <0, 7 symbols, -7 symbols> |  |
| Duplex modes | FDD and TDD |  |
| TDD specific parameters (TDD configuration is in 38.133, section A.3.1.4) | - TDDConf.1.1 (15 kHz) - TDDConf.2.1 (30 kHz) | - TDDConf.3.1 (120 kHz) |
| Data and CCH load in PRS symbols | no other cell transmissions in its positioning symbols, except PRS |  |
| Data and CCH load in non-PRS symbols | - 50% utilization in time - 100% RE utilization |  |
| Cyclic prefix | Normal |  |
| DRX | OFF |  |
| Carrier frequency / BW / SCS / duplex mode | - 2 GHz - 15 kHz - FDD, TDD - 4 GHz - 30 kHz, 60 kHz - FDD, TDD | - 40 GHz - 60 kHz, 120 kHz - TDD |
| Propagation conditions | AWGN TDL-A (30 ns delay spread, 5Hz) | AWGN TDL-C (60 ns delay spread, 300Hz) |
| Es/Iot for three TRPs (cell 1, cell 2, cell 3), [dB] | (-6, -13, -13) for Nsample=4 (-3, -13, -13) for Nsample=4 (-3, -6, -6) for Nsample=2 | (-6, -13, -13) for Nsample =4 (-3, -10, -10) for Nsample =4 (-3, -6, -6) for Nsample=2 |
| Number of samples ( Nsample) | 4 and 2 |  |
| Number of UE receive antennas | 2 |  |
| Number of transmit PRS antennas | 1 |  |
| Cell ID, TRP ID, PRS Resource Set ID | Selected to ensure non-overlapping PRS REs in frequency |  |
| Number of DL PRS Resource sets for a positioning fix | 1 (including all PRS resource repetitions) |  |
| PRS muting | No muting, muting (comb 2) |  |
| Power boosting | No power boosting |  |
| ExpectedRSTD-UncertaintyNOTE 1 | 5 us (15 kHz), 2.5 us (30 kHz), 0.625 us (120 kHz) |  |
| Number of PFLs NOTE 2 | 2 and 3 |  |
| Carrier BW | Smallest channel BW as defined in 38.101 that is larger than the PFL BW |  |
| Carrier spacing | Nominal carrier spacing for intra-band contiguous CA as defined in 38.101 |  |
| NOTE 1: nr-DL-PRS-expectedRSTD-uncerainty is a member of NR-DL-PRS-AssistanceData which is a common NR positioning IE applicable to all positioning methods (see clause 6.4.3 of TS 37.355). NOTE 2: Each PFL belongs to a carrier, and the 2 or 3 carriers to which the PFLs belong to are intra-band contiguous. |  |  |

Table 1: General parameters.

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 104 | 1 | 64 |
|  | 30 | 132 | 1 | 32 |
|  |  | 272 | 1 | 16 |
|  | 60, FR1 | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 60, FR2 | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 120 | 64 | 1 | 16 |
|  |  | 128 | 1 | 8 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| Number of samples | 2, 4 |  |  |  |
| PRS periodicity | 40ms, 200ms |  |  |  |
| TOA estimation | Realistic |  |  |  |
| Path # | First path |  |  |  |

Table 2: PRS transmission configuration parameters for each PFL.

Regarding simulation assumptions, we focused on AWGN channel and TDL-A/TDL-C channels. No RF/BB impairments are included.

Simulation Results

In this section we provide the simulation results for the above listed simulation assumptions. The results are reported for 5-percentile and 95-percentile CDF of the PRS RSTD errors in Tc, and 90-percentile CDF of the UE Rx-Tx time difference errors in Tc for given Es/Iot for three cells [5].

FR1, AWGN Channel

RSTD results

The PRS RSTD error in Tc for AWGN channel in FR1, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells are listed in Table 3, Table 4, and Table 5.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-6, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | -16 | 16 | -16 | 16 |
|  |  |  |  |  |  | 3 | -16 | 16 | -16 | 16 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
|  |  | 272 | 16 | 1 | 4 | 3 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 4 | -2 | 2 | -2 | 2 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |

Table 3: PRS RSTD error for AWGN channel, FR1, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -13) dB |  | RSTD (cell-1, cell-3) (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | -16 | 16 | -16 | 16 |
|  |  |  |  |  |  | 3 | -16 | 16 | -16 | 16 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
|  |  | 272 | 16 | 1 | 4 | 3 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 4 | -2 | 2 | -2 | 2 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |

Table 4: PRS RSTD error for AWGN channel, FR1, in Tc for Es/IoT (-3,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -6) dB |  | RSTD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 2 | 2 | -32 | 32 | -32 | 32 |
|  |  |  |  |  |  | 3 | -32 | 32 | -32 | 32 |
| 30 | 4 | 132 | 32 | 1 | 2 | 2 | -16 | 16 | -16 | 16 |
|  |  |  |  |  |  | 3 | -8 | 8 | -8 | 8 |
|  |  | 272 | 16 | 1 | 2 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
| 60 | 4 | 64 | 32 | 1 | 2 | 2 | -16 | 16 | -16 | 16 |
|  |  |  |  |  |  | 3 | -8 | 8 | -8 | 8 |
|  |  | 132 | 16 | 1 | 2 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |

Table 5: PRS RSTD error for AWGN channel, FR1, in Tc for Es/IoT (-3, -6, -6) dB.

UE Rx-Tx time difference results

The UE Rx-Tx time difference error in Tc for AWGN channel in FR1, derived from the 90-percentile of the CDF, for Es/IoT of three cells are listed in Table 6, Table 7, and Table 8.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | 16 | 16 | 16 |
|  |  |  |  |  |  | 3 | 16 | 16 | 16 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 272 | 16 | 1 | 4 | 3 | 4 | 4 | 4 |
|  |  |  |  |  |  | 4 | 2 | 2 | 2 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |

Table 6: UE Rx-Tx error for AWGN channel, FR1, in Tc for Es/IoT (-6,-13,-13) dB

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | 16 | 16 | 16 |
|  |  |  |  |  |  | 3 | 16 | 16 | 16 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 272 | 16 | 1 | 4 | 3 | 4 | 4 | 4 |
|  |  |  |  |  |  | 4 | 2 | 2 | 2 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |

Table 7: UE Rx-Tx error for AWGN channel, FR1, in Tc for Es/IoT (-3,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 2 | 2 | 16 | 16 | 16 |
|  |  |  |  |  |  | 3 | 16 | 16 | 16 |
| 30 | 4 | 132 | 32 | 1 | 2 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 272 | 16 | 1 | 2 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |
| 60 | 4 | 64 | 32 | 1 | 2 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 132 | 16 | 1 | 2 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |

Table 8: UE Rx-Tx error for AWGN channel, FR1, in Tc for Es/IoT (-3, -6, -6) dB.

FR1, TDL-A Channel

RSTD results

The PRS RSTD error in Tc for TDL-A channel in FR1, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells are listed in Table 9, Table 10, and Table 11.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-6, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | -32 | 40 | -32 | 32 |
|  |  |  |  |  |  | 3 | -32 | 40 | -32 | 40 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | -20 | 20 | -16 | 20 |
|  |  |  |  |  |  | 3 | -8 | 8 | -8 | 8 |
|  |  | 272 | 16 | 1 | 4 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -12 | 12 | -12 | 12 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | -20 | 24 | -20 | 32 |
|  |  |  |  |  |  | 3 | -8 | 10 | -8 | 14 |
|  |  | 132 | 16 | 1 | 4 | 2 | -8 | 8 | -8 | 10 |
|  |  |  |  |  |  | 3 | -12 | 12 | -12 | 12 |

Table 9: PRS RSTD error for TDL-A channel, FR1, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -13) dB |  | RSTD (cell-1, cell-3) (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | -32 | 40 | -32 | 32 |
|  |  |  |  |  |  | 3 | -32 | 40 | -32 | 40 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | -20 | 20 | -16 | 20 |
|  |  |  |  |  |  | 3 | -8 | 8 | -8 | 8 |
|  |  | 272 | 16 | 1 | 4 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -12 | 12 | -12 | 12 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | -20 | 24 | -20 | 32 |
|  |  |  |  |  |  | 3 | -8 | 10 | -8 | 14 |
|  |  | 132 | 16 | 1 | 4 | 2 | -8 | 8 | -8 | 10 |
|  |  |  |  |  |  | 3 | -12 | 12 | -12 | 12 |

Table 10: PRS RSTD error for TDL-A channel, FR1, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -6) dB |  | RSTD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 2 | 2 | -32 | 32 | -32 | 32 |
|  |  |  |  |  |  | 3 | -32 | 32 | -32 | 32 |
| 30 | 4 | 132 | 32 | 1 | 2 | 2 | -16 | 16 | -16 | 16 |
|  |  |  |  |  |  | 3 | -8 | 8 | -8 | 8 |
|  |  | 272 | 16 | 1 | 2 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -12 | 12 | -12 | 12 |
| 60 | 4 | 64 | 32 | 1 | 2 | 2 | -24 | 16 | -16 | 24 |
|  |  |  |  |  |  | 3 | -8 | 8 | -8 | 8 |
|  |  | 132 | 16 | 1 | 2 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -12 | 12 | -12 | 12 |

Table 11: PRS RSTD error for TDL-A channel, FR1, in Tc for Es/IoT (-3, -6, -6) dB.

UE Rx-Tx time difference results

The UE Rx-Tx time difference error in Tc for TDL-A channel in FR1, derived from the 90-percentile of the CDF, for Es/IoT of three cells are listed in Table 12, Table 13, and Table 14.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | 48 | 48 | 48 |
|  |  |  |  |  |  | 3 | 48 | 48 | 48 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | 40 | 40 | 40 |
|  |  |  |  |  |  | 3 | 28 | 30 | 30 |
|  |  | 272 | 16 | 1 | 4 | 2 | 30 | 30 | 30 |
|  |  |  |  |  |  | 3 | 34 | 34 | 34 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | 40 | 40 | 40 |
|  |  |  |  |  |  | 3 | 28 | 32 | 32 |
|  |  | 132 | 16 | 1 | 4 | 2 | 30 | 32 | 32 |
|  |  |  |  |  |  | 3 | 34 | 34 | 34 |

Table 12: UE Rx-Tx error for TDL-A channel, FR1, in Tc for Es/IoT (-6,-13,-13).

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | 48 | 48 | 48 |
|  |  |  |  |  |  | 3 | 48 | 48 | 48 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | 40 | 40 | 40 |
|  |  |  |  |  |  | 3 | 28 | 30 | 30 |
|  |  | 272 | 16 | 1 | 4 | 2 | 30 | 30 | 30 |
|  |  |  |  |  |  | 3 | 34 | 34 | 34 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | 40 | 40 | 40 |
|  |  |  |  |  |  | 3 | 28 | 32 | 32 |
|  |  | 132 | 16 | 1 | 4 | 2 | 30 | 32 | 32 |
|  |  |  |  |  |  | 3 | 34 | 34 | 34 |

Table 13: UE Rx-Tx error for TDL-A channel, FR1, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 2 | 2 | 48 | 48 | 48 |
|  |  |  |  |  |  | 3 | 48 | 48 | 48 |
| 30 | 4 | 132 | 32 | 1 | 2 | 2 | 40 | 40 | 40 |
|  |  |  |  |  |  | 3 | 28 | 28 | 28 |
|  |  | 272 | 16 | 1 | 2 | 2 | 28 | 28 | 28 |
|  |  |  |  |  |  | 3 | 34 | 34 | 34 |
| 60 | 4 | 64 | 32 | 1 | 2 | 2 | 40 | 40 | 40 |
|  |  |  |  |  |  | 3 | 28 | 28 | 28 |
|  |  | 132 | 16 | 1 | 2 | 2 | 28 | 28 | 28 |
|  |  |  |  |  |  | 3 | 34 | 34 | 34 |

Table 14: UE Rx-Tx error for TDL-A channel, FR1, in Tc for Es/IoT (-3, -6, -6) dB.

FR2, AWGN channel

RSTD results

The PRS RSTD error in Tc for AWGN channel in FR2, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells, respective, are listed in Table 15, Table 16, and Table 17.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-6, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |
|  |  | 128 | 8 | 1 | 4 | 2 | -2 | 2 | -2 | 2 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |

Table 15: PRS RSTD error for AWGN channel, FR2, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -13) dB |  | RSTD (cell-1, cell-3) (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | -4 | 4 | -6 | 4 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |
|  |  | 128 | 8 | 1 | 4 | 2 | -2 | 2 | -2 | 2 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |

Table 16: PRS RSTD error for AWGN channel, FR2, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -6) dB |  | RSTD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 2 | 2 | -16 | 16 | -16 | 16 |
|  |  |  |  |  |  | 3 | -8 | 8 | -8 | 8 |
|  |  | 132 | 16 | 1 | 2 | 2 | -8 | 8 | -8 | 8 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
| 120 | 40 | 64 | 16 | 1 | 2 | 2 | -8 | 8 | -8 | 4 |
|  |  |  |  |  |  | 3 | -4 | 4 | -4 | 4 |
|  |  | 128 | 8 | 1 | 2 | 2 | -4 | 4 | -4 | 4 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |

Table 17: PRS RSTD error for AWGN channel, FR2, in Tc for Es/IoT (-3, -6, -6) dB.

UE Rx-Tx time difference results

The UE Rx-Tx time difference error in Tc for AWGN channel in FR2, derived from the 90-percentile of the CDF, for Es/IoT of three cells are listed in Table 18, Table 19, and Table 20.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |
|  |  | 128 | 8 | 1 | 4 | 2 | 2 | 2 | 2 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |

Table 18: UE Rx-Tx error for AWGN channel, FR2, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 132 | 16 | 1 | 4 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |
|  |  | 128 | 8 | 1 | 4 | 2 | 2 | 2 | 2 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |

Table 19: UE Rx-Tx error for AWGN channel, FR2, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 2 | 2 | 8 | 8 | 8 |
|  |  |  |  |  |  | 3 | 4 | 4 | 4 |
|  |  | 132 | 16 | 1 | 2 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |
| 120 | 40 | 64 | 16 | 1 | 2 | 2 | 4 | 4 | 4 |
|  |  |  |  |  |  | 3 | 2 | 2 | 2 |
|  |  | 128 | 8 | 1 | 2 | 2 | 2 | 2 | 2 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |

Table 20: UE Rx-Tx error for AWGN channel, FR2, in Tc for Es/IoT (-3, -6, -6) dB.

FR2, TDL-C channel

RSTD results

The PRS RSTD error in Tc for TDL-C channel in FR2, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells, respective, are listed Table 21, Table 22, and Table 23.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-6, -13) dB/ (-3, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB/ (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | -36 | 40 | -40 | 72 |
|  |  |  |  |  |  | 3 | -38 | 40 | -40 | 40 |
|  |  | 132 | 16 | 1 | 4 | 2 | -38 | 38 | -40 | 38 |
|  |  |  |  |  |  | 3 | -37 | 37 | -37 | 37 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | -46 | 50 | TBD | TBD |
|  |  |  |  |  |  | 3 | -40 | 40 | TBD | TBD |
|  |  | 128 | 8 | 1 | 4 | 2 | -40 | 40 | TBD | TBD |
|  |  |  |  |  |  | 3 | -39 | 39 | -50.325 | 39.5 |

Table 21: PRS RSTD error for TDL-C channel, FR2, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-6, -13) dB/ (-3, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB/ (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | -36 | 40 | -40 | 72 |
|  |  |  |  |  |  | 3 | -38 | 40 | -40 | 40 |
|  |  | 132 | 16 | 1 | 4 | 2 | -38 | 38 | -40 | 38 |
|  |  |  |  |  |  | 3 | -37 | 37 | -36 | 37 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | -46 | 50 | TBD | TBD |
|  |  |  |  |  |  | 3 | -40 | 40 | TBD | TBD |
|  |  | 128 | 8 | 1 | 4 | 2 | -40 | 40 | TBD | TBD |
|  |  |  |  |  |  | 3 | -39 | 39 | -50 | 39.5 |

Table 22: PRS RSTD error for TDL-C channel, FR2, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -6) dB |  | RSTD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 2 | 2 | -48 | 48 | -48 | 48 |
|  |  |  |  |  |  | 3 | -52 | 52 | -52 | 52 |
|  |  | 132 | 16 | 1 | 2 | 2 | -52 | 52 | -52 | 52 |
|  |  |  |  |  |  | 3 | -50 | 50 | -50 | 50 |
| 120 | 40 | 64 | 16 | 1 | 2 | 2 | -56 | 56 | -56 | 56 |
|  |  |  |  |  |  | 3 | -52 | 52 | -52 | 52 |
|  |  | 128 | 8 | 1 | 2 | 2 | -52 | 52 | -52 | 52 |
|  |  |  |  |  |  | 3 | -52 | 52 | -52 | 52 |

Table 23: PRS RSTD error for TDL-C channel, FR2, in Tc for Es/IoT (-3, -6, -6) dB.

UE Rx-Tx time difference results

The UE Rx-Tx time difference error in Tc for TDL-C channel in FR1, derived from the 90-percentile of the CDF, for Es/IoT of three cells are listed in Table 24, Table 25, and Table 26.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | 72 | 72 | 80 |
|  |  |  |  |  |  | 3 | 26 | 26 | 26 |
|  |  | 132 | 16 | 1 | 4 | 2 | 24 | 24 | 24 |
|  |  |  |  |  |  | 3 | 47 | 47 | 47 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | 30 | 32 | TBD |
|  |  |  |  |  |  | 3 | 48 | 48 | TBD |
|  |  | 128 | 8 | 1 | 4 | 2 | 48 | 48 | TBD |
|  |  |  |  |  |  | 3 | 63 | 63 | 63 |

Table 24: UE Rx-Tx error for TDL-C channel, FR2, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | 72 | 72 | 80 |
|  |  |  |  |  |  | 3 | 26 | 26 | 26 |
|  |  | 132 | 16 | 1 | 4 | 2 | 24 | 24 | 24 |
|  |  |  |  |  |  | 3 | 47 | 47 | 47 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | 30 | 32 | TBD |
|  |  |  |  |  |  | 3 | 48 | 48 | TBD |
|  |  | 128 | 8 | 1 | 4 | 2 | 48 | 48 | TBD |
|  |  |  |  |  |  | 3 | 63 | 63 | 63 |

Table 25: UE Rx-Tx error for TDL-C channel, FR2, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 2 | 2 | 80 | 80 | 80 |
|  |  |  |  |  |  | 3 | 36 | 36 | 36 |
|  |  | 132 | 16 | 1 | 2 | 2 | 32 | 32 | 32 |
|  |  |  |  |  |  | 3 | 48 | 48 | 48 |
| 120 | 40 | 64 | 16 | 1 | 2 | 2 | 32 | 32 | 32 |
|  |  |  |  |  |  | 3 | 50 | 50 | 50 |
|  |  | 128 | 8 | 1 | 2 | 2 | 50 | 48 | 48 |
|  |  |  |  |  |  | 3 | 63 | 63 | 63 |

Table 26: UE Rx-Tx error for TDL-C channel, FR2, in Tc for Es/IoT (-3, -6, -6) dB.

Discussion

From the PRS BW aggregated RSTD and UE Rx-Tx time difference simulation results reported above, we observed:

- in general, increasing the PRS BW by aggregating PFLs, improves the accuracy of the positioning measurements.

- if the PFL aggregation does not increase the baseband sampling rate, there may not be improvement in the measurement accuracy. For example, there is no change in the measurement accuracy associated with SCS = 15kHz with number of PFLs = 2 and 3, as going from 2 PFLs to 3PFLs is not increasing the baseband sampling rate.

- accuracy of RSTD measurement increases with the increase in the number of measurement samples (i.e., using 4 measurement samples rather than 2 measurement samples)

- Es/IoT (-6, -13, -13) dB and (-3, -13, -13) dB yields the same results for the current simulation setup. These results imply that Es/IoT (-6, -13, -13) dB is enough to successfully detect the first path.

Thus, following observations are made:

PRS BW aggregation improves the measurement accuracy.

The RSTD/UE Rx-Tx time difference measurement accuracy improves with the increase in the number of measurement samples.

For three PFLs, accuracy improvements for RSTD/UE Rx-Tx time difference measurements over two PFLs are not observed, if there is no change in the baseband sampling rate.

Following proposals ares made.

To define the performance requirements for PRS BW aggregation, use 4 measurement samples as a baseline.

The measurement accuracy requirement for RSTD/UE Rx-Tx time difference should depend on the baseband sampling rate as well as the number of PFLs.

Conclusion

In this contribution we have provided revised simulation results for PRS RSTD error and also UE Rx-Tx time difference error without RF/BB impairments for PRS BW aggregation based on simulation assumptions in [5].

Following observations are made.

- PRS BW aggregation improves the measurement accuracy.

- The RSTD/UE Rx-Tx time difference measurement accuracy improves with the increase in number of measurement samples.

- For three PFLs, accuracy improvements for RSTD/UE Rx-Tx time difference measurements over two PFLs are not observed, if there is no change in the baseband sampling rate.

- Following proposals are made.

- To define the performance requirements for PRS BW aggregation, use 4 measurement samples as a baseline.

The measurement accuracy requirement for RSTD/UE Rx-Tx time difference should depend on the baseband sampling rate as well as the number of PFLs.

- References

| [1] | RP-233382, "Revised WID on Expanded and Improved NR Positioning", 3GPP TSG RAN#102. |
| --- | --- |
| [2] | TR 38.859, "Study on Expanded and Improved NR Positioning (Release 18)", V18.0.0. |
| [3] | R4-2002287, "Updated link level simulation assumption for RSTD and RSRP", Huawei. |
| [4] | R4-2211061, "Simulation assumption for PRS-RSRPP", Huawei. |
| [5] | R4-2321461, "Simulation assumptions for PRS measurement accuracy for PRS/SRS BW aggregation", Huawei, HiSilicon. |
| [6] | R4-2401874, "Simulation results for PRS Bandwidth Aggregation", Nokia, Nokia Shanghai Bell. |