# 3GPP TSG-RAN WG4 Meeting # 110bis				R4-2405898

# Changsha, China, April 15th – 19th, 2024

Agenda item:	6.5.1

Source: 	MediaTek Inc,

Title: 	Discussion on feature list for MGE-2 part 1

Document for:	Discussion

- Introduction

Based on the guidance from RAN4 Chairman sent by email, as below, the feature list for MGE-2 is provided in this document.

In order to facilitate the proceed of the Rel-18 feature list/UE capabilities, I would like invite Xiaoran from CMCC to help organizing and monitoring the discussion. Please Xiaoran help circulating the template for Rel-18 feature list/UE capabilities. It is appreciate that companies can prepare the contribution of Rel-18 feature list based on the template shared by Xiaoran. n Please submit one contribution per companies to the dedicated AI for feature list, i.e., AI 10, which can include the previous agreed UE capabilities and the newly proposed UE capabilities to facilitate the moderator to collect them. If the newly proposed UE capability is for closed Rel-18 WI, the technique discussion can be held under AI 10 and corresponding topic thread. n Please submit discussion papers with details for newly proposed UE capability under the individual agenda for different WIs. The UE capabilities are expected to be discussed in details and agreed under the agenda for each WIs. n The detailed information can be found in the meeting arrangement & guideline document. And the approach is the same as for Rel-17.

- Discussion on feature list for MGE-2 WI

Based on the current progress in MGE-2 for part 1 in the last meeting [1], the following way forward was reached:

<Way Forward> Session Chair: Define in total 3 capabilities for MG enh partl (including both case 1 and case 2), with the condition below. FFS Details. For 32-5, the capability can be further discussed based on R17 progress. Before any agreement reached for Rel-17, 32-5 is not considered.

In general, based on RRM discussion regarding this issue, we believe there is a need to define a UE capability for dynamic collision. This is because the implementation and spec complexity for the dynamic collision is very difficult task to achieve. Besides, having two pre-MG activated simultaneously could lead to higher UE complexities in terms of implementation and thus a UE capability is needed too. In addition, for the two NCSG [32-5], we agree with chairman comment that this UE capability can be further discussed based on R17 progress. Therefore, this contribution includes the updated RAN4 UE feature list for Rel-18 NR.

- NR_MG_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 | Concurrent gap with Pre-MG | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Detail in Table 9.1.x-1 of TS 38.133. | 19-3-x and 19-2 x = 1 or 2 | Yes | No | Network should not configure concurrent gap with Pre-MG | Per UE | No | No |  |  | Optional with capability signalling |
|  | 32-2 | 2 Pre-MG configuration with simultaneous activation/deactivation | Support configurations of 2 Pre-MG with simultaneous activation/deactivation in the same FR. | 32-1 | Yes | No | Network should not configure 2 Pre-MG in the same FR | Per UE | No | No |  |  | Optional with capability signalling |
|  | 32-3 | Dynamic collision | Support the RRM requirements when the activation/deactivation delay of Pre-MG overlaps the other measurement gap or Pre-MG | 32-1 | Yes | No | UE is not expected to meet the requirements | Per UE | No | No |  |  | Optional with capability signalling |
|  | 32-4 | Concurrent gap with NCSG | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Detail in Table 9.1.y-1 of TS 38.133. | 19-1 and 19-2 | Yes | No | Network should not configure concurrent gap with NCSG | Per UE | No | No |  |  | Optional with capability signalling |
|  | [32-5] | 2 NCSG configuration | Support configurations of 2 NCSG in the same FR | 32-4 | Yes | No | Network should not configure 2 NCSG in the same FR | Per UE | No | No |  |  | Optional with capability signalling |

- RAN4 RRM shall send the feature list for MGE-2 part 1 is provided in this contribution.

- Summary

Proposal 1: RAN4 RRM shall send the feature list for MGE-2 part 1 is provided in this contribution.

- References

- R4-2403542, ‘WF on NR_MG_enh2_part1’, MediaTek, Athens, Greece, 2024.