3GPP TSG-RAN WG4 Meeting # 110-bis 	R4-2405901

Changsha, China, April 15th – April 19th, 2024

Agenda Item: 6.5.3

Source: MediaTek Inc.

Title: Discussion on RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG

Document for: Discussion

# Introduction and discussion

In the last meeting, RAN4 agreed on the work-split for test cases for MGE-2 part 1 [1]. Yet, some test cases are still FFS on whether to define them or not. In this contribution document, we provide our views for these FFS test cases.

# Discussion on Concurrent gaps with Pre-MG test cases

From the WF of the last meeting [1]:

< Way Forward > FFS the following test cases: No; Test case category; Test purpose. Con-Pre-MG TC5; Inter-frequency measurement with autonomous activation/deactivation of Pre-MG in FR1 with dynamic collision; - Verify the gap association; - Verify the dynamic gap collision when Pre-MG activation • the UE shall NOT report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Type2 MG occasions. • the UE shall be able to receive PDSCH and report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasion overlapped in dynamic collision. Con-Pre-MG TC6; Inter-frequency measurement with network-controlled activation/deactivation of Pre-MG in FR2 with dynamic collision; · Verify the gap association; · Verify the dynamic gap collision when Pre-MG activation the UE shall NOT report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Type2 MG occasions. • the UE shall be able to receive PDSCH and report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasion overlapped in dynamic collision.

Based on the discussion from the last meeting, RAN4 agreed that if dynamic collision has a separate UE capability then the test cases above shall be defined.

- If a new UE capability for dynamic collision is defined then RAN4 shall define test cases Con-Pre-MG TC5 and Con-Pre-MG TC6.

# Discussion on Concurrent gaps with NCSG test cases

From the WF, the following open issues are captured [1]:

< Way Forward > FFS the following test cases: No; Test case category; Test purpose. Con-NCSG TC3; Event triggered reporting test on inter-frequency in FR2 with concurrent gap and NCSG of two NCSGs; · Inter-frequency cell search delay is met for Cell2 in NCSG1 and Inter-frequency cell search/measurement delay for Cell3 in NCSG2, and · UE receives data in Cell1 meeting scheduling restriction requirement Verify the gap collision rule. Con-NCSG TC4; Event triggered reporting test on deactivated SCell in FR1 with concurrent gap and NCSG; · Intra-frequency cell search/measurement delay for deactivated SCC is met for Cell2 in NCSG, and Inter-frequency cell search/measurement delay for Cell3 in MG · UE receives data in Cell1 meeting scheduling restriction requirements, and UE will not cause any interruption on Cell1 outside VIL windows..

TC Con-NCSG TC3: The case of having two NCSG has no additional challenges compared to existing scenarios, such as concurrent gaps and concurrent gaps with NCSG. In other words, this test case is implicitly tested within other test cases. Therefore, there is no need to define this test case

TC Con-NCSG TC4: The above test case is dependent of the outcome of the issues related to UE behaviour for deactivated SCell measurements with NCSG and given that these issues are still open then RAN4 shall wait for the outcome of those deactivated SCell issue.

- RAN4 shall not define test case Con-NCSG TC3 because the case of two NCSG is implicitly tested within other test cases.

- For test case Con-NCSG TC4, RAN4 shall wait the outcome of the discussion on UE behaviour for deactivated SCell measurements with NCSG.

# Summary

In this contribution, we have the following proposal:

Proposal 1: If a new UE capability for dynamic collision is defined then RAN4 shall define test cases Con-Pre-MG TC5 and Con-Pre-MG TC6.

Proposal 2: RAN4 shall not define test case Con-NCSG TC3 because the case of two NCSG is implicitly tested within other test cases.

Proposal 3: For test case Con-NCSG TC4, RAN4 shall wait the outcome of the discussion on UE behaviour for deactivated SCell measurements with NCSG.

# References

- R4-2403542, “WF on NR_MG_enh2_part1”, MediaTek Inc, Feb-March 2024.