# 3GPP TSG-RAN WG4 Meeting # 110-bis                                                         R4-2405962

# Changsha, China, 15 – 19th April 2024

# Title:	n28 NS_17 PC2

# Agenda Item:	5.19.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This contribution presents power amplifier (PA) measurements to evaluate the back-off required to meet the NS_17 requirements for Power Class 2 (PC2) operation, without filter rejection. Due to insufficient time, we are unable to present measurements to verify the full filter rejection requirements necessary to meet the stringent UE-to-UE coexistence requirements.

# Discussion

## 2.1 Power Amplifier Calibration

- Local Oscillator (LO) leakage: -28 dBc, IQ Image rejection: -28 dB, C-IM3 -60dBc, C-IM5 -70dBc.

- Power Class 3 (PC3) and PC2 operation, and 4dB post PA insertion losses.

- The PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB, at the lower channel edge, with 1 dB MPR:

  - PC3 to meet ACLR -30 dBc

  - PC2 to meet ACLR -31 dBc

- Waveforms: DFT-s-OFDM QPSK, 1RB edge and full RB allocation for 3, 5, and 10MHz channel bandwidth (CBW)

## 2.2 Measurement Results

The measured PA output back-off (OBO) is summarized in Table 2 for PC3, PC3+PC3 and 1xPC2. When no back-off is needed, we present (in brackets “[ ]”) the margin, relative to the NS_17 emission level requirements at MPR 0.

Table 2: Measured DFT-s-OFDM QPSK OBO for n28 NS_17.

| OBO/[Margin @MPR0] | PC3 | PC+PC3 | PC2 |
| --- | --- | --- | --- |
| 3MHz_1RB Edge | [13.8] | [5.3] | n/a |
| 3MHz Full allocation | [3.4] | 1 | n/a |
| 5MHz 1RB Edge | [11.9] | [6.3] | [11.8] |
| 5M Full allocation | [3.9] | 1 | [2] |
| 10MHz 1RB Edge | ~3 | 4.5 | ~2.5 |
| 10M Full allocation | 3.5 | ~4.5 | n/a |

Observation: For PC3+PC3, a minimum of 4.5dB back-off is needed to meet NS_17 requirements, without filter rejection, for DFT-s-OFDM QPSK waveforms. Additional measurements are needed to verify the OBO for CP-OFDM, and to verify the filter rejection requirements to meet the UE-to-UE coexistence requirements.

# Conclusion

In this contribution, we provide a partial set of PA measurements. These measurements indicate that a minimum of 4.5dB back-off is needed to meet the NS_17 requirements for DFT-s-OFDM QPSK waveforms. Additional measurements are needed to evaluate the back-off required for CP-OFDM, and to evaluate the minimum filter rejection necessary to meet the UE-to-UE coexistence requirements.

Observation: For PC3+PC3, a minimum of 4.5dB back-off is needed to meet NS_17 requirements, without filter rejection, for DFT-s-OFDM QPSK waveforms. Additional measurements are needed to verify the OBO for CP-OFDM, and to verify the filter rejection requirements to meet the UE-to-UE coexistence requirements.