3GPP TSG-RAN WG4 Meeting #110bis						                 				    R4-2405969

Changsha, CN, April 15 – April 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Core maintenance for R18 EMR enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-4-5 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The capability “idleInactiveEUTRA-MeasReport-r16” is to indicate whether the UE supports configuration of E-UTRA measurements in RRC_IDLE/RRC_INACTIVE and reporting of the corresponding results upon network request. As agreed in RAN4#106 (R4-2303309), in this WI, enhancement on E-UTRA measurement report is not in the scope. So the capability “idleInactiveEUTRA-MeasReport-r16” is irrevalant of this WI. Issue 2-1-1: scope of improvement on SCell/SCG setup delay - Agreement - RAN4 shall focus on inter-band target cell in FR2. If final solution to be agreed can cover intra-band and FR1 without extra standardization effort, it is unnecessary to exclude these two scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the capability “idleInactiveEUTRA-MeasReport-r16” in 4.7 and 5.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Corresponding RRM requirement would be not accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7, 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

## 4.7	Measurement report for fast CA/DC setup

### 4.7.1	Introduction

The requirements of measurement report for fast CA/DC setup apply in this clause for UE supporting [solution based on existing measurement] and configured with Rel-18 CA/DC candidate frequencies/cells indicated by higher layers.

A UE supporting [solution based on existing measurement] and idleInactiveNR-MeasReport-r16 shall report based on the idle mode measurement which is specified in clause 4.4 with fulfil the requirement specifiy in clause 4.7.3. If UE does not support idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 but UE supports [solution based on existing measurement], UE shall report based on the idle mode measurement which is specified in clause 4.2.2 with fulfill the requirement specified in clause 4.7.3.

### 4.7.2	Void

### 4.7.3	Measurement Report Requirements

A UE supporting [solution based on existing measurement] capability shall be able to report valid measurement results. The measurement results are considered valid if the following conditions are met:

-	the measurements are performed within the last X seconds before msg1 transmission for RRC resume/setup request, where X is configured by [TBD], and

-	the measurement results satisfy measurement accuracy requirement at the measurement instance.

Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when X is configured.

If the [X] is not configured, UE is not required to perform validity check and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

End of Change 1

Start of Change 2

## 5.8	Measurement report for fast CA/DC setup

### 5.8.1	Introduction

The requirements of measurement report for fast CA/DC setup apply in this clause for UE supporting [solution based on existing measurement] and configured with Rel-18 CA/DC candidate frequencies/cells indicated by higher layers.

A UE supporting [solution based on existing measurement] and idleInactiveNR-MeasReport-r16 shall report based on the inactive mode measurement which is specified in clause 5.4 with fulfil the requirement specifiy in clause 5.8.3. If UE does not support idleInactiveNR-MeasReport-r16 but UE supports [solution based on existing measurement], UE shall report based on the inactive mode measurement which is specified in clause 5.1.2 with fulfill the requirement specified in clause 5.8.3.

### 5.8.2	Void

### 5.8.3	Measurement Report Requirements

The requirements in clause 4.7.3 shall apply.

End of Change 2