3GPP TSG-RAN WG4 Meeting # 110 bis  				      R4-2405992

# Changsha, Hunan Province, CN, 15  – 19 Apr, 2024

Source: 	ZTE Corporation

Title: 	WF for NR_netcon_repeater_RFConformance for EMC conformance

Agenda Item:	6.18.4

Document for:	Approval

- Introduction

During RAN4#110 bis meeting, the issues from moderator summary[1] have been well discussed. Therefore based on online and offline discussion, the following WF was formulated.  The work split of NCR EMC Perf are also capture in this WF. Companies are welcomed to submit draft CR in next meeting.

- Way Forward

The open issues, agreements and tentative agreements reached in RAN4#110 bis meeting are listed below.

Agreements:

Proposal 1: The performance metrics for NCR-MT shall follow the BS approach, which is throughput≥95%

Proposal 2: The performance metrics for NCR-Fwd shall follow the R17 repeater approach, which is Power accuracy, with the value no more than ± 2 dB for NCR-Fwd type 1-C and type 1-H, and no more than ± 3 dB for NCR-Fwd type 2-O.

Proposal 3: The RSE limits for NCR shall follow the tables below.

Table 1: RSE limit for NCR EMC

|  | Type 1-C | Type 1-H | Type 2-O (NOTE 2) |
| --- | --- | --- | --- |
| NCR-Fwd | Reuse NR repeater(38.114) | Reuse NR repeater(38.114) | Reuse NR repeater(38.106) |
| WA NCR-MT | Reuse BS (38.113) | Reuse BS (38.113) | Reuse NR repeater(38.106) |
| LA NCR-MT | Reuse UE (38.124) | Reuse UE (38.124) | [Reuse UE (38.101-2)] (NOTE 3) |
| NOTE 1: When NCR-Fwd and NCR-MT are transmitting simultaneously, the RSE requirements should apply for sum of NCR-MT and NCR-Fwd transmission. NOTE 2: The limits for NCR type 2-O should be pointed to corresponding RF Spec. NOTE 3: The RSE limit for LA NCR-MT requires confirmation on the MU related derivation (UE-based, vs. repeater-based). |  |  |  |

Proposal 4: For radiated emission test, NCR-Fwd and NCR-MT should be monitored together, if they are working simultaneously.

Proposal 5: FFS on the testability of monitoring NCR-Fwd and NCR-MT simultaneously in immunity test. Monitoring NCR-Fwd and NCR-MT simultaneously will reduce the testing time but is expected to increase the complexity of test set up. Related test setup analyses/diagrams are encouraged for next meeting.

Work Split for NCR EMC Perf

Companies shall follow the instruction of the work split table and submit the draft CR in RAN4#111 meeting. A big CR that merge all of the corresponding draft CR will be merged.

Table 2: Work split for NCR EMC Perf

| Clause |  | Company |
| --- | --- | --- |
| 4. Test conditions |  |  |
| 4.1 General | Some editorial changes corresponding to NCR are required. | Ericsson |
| 4.2 Arrangements for establishing a communication link | For NCR-Fwd reuse the NR-ARFCN for BS. For NCR-MT reuse the NR-ARFCN for UE. | Nokia |
| 4.3 Narrow band responses | Some editorial changes corresponding to NCR are required. | Huawei |
| 4.4 Exclusion bands | Reuse the exclusion band of NR repeater but introduce the content of NCR | Huawei |
| 4.5 NR repeaters test configurations | Wait until the discussion of RF conformance test | Huawei |
| 5. Performance assessment |  |  |
| 5.1 General | Some editorial changes corresponding to NCR are required. | Ericsson |
| 5.2 NR repeaters | Introduce the performance assessment method for NCR-Fwd and NCR-MT. For NCR-Fwd it is recommend to reuse the parameter of NR repeater. For NCR-MT it is recommend to use the parameter of throughput. | Ericsson |
| 5.3 Ancillary equipment | No change |  |
| 6. Performance criteria |  |  |
| 6.1 Performance criteria for continuous phenomena for NR repeaters | No change |  |
| 6.2 Performance criteria for transient phenomena for NR repeaters | No change |  |
| 6.3 Performance criteria for continuous phenomena for NCR | Introduce the performance criteria for NCR-Fwd and NCR-MT. For NCR-Fwd it is recommend to reuse the parameter of NR repeater(power accuracy). For NCR-MT it is recommend to use the parameter of throughput. | Nokia |
| 6.4 Performance criteria for transient phenomena for NCR | Introduce the performance assessment method for NCR-Fwd and NCR-MT. For NCR-Fwd it is recommend to reuse the parameter of NR repeater(power accuracy). For NCR-MT it is recommend to use the parameter of throughput. | Nokia |
| 6.5 Performance criteria for continuous phenomena for Ancillary equipment | No change |  |
| 6.6 Performance criteria for transient phenomena for Ancillary equipment | No change |  |
| 8. Emission |  |  |
| 8.1 Test configurations | Some editorial changes corresponding to NCR are required. | ZTE |
| 8.2 Radiated emission | Follow the instruction described in Table 1. | ZTE |
| - Immunity |  |  |
| 9.1 Test configurations | Some editorial changes corresponding to NCR | ZTE |
| 9.2-9.7 | Some editorial changes corresponding to NCR | ZTE |

- References

- R4-2405829	Topic summary for [110bis][309] NR_netcon_repeater_RFConformance