# 3GPP TSG-RAN WG4 Meeting #110bis	R4-2406002

# Changsha, China, 15th – 19th April, 2024

Title: 	Way Forward for [110bis][306] NR_NTN_enh_Part2 Extreme conditions and MU

Agenda Item:	6.16.4

Source: 	Huawei, HiSilicon

Document for:	Approval

# Extreme testing conditions for NTN SAN (Ka band)

Way forward 1: Consider the following options (with other not precluded) for SAN extreme testing conditions decision next meeting:

- Option 1: Do not consider extreme conditions testing;

- Option 2: Consider to introduce new dedicated set of optional manufacturer declarations in TS 38.181, table 4.6-1, and Annex B as below (Track Changes):

| Condition | Minimum | Maximum |
| --- | --- | --- |
| Barometric pressure | D.106 | D.107 |
| Relative humidity | D.108 | D.109 |
| Vibration | D.110 | D.111 |
| Additional conditions | D.112 | D.112 |
| NOTE: Additional extreme test conditions might be decided outside 3GPP, according to the specific mission requirements, together with operator and manufacturer customizations. |  |  |

Way forward 2: If Option 2 in Way Forward 1 is agreed, agree that extreme conditions testing for NTN SAN is a in band-agnostic topic, i.e. not to re-open this discussion for new NTN frequency range or band(s), e.g. Ku, etc.

## MU values (Ka band)

Way forward 3: Reuse of MU values for 24.25-29.5 GHz range for the purpose of SAN OTA conformance testing in Ka band is possible only under the following conditions (reuse from TS 38.181, clause 4.1):

The test tolerances for the radiated test requirements (TTOTA) for SAN were reused from TR 37.941 [13]. Reuse of TR 37.941 [13] TTOTA values for SAN LEO radiated conformance testing is subject to the following conditions:

-	EUT suitability to fit OTA chambers considered in TR 37.941 [13], and

-	Environmental test conditions assumed for BS testing in TR 37.941 [13].

Reuse of TR 37.941 [13] TTOTA values for SAN GEO radiated conformance testing may not be justified for some products due to too large SAN GEO antenna array dimensions, and required OTA RF chamber size.

## References

[1]	R4-2405928	Discussion on the Extreme testing conditions for NTN SAN (Ka band)	Huawei, HiSilicon

[2]	R4-2405929	Discussion on the MU values for Ka band	Huawei, HiSilicon