3GPP TSG-RAN WG4 Meeting #110bis	R4-2406008

# Changsha, China, Apr. 15th - Apr. 19th,2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-4: Minimum requirements and Measurement Channel for mDCI non-overlapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRx_DL_Demod |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Corrections to wrong TCI state numbering and fix update of reference to correlation matrix and clause still containing X |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected wrong TCI state numbering. Updated reference to correlation matrix section. Fixed reference to clause in Note still containing x |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TCI states are wrongly numbered. Reference to correlation matrix is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2404532 |  |  |  |  |  |  |  |  |

## << Start of change 1>>

##### 7.2.2.2.5	Minimum requirements for Multi-DCI non-overlapping based transmission scheme

The performance requirements are specified in Table 7.2.2.2.5-3 with the addition of the parameters in Table 7.2.2.2.5-2 and the downlink physical channel setup according to Annex C.5.1.

The test purposes are specified in Table 7.2.2.2.5-1.

Table 7.2.2.2.5-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance when UE is configured two different values of CORESETPoolIndex in ControlResourceSet and when UE receives multiple PDCCHs scheduling non-overlapping PDSCHs | 1-1 |

Table 7.2.2.2.5-2: Test Parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| SSB |  |  |  |  | SSB#0 | SSB#1 |
| PDCCH configuration |  | TCI state |  |  | TCI State #0 | TCI State #1 |
|  |  | CORESETPoolIndex |  |  | 0,1 |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 160 |  |
|  |  | CSI-RS offset |  | Slots | 80 for CSI-RS resources 1 and 2 81 for CSI-RS resources 3 and 4 | 80 for CSI-RS resources 5 and 6 81 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 | TCI state #1 |
| CSI-RS for beam refinement |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2 | k0=1 for CSI-RS resources 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 | 1 for CSI-RS resource 3,4 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 160 |  |
|  |  | CSI-RS offset |  | Slots | 0 for CSI-RS resources 1,2 | 0 for CSI-RS resources 3, 4 |
|  |  | QCL info |  |  | TCI state #2 | TCI state #3 |
| PTRS |  | Frequency density (KPT-RS) |  |  | 2 | 2 |
|  |  | Time density (LPT-RS) |  |  | 1 | 1 |
|  |  | Resource Element Offset |  |  | 2 | 2 |
| Duplex mode |  |  |  |  | TDD |  |
| Active DL BWP index |  |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |
|  | k0 |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  | 2 |  |
|  | Length (L) |  |  |  | 12 |  |
|  | PRB bundling type |  |  |  | Static |  |
|  | PRB bundling size |  |  |  | 2 |  |
|  | Resource allocation type |  |  |  | Type 1 |  |
|  | RBG size |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |
|  | Num CDM groups without data |  |  |  | 2 | 2 |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | {1000, 1001} | {1002, 1003} |
|  | TCI state |  |  |  | TCI State #0 | TCI State #1 |
|  | DMRS Type |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |
| TCI State #0 | Type 1 QCL information |  | SSB resource |  | SSB #0 | N/A |
|  |  |  | QCL Type |  | Type C | N/A |
|  | Type 2 QCL information |  | SSB resource |  | SSB #0 | N/A |
|  |  |  | QCL Type |  | Type D | N/A |
| TCI State #1 | Type 1 QCL information |  | SSB resource |  | N/A | SSB #1 |
|  |  |  | QCL Type |  | N/A | Type C |
|  | Type 2 QCL information |  | SSB resource |  | N/A | SSB #1 |
|  |  |  | QCL Type |  | N/A | Type D |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type D | N/A |
| TCI State #3 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type D |
| Resource allocation |  |  |  |  | Non-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  | us | 0.25 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  | Hz | 0 |  |
| Number of HARQ Processes |  |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Precoding configuration |  |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |  |

Table 7.2.2.2.5-3: Minimum performance

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition(Note 1) | Correlation matrix and antenna configuration(Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB)(Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 | R.PDSCH.5-2.4 TDD | R.PDSCH.5-2.5 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 4x4 FR2- mTRxP-mRX ρ = -12dB | 70 | [15.1] |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix according to the RFR2-mTxRP-mRX in B.2.3.3. TRxP#1 uses TX antenna indices (1,2) and TRxP#2 uses TX antenna indices (3,4) corresponding to the respective antenna configuration matrix rows. Note 3: SNR is defined per UE Rx chain. SNR of Rx chain i (i=1,2) is derived based on Es from TRxP#i, as defined in 4.5.2. |  |  |  |  |  |  |  |  |  |

## << End of change 1>>