3GPP TSG-RAN4 Meeting #110Bis	R4-2406009

# Changsha, China, Apr 15 - Apr 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on PDSCH demod requirements for mDCI fully-overlapping with multi-RX in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-Perf |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 7.2.2.2.6 is under clause 7.1.1.3 Requirements need to be updated TBS size in FRC needs to be updated |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved Clause 7.2.2.2.6 to clause 7.2.2.2 Updated the requirements based on latest simultion result summary Updated TBS size in FRC table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for mDCI fully-overlapping with multi-RX will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3, 7.2.2.2, A.3.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change #1

#### 7.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 7.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 7.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |  |
| --- | --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR2-1 TDD | PDSCH | Clause 7.2.2.2.1 (Test 3-1) |  |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 7.5A.1 | 1) Up to 16 DL carriers 2) Same numerology across carrier for data/control channel at a given time |  |
| PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots) | FR2-1 TDD | PDSCH | Clause 7.2.2.2.2 |  |  |
| Alternative 64QAM MCS table for PDSCHNew 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) | FR2 TDD | PDSCH | Clause 7.2.2.2.2 |  |  |
| DRX Adaptation (drx-Adaptation-r16) | FR2-1 TDD | PDCCH | Clause 7.3.2.2.3 | If the Test 3-1 in Clause 7.3.2.2.3 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 7.3.2.2.1. |  |
| 256QAM for PDSCH (pdsch-256QAM-FR2) | FR2-1 TDD | PDSCH | Clause 7.2.2.2.1 (Test 1-4) |  |  |
| 256QAM for PDSCH (pdsch-256QAM-FR2) | FR2-1 TDD | SDR | Clause 7.5A.1 | For UE capable of pdsch-256QAM-FR2 for certain band(s), mcs-Table is configured to ‘64QAM’ for SDR test. |  |
| Support of FR2 HST operation [(FR2 UE power class PC6 signalling is used to indicate support of feature group)] | FR2-1 TDD | PDSCH | [Clause 7.2.2.2.4] |  |  |
| Support of Single Carrier operations with 120kHz SCS for FR2-2 (initialAccessSSB-120kHz-r17) | FR2-2 TDD | PDSCH | Clause 7.2.2.2.1 (Table 7.2.2.2.1-6: Test 4-1, 4-2, 4-3, 4-4) |  |  |
|  |  | PDCCH | Clause 7.3.2.2 (Table 7.3.2.2.1-2: Test 1a-1, 1a-2, 1a-3) (Table 7.3.2.2.2-2, Test 3-1, 3-2) |  |  |
|  |  | PBCH | Clause 7.4.2.2 (Table 7.4.2.2-2: Test 3) |  |  |
| Support of 480kHz SCS for FR2-2 (ul-FR2-2-SCS-480kHz-r17 and initialAccessSSB-480kHz-r17) | FR2-2 TDD | PDSCH | Clause 7.2.2.2.1 (Table 7.2.2.2.1-6: Test 4-5, 4-6) |  |  |
|  |  | PDCCH | Clause 7.3.2.2 (Table 7.3.2.2.1-2: Test 1a-4) (Table 7.3.2.2.2-2, Test 3-3) |  |  |
|  |  | PBCH | Clause 7.4.2.2 (Table 7.4.2.2-2: Test 4) |  |  |
| Support simultaneous reception with different QCL Type-D RSs (simultaneousReceptionDiffTypeD-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.X1 Clause 7.2.2.2.6 Clause 7.2.2.2.7 |  |  |
| Single DCI based SDM transmission for simultaneous reception support (singleDCI-SDM-scheme-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.7 |  |  |
| Multi DCI based simultaneous reception non-overlapping support (multiDCI-MultiTRP-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.X1 |  |  |
| Multi DCI based simultaneous reception fully-overlapping support (overlapPDSCHsFullyFreqTime-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.6 |  |  |
| Support of 2-port DL PTRS (supportTwoPortDL-PTRS-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.7 Test 1-2 |  |  |

##### 7.2.2.2.6	Minimum requirements for PDSCH Multi-DCI based transmission scheme

The performance requirements are specified in Table 7.2.2.2.6-3, with the addition of test parameters in Table 7.2.2.2.6-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 7.2.2.2.6-1.

Table 7.2.2.2.6-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance when UE is configured two different values of CORESETPoolIndex in ControlResourceSet and when UE receives multiple PDCCHs scheduling fully-overlapping PDSCHs with simultaneous reception | 1-1 |

Table 7.2.2.2.6-2: Test parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| SSB |  |  |  |  | SSB#0 | SSB#1 |
| PDCCH configuration |  | TCI state |  |  | TCI State #0 | TCI State #1 |
|  |  | CORESETPoolIndex |  |  | 0,1 |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 160 |  |
|  |  | CSI-RS offset |  | Slots | 80 for CSI-RS resources 1 and 2 81 for CSI-RS resources 3 and 4 | 80 for CSI-RS resources 5 and 6 81 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 | TCI state #1 |
| CSI-RS for beam refinement |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2 | k0=1 for CSI-RS resources 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 | 1 for CSI-RS resource 3,4 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 160 |  |
|  |  | CSI-RS offset |  | Slots | 0 for CSI-RS resources 1,2 | 0 for CSI-RS resources 3, 4 |
|  |  | QCL info |  |  | TCI state #2 | TCI state #3 |
| PTRS |  | Frequency density (KPT-RS) |  |  | 2 | 2 |
|  |  | Time density (LPT-RS) |  |  | 1 | 1 |
|  |  | Resource Element Offset |  |  | 2 | 0 |
| Duplex mode |  |  |  |  | TDD |  |
| Active DL BWP index |  |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |
|  | k0 |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  | 2 |  |
|  | Length (L) |  |  |  | 12 |  |
|  | PRB bundling type |  |  |  | Static |  |
|  | PRB bundling size |  |  |  | 2 |  |
|  | Resource allocation type |  |  |  | Type 1 |  |
|  | RBG size |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |
|  | Num CDM groups without data |  |  |  | 2 | 2 |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | {1000} | {1002} |
|  | TCI state |  |  |  | TCI State #0 | TCI State #1 |
|  | DMRS Type |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |
| TCI State #0 | Type 1 QCL information |  | SSB resource |  | SSB #0 | N/A |
|  |  |  | QCL Type |  | Type C | N/A |
|  | Type 2 QCL information |  | SSB resource |  | SSB #0 | N/A |
|  |  |  | QCL Type |  | Type D | N/A |
| TCI State #1 | Type 1 QCL information |  | SSB resource |  | N/A | SSB #1 |
|  |  |  | QCL Type |  | N/A | Type C |
|  | Type 2 QCL information |  | SSB resource |  | N/A | SSB #1 |
|  |  |  | QCL Type |  | N/A | Type D |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type D | N/A |
| TCI State #3 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type D |
| Resource allocation |  |  |  |  | Fully-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  | us | -0.0625 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  | Hz | 600 |  |
| Number of HARQ Processes |  |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Precoding configuration |  |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |  |

Table 7.2.2.2.6-3: Minimum performance

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition(Note 1) | Correlation matrix and antenna configuration(Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB)(Note 3) |
| 1-1 | TRxP #1 | TRxP #2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | TDLA30-75 | 4x4 FR2- mTRxP-mRX =-12dB | 70 | [14.3] |
|  | R.PDSCH.5-3.3 TDD | R.PDSCH.5-3.3 TDD |  |  |  |  |  |  |  |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix according to the FR2-mTxRP-mRX in B.2.3.3. TRxP#1 uses TX antenna indices (1,2) and TRxP#2 uses TX antenna indices (3,4) corresponding to the respective antenna configuration matrix rows. Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 |  |  |  |  |  |  |  |  |  |

Change #2

#### 7.2.2.2	TDD

<Unchanged sections omitted>

##### 7.2.2.2.6	Minimum requirements for PDSCH Multi-DCI based transmission scheme

The performance requirements are specified in Table 7.2.2.2.6-3, with the addition of test parameters in Table 7.2.2.2.6-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 7.2.2.2.6-1.

Table 7.2.2.2.6-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance when UE is configured two different values of CORESETPoolIndex in ControlResourceSet and when UE receives multiple PDCCHs scheduling fully-overlapping PDSCHs with simultaneous reception | 1-1 |

Table 7.2.2.2.6-2: Test parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| SSB |  |  |  |  | SSB#0 | SSB#1 |
| PDCCH configuration |  | TCI state |  |  | TCI State #0 | TCI State #1 |
|  |  | CORESETPoolIndex |  |  | 0,1 |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 160 |  |
|  |  | CSI-RS offset |  | Slots | 80 for CSI-RS resources 1 and 2 81 for CSI-RS resources 3 and 4 | 80 for CSI-RS resources 5 and 6 81 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 | TCI state #1 |
| CSI-RS for beam refinement |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2 | k0=1 for CSI-RS resources 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 | 1 for CSI-RS resource 3,4 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 160 |  |
|  |  | CSI-RS offset |  | Slots | 0 for CSI-RS resources 1,2 | 0 for CSI-RS resources 3, 4 |
|  |  | QCL info |  |  | TCI state #2 | TCI state #3 |
| PTRS |  | Frequency density (KPT-RS) |  |  | 2 | 2 |
|  |  | Time density (LPT-RS) |  |  | 1 | 1 |
|  |  | Resource Element Offset |  |  | 2 | 0 |
| Duplex mode |  |  |  |  | TDD |  |
| Active DL BWP index |  |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |
|  | k0 |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  | 2 |  |
|  | Length (L) |  |  |  | 12 |  |
|  | PRB bundling type |  |  |  | Static |  |
|  | PRB bundling size |  |  |  | 2 |  |
|  | Resource allocation type |  |  |  | Type 1 |  |
|  | RBG size |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |
|  | Num CDM groups without data |  |  |  | 2 | 2 |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | {1000} | {1002} |
|  | TCI state |  |  |  | TCI State #0 | TCI State #1 |
|  | DMRS Type |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |
| TCI State #0 | Type 1 QCL information |  | SSB resource |  | SSB #0 | N/A |
|  |  |  | QCL Type |  | Type C | N/A |
|  | Type 2 QCL information |  | SSB resource |  | SSB #0 | N/A |
|  |  |  | QCL Type |  | Type D | N/A |
| TCI State #1 | Type 1 QCL information |  | SSB resource |  | N/A | SSB #1 |
|  |  |  | QCL Type |  | N/A | Type C |
|  | Type 2 QCL information |  | SSB resource |  | N/A | SSB #1 |
|  |  |  | QCL Type |  | N/A | Type D |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type D | N/A |
| TCI State #3 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type D |
| Resource allocation |  |  |  |  | Fully-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  | us | -0.0625 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  | Hz | 600 |  |
| Number of HARQ Processes |  |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Precoding configuration |  |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |  |

Table 7.2.2.2.6-3: Minimum performance

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition(Note 1) | Correlation matrix and antenna configuration(Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB)(Note 3) |
| 1-1 | TRxP #1 | TRxP #2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | TDLA30-75 | 4x4 FR2- mTRxP-mRX =-12dB | 70 | [14.8] |
|  | R.PDSCH.5-3.3 TDD | R.PDSCH.5-3.3 TDD |  |  |  |  |  |  |  |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix according to the RFR2-mTxRP-mRX in B.2.3.3. TRxP#1 uses TX antenna indices (1,2) and TRxP#2 uses TX antenna indices (3,4) corresponding to the respective antenna configuration matrix rows. Note 3: SNR is defined per UE Rx chain. SNR of Rx chain i (i=1,2) is derived based on Es from TRxP#i, as defined in 4.5.2. |  |  |  |  |  |  |  |  |  |

<Unchanged sections omitted>

Change #3

Table A.3.2.2.5-3: PDSCH Reference Channel for TDD UL-DL pattern FR2.120-1 (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-3.1 TDD | R.PDSCH.5-3.2 TDD | R.PDSCH.5-3.3 TDD |  |  |
| Channel bandwidth | MHz | 100 | 100 | 100 |  |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |  |
| Allocated resource blocks | PRBs | 66 | 66 | 66 |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |  |
| For Slots i = 80, 81 |  | 13 | N/A | 12 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 8 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 | 13 | 12 |  |  |
| Allocated slots per 2 frames |  | 127 | 125 | 127 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 18 | 17 | 17 |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM |  |  |
| Target Coding Rate |  | 0.46 | 0.43 | 0.43 |  |  |
| Number of MIMO layers |  | 1 | 1 | 1 |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |  |
| For Slots i = 80, 81 |  | 12 | N/A | 24 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 12 | 12 | 24 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 12 | 12 | 24 |  |  |
| Overhead for TBS determination |  | 6 | 6 | 6 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 80, 81 | Bits | 25104 | N/A | 19464 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 16136 | 15112 | 11272 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 25104 | 23568 | 19464 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 80, 81 | Bits | 24 | N/A | 24 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 | 24 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |  |
| For Slots i = 80, 81 | CBs | 3 | N/A | 3 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 2 | TBA | 2 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 3 | TBA | 3 |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |  |
| For Slots i = 80, 81 | Bits | 52470 | N/A | 43164 |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 36630 | 35640 | 27324 |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79,82,…,159} | Bits | 54846 | 54648 | 45540 |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 145.062 | 136.1272 | 110.489 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |