3GPP TSG-RAN WG4 Meeting # 110bis	R4-2406016

# Changsha, China, April 14th- 19th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | Draft CR | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR: Introduction of Reference measurement channel for DSS enhancement demodulation requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DSS_enh |  |  |  |  |  | Date: |  |  | 2024-03-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on WF R4-2402873, Huawei is responsible for CR work: Reference measurement channel for DSS enhancement demodulation requirement FDD and TDD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the reference measurement channels |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reference measurement channels will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.1.1, A.3.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

A.3.3	Reference measurement channels for PDCCH performance requirements

A.3.3.1	FDD

A.3.3.1.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.3.1.1-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-1.1 FDD | R.PDCCH.1-1.2 FDD | R.PDCCH.1-1.3 FDD |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |  |
| CORESET frequency domain allocation |  | 48 | 48 | 48 |  |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 |  |  |  |
| Aggregation level |  | 4 | 4 | 8 |  |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 |  |  |  |
| Payload (without CRC) | Bits | 39 | 52 | 52 |  |  |  |

Table A.3.3.1.1-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-2.1 FDD | R.PDCCH.1-2.2 FDD | R.PDCCH.1-2.3 FDD | R.PDCCH.1-2.4 FDD | R.PDCCH.1-2.5 FDD | R.PDCCH.1-2.6 FDD | R.PDCCH.1-2.7 FDD | R.PDCCH.1-2.8 FDD | R.PDCCH.1-2.9 FDD |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| CORESET frequency domain allocation |  | 24 | 24 | 24 | 48 | 48 | 48 | 48 | 48 | 48 |
| CORESET time domain allocation |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Aggregation level |  | 2 | 4 | 2 | 4 | 8 | 16 | 8 | 4 | 8 |
| DCI Format |  | 1_0 | 1_0 | 1_1 | 1_1 | 1_1 | 1_0 | 2_6 | 1_0 | 1_0 |
| Payload (without CRC) | Bits | 39 | 39 | 52 | 52 | 52 | 39 | 12 | 39 | 39 |

A.3.3.2	TDD

A.3.3.2.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.3.2.1-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-1.1 TDD | R.PDCCH.1-1.2 TDD | R.PDCCH.1-1.3 TDD |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |  |
| CORESET frequency domain allocation |  | 48 | 48 | 48 |  |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 |  |  |  |
| Aggregation level |  | 4 | 4 | 8 |  |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 |  |  |  |
| Payload (without CRC) | Bits | 39 | 52 | 52 |  |  |  |

Table A.3.3.2.1-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-2.1 TDD | R.PDCCH.1-2.2 TDD | R.PDCCH.1-2.3 TDD | R.PDCCH.1-2.4 TDD | R.PDCCH.1-2.5 TDD | R.PDCCH.1-2.6 TDD | R.PDCCH.1-2.7 TDD | R.PDCCH.1-2.8 TDD |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| CORESET frequency domain allocation |  | 24 | 24 | 24 | 48 | 48 | 48 | 48 | 48 |
| CORESET time domain allocation |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Aggregation level |  | 2 | 4 | 2 | 4 | 8 | 16 | 4 | 8 |
| DCI Format |  | 1_0 | 1_0 | 1_1 | 1_1 | 1_1 | 1_0 | 1_0 | 1_0 |
| Payload (without CRC) | Bits | 39 | 39 | 52 | 52 | 52 | 39 | 39 | 39 |