3GPP TSG-RAN WG4 Meeting #110bis	R4-2406024

# Changsha, China, April 15th – 19th, 2024

Agenda item:			6.16.9

Source:	Huawei, HiSilicon

Title:	WF on NR_NTN_enh_SAN_UE_demod

Document for:	Approval

- Introduction

This contribution is the way forward for NR NTN enhancement demod for both SAN and UE. The following topics are captured:

- Topic #1: General

- Topic #2: UE demodulation performance requirements

- Topic #3: SAN demodulation performance requirements

Channel model

Issue 1-1: Doppler Spread for both UL and DL

- Agreement

  - 1200Hz

UE demodulation performance requirements

Issue 2-1: PDSCH demodulation requirements for above 10 GHz bands

- Agreement:

| Prop. Channel | MCS | HARQ Config |
| --- | --- | --- |
| NTN-TDLC5-1200 | MCS4 | Disabled HARQ (4/16) |
|  | MCS4 | 16 HARQ Proc |
| NTN-TDLC5-1200 | MCS4 | 32 HARQ Proc |
|  | MCS13 | 16 HARQ Proc |

Issue 2-2: How to define requirements for GSO and NGSO for above 10 GHz bands

- Agreement

  - Define one set of performance requirements for both NGSO and GSO

    - Only consider K_offset = [8] that corresponding to 64 slots for 120kHz SCS

Issue 2-3: PDCCH demodulation requirements for above 10 GHz bands

- Agreement

- AL8 and AL16, with the following test configurations

| Case | Bandwidth (MHz) | SCS (kHz) | CORESET RB | CORESET duration | Aggregation level | CCE to REG mapping type | REG bundle size | Interleaver size | Shift index | DCI format | Payload (without CRC) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 200 | 120 | 60 | 1 | 8 | Interleaved | 2 | 3 | 0 | [1-0] | [40] |
| 2 | 200 | 120 | 60 | 2 | 16 | Interleaved | 2 | 3 | 0 | [1-1] | [56] |

Issue 2-4: Antenna configuration

- Agreement

  - 1Tx1Rx

Issue 2-5: Applicability rule

- Agreement

  - All demodulation performance requirements for NTN enhancements are applicable for Type 1 fixed and Type 2 mobile VSAT UEs

| UE feature/capability [TBD] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| NR NTN access (nonTerrestrialNetwork-r17) | FR2-NTN | PDSCH | Clause x (Testxx) |  |
|  |  | PDCCH | Clause y (Test 1, Test 2) |  |
| Increasing the number of HARQ processes (max-HARQ-ProcessNumber-r17) | FR2-NTN | PDSCH | Clause x (Testxx) |  |
| Disabled HARQ feedback for downlink transmission (harq-FeedbackDisabled-r17) | FR2-NTN | PDSCH | Clause x (Testxx) |  |

- SAN demodulaton requirements

Test setup for normal PUSCH with CP-OFDM for above 10 GHz bands

Issue 3-1-1: MCS

- Agreement

  - MCS 2 and [MCS 12]

Issue 3-1-2: PTRS configuration

- Agreement

  - Not configure PTRS for test with MCS 2 and [MCS12]

Test setup for PUCCH for above 10 GHz bands

Issue 3-2-1: UCI info

- Agreement

  - The UCI information can include both ACK/NACK and CSI part 1 for specifying PUCCH UCI BLER performance requirement.

Issue 3-2-2: Additional DMRS configuration for PUCCH format 3/4

- Agreement

  - Define PUCCH format 3/4 requirements with additional DMRS configuration both enabled and disabled.

  - Reuse the applicability of requirements for different configurations in section 8.1.2.3.4 of TS 38.181

Test setup for PUSCH with DM-RS bundling for FR1 UL coverage enhancement

Issue 3-3-1: RB assignment

- Agreement

  - 6RBs in the middle of the test bandwidth for both 15KHz and 30KHz

Issue 3-3-2: PUSCH aggregation factor

- Agreement

  - n4 for 15kHz SCS

  - n8 for 30kHz SCS

Issue 3-3-3: pusch-TimeDomainWindowLength

- Agreement

  - 4 for 15kHz SCS

  - 8 for 30kHz SCS

- CR splits

| 38.101-5 | Big CR | Huawei |
| --- | --- | --- |
| 11 | Radiated performance requirements | Nokia |
| 11.1 | General |  |
| 11.2 | Demodulation performance requirements |  |
| 11.2.1 | General |  |
| 11.2.2 | PDSCH demodulation requirements | Huawei |
| 11.2.2.1 | 1RX requirements |  |
| 11.2.3 | PDCCH demodulation requirements | Apple |
| 11.2.3.1 | 1RX requirements |  |
| Annex A | Measurement channels | Ericsson |
| A.3.2.1.4 | Reference measurement channels for PDSCH performance requirements |  |
| A.3.3 | Reference measurement channels for PDCCH performance requirements | Qualcomm |
| Annex B | Propagation conditions |  |

| 38.108 | Big CR | Nokia |
| --- | --- | --- |
| 8.2.5 | Performance requirements for PUSCH with DM-RS bundling | Samsung |
| 11 | Radiated performance requirements | Huawei |
| 11.1 | General |  |
| 11.2 | Performance requirements for PUSCH |  |
| 11.3 | Performance requirements for PUCCH | Nokia |
| 11.4 | Performance requirements for PRACH | CATT |
| Annex A | Reference measurement channels | Nokia |
| Annex D | Propagation conditions |  |

| 38.181 | Big CR | Ericsson |
| --- | --- | --- |
| 8.2.5 | Performance requirements for PUSCH with DM-RS bundling | Nokia |
| 11.2.5 | Performance requirements for PUSCH with DM-RS bundling |  |
| 4.1.2.4 | Measurement of performance requirements | Huawei |
| 4.6 | Manufacturer declarations |  |
| 11 | Radiated performance requirements |  |
| 11.1 | General |  |
| 11.2 | OTA performance requirements for PUSCH | Samsung |
| 11.3 | OTA performance requirements for PUCCH | Huawei |
| 11.4 | OTA performance requirements for PRACH | CATT |
| Annex A | Reference measurement channels | Ericsson |
| Annex C | Test tolerances and derivation of test requirements |  |
| Annex D | Measurement system set-up |  |
| Annex G | Propagation conditions |  |

- Reference

- R4-2402865, WF on NR_NTN_enh_SAN_UE_demod, RAN4#110, Huawei, HiSilicon

- R4-2405146 Simulation assumption on demodulation requirements for NR NTN enhancements, RAN4#110bis, Huawei, HiSilicon