3GPP TSG-WG4 Meeting #110bis 	R4-2406027

# Changsha, China, Apr 15 - Apr 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_ENDC_RF_FR1_enh2-Perf] Introduction of 8Rx CA Performance Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-04-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of PDSCH CA Requirements for 8Rx. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated tables in Clause 5.2A, introduction of 8Rx CA PDSCH Requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 8Rx PDSCH CA requirements not be included in TS 38.101-4 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.2A.4 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Revision of R4-2404128 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

5.2A	PDSCH demodulation requirements for CA

The parameters specified in Table 5.2-1 for PDSCH single carrier tests are reused for PDSCH CA tests unless otherwise stated.

Table 5.2A-1: Common test parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | FDD: 12TDD: Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | As defined in Table 5.2A-2 |
| TDD UL-DL pattern |  |  | 15kHz SCS: FR1.15-1 30kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table 5.2A-3 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 for cases where the number of ACK/NACK to be transmitted on single PUCCH is 2 or less. PUCCH format 3 for cases where the number of ACK/NACK to be transmitted on single PUCCH is more than 2. |

Table 5.2A-2: Test parameters for number of HARQ processes

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | For CC with Rank 2: 10 For CC with Rank 8: 8 | 8 |
| FDD 15 kHz + TDD 15 kHz CA | FDD PCell | 4 | 4 |
|  | TDD PCell | 8 | 8 |
| TDD 15 kHz + TDD 30 kHz CA | 15kHz PCell | 8 | 12 |
|  | 30kHz PCell | 8 | 8 |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | 4 | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 8 | N/A |

Table 5.2A-3: Test parameters for K1 values

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,11} For CC with Rank 8: {8,7,6,5,5,4,3} | {7,5,4,11,9} |
| FDD 15 kHz + TDD 15 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | {4,3,2,6} | {4,3,2,6,5} |
| TDD 15 kHz + TDD 30 kHz CA | 15kHz PCell | {4,3,2,6} | {4,4,3,3,2,2,6,6} |
|  | 30kHz PCell | {8,7,6,5,5,4,3,2} | {7,5,4,11} |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | {2} | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,2} For CC with Rank 8: {8,7,6,5,5,4,3} | N/A |

<End of change 2>

# <Start of change 2>

5.2A.4	8RX requirements

5.2A.4.1	Minimum requirements

For CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.4.1-7 or Table 5.2A.4.1-8 (dependent on Baseline or Simplified SU-MIMO 8Rx Receiver), the requirements are based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.4.1-1 ~ Table 5.2A.4.1-6, with the parameters in Table 5.2A-1 ~ Table 5.2A-3 and the downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

Table 5.2A.4.1-1: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank 2, Baseline SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | [R.PDSCH.1-22.1 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.6] |
| 10 | R.PDSCH.1-2.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.5] |
| 15 | [R.PDSCH.1-22.2 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.6] |
| 20 | [R.PDSCH.1-22.3 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.7] |
| 25 | [R.PDSCH.1-22.4 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.7] |
| 30 | [R.PDSCH.1-23.1 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.7] |
| 30 | [R.PDSCH.1-23.2 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.7] |
| 40 | [R.PDSCH.1-23.3 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.8] |
| 45 | [R.PDSCH.1-23.4 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.9] |
| 50 | [R.PDSCH.1-23.5 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.9] |

Table 5.2A.4.1-2: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank 2, Simplified SU-MIMO 8Rx  Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | [R.PDSCH.1-22.1 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.6] |
| 10 | R.PDSCH.1-2.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.7] |
| 15 | [R.PDSCH.1-22.2 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.7] |
| 20 | [R.PDSCH.1-22.3 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.8] |
| 25 | [R.PDSCH.1-22.4 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.8] |
| 30 | [R.PDSCH.1-23.1 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.9] |
| 30 | [R.PDSCH.1-23.2 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.8] |
| 40 | [R.PDSCH.1-23.3 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.9] |
| 45 | [R.PDSCH.1-23.4 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [16.0] |
| 50 | [R.PDSCH.1-23.5 FDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [16.1] |

Table 5.2A.4.1-3: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank 8

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | [R.PDSCH.1-20.1 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [22.5] |
| 10 | R.PDSCH.1-3.7 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [22.6] |
| 15 | [R.PDSCH.1-20.2 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [22.8] |
| 20 | [R.PDSCH.1-20.3 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [23.0] |
| 25 | [R.PDSCH.1-20.4 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [22.9] |
| 30 | [R.PDSCH.1-23.1 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [23.4] |
| 35 | [R.PDSCH.1-23.2 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [23.4] |
| 40 | [R.PDSCH.1-23.3 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [23.7] |
| 45 | [R.PDSCH.1-23.4 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [23.7] |
| 50 | [R.PDSCH.1-23.5 FDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [23.6] |

Table 5.2A.4.1-4: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 2, Baseline SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | [R.PDSCH.2-36.1 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.1] |
| 10 | [R.PDSCH.2-36.2 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.5] |
| 15 | [R.PDSCH.2-36.3 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.5] |
| 20 | [R.PDSCH.2-36.4 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.5] |
| 25 | [R.PDSCH.2-36.5 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.6] |
| 30 | [R.PDSCH.2-36.6 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.6] |
| 40 | R.PDSCH.2-3.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.7] |
| 50 | [R.PDSCH.2-37.1 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.7] |
| 60 | [R.PDSCH.2-37.2 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.7] |
| 80 | [R.PDSCH.2-37.3 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.9] |
| 90 | [R.PDSCH.2-37.4 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [13.9] |
| 100 | [R.PDSCH.2- 37.5 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [14.0] |

Table 5.2A.4.1-5: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 2, Simplified SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | [R.PDSCH.2-36.1 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.7] |
| 10 | [R.PDSCH.2-36.2 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.8] |
| 15 | [R.PDSCH.2-36.3 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.8] |
| 20 | [R.PDSCH.2-36.4 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.8] |
| 25 | [R.PDSCH.2-36.5 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.9] |
| 30 | [R.PDSCH.2-36.6 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.9] |
| 40 | R.PDSCH.2-3.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.8] |
| 50 | [R.PDSCH.2-37.1 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [16.0] |
| 60 | [R.PDSCH.2-37.2 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [15.9] |
| 80 | [R.PDSCH.2-37.3 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [16.1] |
| 90 | [R.PDSCH.2-37.4 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [16.3] |
| 100 | [R.PDSCH.2- 37.5 TDD] | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | [16.3] |

Table 5.2A.4.1-6: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 8

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | [R.PDSCH.2-34.1 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [22.5] |
| 10 | [R.PDSCH.2-34.2 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [22.5] |
| 15 | [R.PDSCH.2-34.3 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [22.8] |
| 20 | [R.PDSCH.2-34.4 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [22.7] |
| 25 | [R.PDSCH.2-34.5 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [22.8] |
| 30 | [R.PDSCH.2-34.6 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [23.2] |
| 40 | R.PDSCH.2-3.7 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [23.1] |
| 50 | [R.PDSCH.2-35.1 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [22.9] |
| 60 | [R.PDSCH.2-35.2 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [23.4] |
| 80 | [R.PDSCH.2-35.3 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [23.4] |
| 90 | [R.PDSCH.2-35.4 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [23.7] |
| 100 | [R.PDSCH.2- 35.5 TDD] | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | [23.8] |

Table 5.2A.4.1-7: Minimum performance for multiple CA configurations, Baseline Receiver

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Tables 5.2A.4.1-1, and 5.2A.4.1-3 per CC |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-4, and 5.2A.4.1-6 per CC |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-1, 5.2A.4.1-3, 5.2A.4.1-4, and 5.2A.4.1-6 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. Note 2: For CA combinations between 8Rx and 4Rx or 2Rx, Rank 2 requirements in Tables 5.2A.4.1-1 and 5.2A.4.1-4 shall be applied for both CCs. Note 3: For CA Combinations with two 8Rx CCs, Rank 8 requirements in Tables 5.2A.4.1-3 and 5.2A.4.1-6 shall be applied for both CCs. |  |  |

Table 5.2A.4.1-8: Minimum performance for multiple CA configurations, Simplified Receiver

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.4.1-2 per CC |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-5 per CC |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-2, and 5.2A.4.1-5 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. Note 2: For CA combinations between 8Rx and 4Rx or 2Rx, Rank 2 requirements in Tables 5.2A.4.1-2 and 5.2A.4.1-5 shall be applied for both CCs. |  |  |

<End of change 2>