3GPP TSG-RAN4 Meeting # 110bis	R4-2406051

# Changsha, China, April 15 – 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | Draft | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft Big CR to TS38.101-4 on Sidelink enhancement demod performance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-04-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce demodulation performance for NR_SL_enh2-Perf |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes are: - Endorsed draft CR R4-2406046 - Add PSCCH decoding capability tests for different BWs. - Endorsed draft CR R4-2406047 - Add PSSCH demodulation requirements for SL-CA based on single carrier performance. - Endorsed draft CR R4-2406048 - Capture PSSCH requirements, Applicability rules, RMC and corresponding transmission burst model. - Endorsed draft CR R4-2406049 - Adding a new section (11.1.9A) to introduce PSFCH decoding capability test for CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be incomplete enhancement on sidelink demodulation performance specifications in TS 38.101-4. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.1.2, A.6.2.2, B.5 New clauses: 11.1.1.1.3, 11.1.1.1.4, 11.1.2.1.2, 11.1.2A, 11.1.8A, 11.1.9A, B.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE 1----------------------------

11	V2X requirements

This clause contains the performance requirements for the sidelink physical channels specified for V2X Sidelink Communication.

11.1	Demodulation performance requirements (Conducted requirements)

### 11.1.1	General

11.1.1.1	Applicability of requirements

11.1.1.1.1	General

The minimum performance requirements are applicable to all V2X operating bands defined in TS 38.101-1[6] Clause 5.2E.

The minimum performance requirements in Clause 11.1 are mandatory for UE supporting NR SL operation (sl-Reception-r16), except test cases listed in Clauses 11.1.1.1.2.

11.1.1.1.2	Applicability of requirements for mandatory UE V2X features with capability signalling

The performance requirements in Table 11.1.1.1.2-1 shall apply for V2X UEs which support mandatory UE features with capability signalling only.

Table 11.1.1.1.2-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Support of synchronization sources for NR sidelink (sync-Sidelink-r16) | FR1 | PSSCH | Clause 11.1.2.1.1 Clause 11.1.6.1.1 Clause 11.1.7.1.1 Clause 11.1.2.1.2 Clause 11.1.2A Clause 11.1.8A Clause 11.1.9A |  |
|  |  | PSCCH | Clause 11.1.3.1.1 Clause 11.1.8.1.1 |  |
|  |  | PSBCH | Clause 11.1.4.1.1 |  |
|  |  | PSFCH | Clause 11.1.5.1.1 Clause 11.1.9.1.1 |  |
| Supports of PSFCH format 0 (psfch-FormatZeroSidelink-r16) | FR1 | PSSCH | Clause 11.1.2.1.1 Clause 11.1.6.1.1 Clause 11.1.7.1.1 Clause 11.1.2.1.2 Clause 11.1.2A Clause 11.1.8A Clause 11.1.9A |  |
|  |  | PSCCH | Clause 11.1.3.1.1 Clause 11.1.8.1.1 |  |
|  |  | PSFCH | Clause 11.1.5.1.1 Clause 11.1.9.1.1 |  |

11.1.1.1.3	Applicability of requirements for CA

The performance requirement for sidelink CA UE demodulation and decoding capability tests in Clause 11.1.2A, 11.1.8A and 11.1.9A are defined independent of CA configurations and bandwidth combination sets specified in Clause 5.3E.1A of TS 38.101-1. For simplicity, CA configuration below refers to combination of CA configuration and bandwidth combination set.

Table 11.1.1.1.3-1: Applicability and test rules for sidelink CA UE demodulation and decoding capability tests

| Tests | CA configuration | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- |
| Test 1 in Clause 11.1.2A.1.1 | Table 5.3E.1A-1 of TS38.101-1 | Largest aggregated CA bandwidth combination |
| Test 1 in Clause 11.1.8A.1.1 | Table 5.3E.1A-1 of TS38.101-1 | Largest aggregated CA bandwidth combination |
| Test 1 in Clause 11.1.9A.1.1 | Table 5.3E.1A-1 of TS38.101-1 | Largest aggregated CA bandwidth combination |

11.1.1.1.4	Applicability of requirements for PSSCH with shared spectrum access

The applicability rules of requirements for PSSCH with shared spectrum access are captured in Table 11.1.1.1.4-1

Table 11.1.1.1.4-1: Requirements applicability for PSSCH with shared spectrum access

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Support of NR SL in shared spectrum(sl-DynamicChannelAccess-r18) | FR1 | PSSCH | Clause 11.1.2.1.2 |  |
| Support of interlace RB-based SL transmissons and receptions (sl-Interlace-RB-TxRx-r18) | FR1 | PSSCH | Clause 11.1.2.1.2 |  |

----------------------END OF CHANGES 1----------------------------

----------------------START OF CHANGE 2----------------------------

11.1.2	PSSCH demodulation requirements

11.1.2.1	2Rx requirements

11.1.2.1.1	Minimum requirements

<Unchanged skipped>

11.1.2.1.2	Minimum requirements for PSSCH with shared spectrum access

The purpose of the requirements in this subclause is to verify the PSSCH for V2X demodulation performance with shared spectrum access with interlace RB allocation

The minimum requirements are specified in Table 11.1.2.1.2-2 with the test parameters specified in Table 11.1.2.1.2-1. In this test scenario, GNSS or GNSS-equivalent synchronization source is used and sidelink UE 1 transmits PSCCH and PSSCH.

Table 11.1.2.1.2-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| SL transmission model |  |  | As specified in B.5.2 |
| Resource pool configuration | Number of interlaces per sub-channel within a resource pool (sl-NumInterlacePerSubchannel-r18) |  | 1 |
|  | Reference number of PRBs of one interlace within 1 RB set for TBS determination (sl-NumReferencePRBs-OfInterlace) |  | 11 |
|  | Reference number of symbols for TBS determination. (sl-NumRefSymbolLength) |  | 14 |
|  | CPE starting position within the GP symbol before PSFCH transmission(sl-CPE-StartingPositionPSFCH ) |  | Not configured |
|  | PSFCH transmission structure (sl-TransmissionStructureForPSFCH-r18) |  | CommonInterlace |
|  | Number of dedicated PRBs for PSFCH (sl-NumDedicatedPRBs-ForPSFCH-r18) |  | 1 |
|  | The common interlace index for PSFCH (sl-PSFCH-CommonInterlaceIndex-r18) |  | 0 |
|  | Number of PSFCH occasiopns corresponding to one PSSCH/PSCCH transmission. (sl-NumPSFCH-Occasions-r18) |  | 1 |
| Sidelink Transmisson Model parameters | SL transmission duration values | Slots | {2,4,8} |
|  | Channel access procedure |  | Type 1 |
|  | LBT failure probability (pLBT) |  | [0.25] |
| Sidelink UE 1 | Sidelink transmissions |  | PSCCH + PSSCH |
|  | PSSCH DMRS pattern (Note 1) |  | {2,2} |
|  | Allocated RBs |  | Subchannel 0 |
|  | Timing offset (Note 2) | s | CP/2-12*64*Tc |
|  | Frequency offset (Note 3) | Hz | +650 |
|  | Synchronization |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 Low |
| PSFCH resource period |  | Slot | 4 |
| MinTimeGapPSFCH |  | Slot | 3 |
| Note 1: {x, y}: x and y means the number of DMRS symbols for slot with PSFCH transmission and without PSFCH transmission, respectively. Note 2: Time offset of transmitted Sidelink UE signal with respect to GNSS referring timing. Note 3: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. |  |  |  |

Table 11.1.2.1.2-2: Minimum performance

| Test num. | Reference channel | Bandwidth (MHz)/ Subcarrier spacing(kHz) | Modulation format and code rate | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PSSCH BLER (%) | SNR(dB) of PSSCH |
| 1 | R.PSSCH.2-1.6 | 20 / 30 | 16QAM, 0.37 | TDLA30-195 | 10 % | [9.3] |

----------------------END OF CHANGES 2----------------------------

----------------------START OF CHANGE 3----------------------------

## 11.1.2A	PSSCH demodulation requirements for CA

## 11.1.2A.1	2Rx requirements

## 11.1.2A.1.1	Minimum requirements

## The purpose of the requirements in this subclause is to verify the PSSCH for NR SL demodulation performance for CA based on single carrier performance.

## For CA with various bandwidth combination of DL component carriers, the requirements are defined in Table 11.1.2A.1.1-3 based on the single carrier requirements for different bandwidth specified in Table 11.1.2A.1.1-2 with the addition of test parameters in Table 11.1.2A.1.1-1.

Table 11.1.2A.1.1-1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| Sidelink UE 1 | Sidelink transmissions |  | PSCCH + PSSCH |
|  | PSSCH DMRS pattern (Note 1) |  | {2,3} |
|  | Index of sub-channel allocation |  | [0,1] |
|  | Timing offset (Note 2) | s | CP/2-12*64*Tc |
|  | Frequency offset (Note 3) | Hz | +600 |
|  | Synchronization |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 Low |
| PSFCH resource period |  | Slot | 4 |
| MinTimeGapPSFCH |  | Slot | 3 |
| Note 1: {x, y}: x and y means the number of DMRS symbols for slot with PSFCH transmission and without PSFCH transmission, respectively. Note 2: Time offset of transmitted Sidelink UE signal with respect to GNSS referring timing. Note 3: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. |  |  |  |

Table 11.1.2A.1.1-2: Single carrier performance for CA configurations

| Test num. | Reference channel (Note 1) | Bandwidth (MHz)/ Subcarrier spacing(kHz) | Modulation format and code rate | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PSSCH BLER (%) | SNR(dB) of PSSCH |
| 1 | R.PSSCH.2-1.2 | 10 / 30 | 16QAM, 0.37 | TDLA30-1400 | 10% | 8.8 |
| 2 | R.PSSCH.2-1.2 | 20 / 30 | 16QAM, 0.37 | TDLA30-1400 |  | 8.8 |
| 3 | R.PSSCH.2-1.2 | 30 / 30 | 16QAM, 0.37 | TDLA30-1400 |  | 8.8 |
| 4 | R.PSSCH.2-1.2 | 40 / 30 | 16QAM, 0.37 | TDLA30-1400 |  | 8.8 |
| Note 1: Reference measurement channel R.PSSCH.2-1.2 can be used at all bandwidth regardless the value of CBW in the reference measurement channel table. |  |  |  |  |  |  |

Table 11.1.2A.1.1-3: Minimum performance for CA configuration

| Test number | CA Bandwidth combination | Minimum performance requirement |
| --- | --- | --- |
| 1 | Largest aggregated CA bandwidth combination | As defined in Table11.1.2A.1.1-2 |

----------------------END OF CHANGES 3----------------------------

----------------------START OF CHANGE 4----------------------------

11.1.8A	PSCCH decoding capability test for CA

11.1.8A.1	2RX requirements

11.1.8A.1.1	Minimum requirements

The purpose of this test is to verify the maximum number of received PSCCHs per TTI supported by the V2X UE.

The minimum requirements are specified in Table 11.1.8.1.1-2 with the test parameters specified in Table 11.1.8.1.1-1 and the test procedure specified as follows for each carrier with index k:

-	Nk UEs transmit PSCCHs and corresponding PSSCHs to the tested UE per slot with each UE occupying one subchannel.

-	xk UEs transmit PSCCHs and corresponding PSSCHs with high priority level on xi subchannels that are randomly selected from Nk subchannels per slot and Nk - xk UEs transmit PSCCHs and corresponding PSSCHs with low priority level on the remaining subchannels. The indication of priority level specified in Clause 5.4.3.3 of TS 23.287 [12] and Clause 5.22.1.3.1 of TS 38.321 [8] is included in PSCCH.

Where Nk is the number of subchannels in the bandwidth defined in the test; [the number xk is derived by first evenly distributing the number of PSFCH(s) resources that the tested UE can transmit in a slot (i.e. [IE psfch-TxNumber] specified in clause 4.2.16.1.6 of  TS 38.306 [14]) across all carriers, and carrier k gets yk, then xi = min(yk , Nk).]

The probability of PSCCH miss detection is calculated as follows:

ProbPSCCH miss detection = #(missing ACK/NACK)#(Tx high priority PSCCH/PSSCH)

Where:

-	# (Tx high priority PSCCH/PSSCH) denotes the total number of transmitted PSCCH/PSSCH with high priority level.

-	# (missing ACK/NACK) denotes the total number of missing ACK/NACK with high priority.

Table 11.1.8A.1.1-1:  Test Parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Member ID (Note 1) |  |  |  | 0 |
| Sidelink UE i, 0 ≤ i ≤ Nk (Note 5) | Sidelink Transmissions |  |  | PSCCH + PSSCH |
|  | Timing offset (Note 2) |  | s | 0 |
|  | Frequency offset (Note 3) |  | Hz | 0 |
|  | Synchronization source |  |  | GNSS |
|  | Propagation Channel |  |  | Static propagation condition without external noise |
|  | Antenna configuration |  |  | 1x2 Low |
|  | PSSCH RMC |  |  | R.PSSCH.2-1.4 |
|  | PSCCH RMC (Note 4) |  |  | R.PSCCH.2-1.1 |
|  | Source ID |  |  | 0 |
|  | PSFCH periodicity |  | Slots | 1 |
|  | MinTimeGapPSFCH |  | Slots | 2 |
|  | PSFCH Resource (Note 6) | RB index |  | 10*i |
|  |  | CS pair index |  | 0 |
| Note 1: Member ID is an identifier uniquely identifying a member. Note 2: Time offset of transmitted Sidelink UE signal with respect to GNSS reference timing. Note 3: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. Note 4: OCC index for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission as per in Clause 8.4.1.3.2 of TS 38.211[9]. Note 5: Each UE occupies one sub-channel so that all sub-channels are filled. Nk is the total number of UEs on carrier k, determined by the bandwidth and subcarrier spacing. In the tests specified in Table 11.1.8.1.1-2, Test 1, Nk =2; Test 2, Nk =5; Test 3, Nk =7; Test 4, Nk =10. Note 6: The mapping procedure of PSSCH resource and PSFCH resource is specified in Clause 16.3 of TS 38.213 [11]. |  |  |  |  |

Table 11.1.8A.1.1-2: Minimum performance

| Test Number | Bandwidth (MHz) / Subcarrier spacing(kHz) | PSCCH Reference channel | Propagation Channel | Reference value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Probability of missed PSCCH (%) |
| 1 | 10 / 30 | R.PSCCH.2-1.1 | Static propagation condition without external noise | 1 |
| 2 | 20 / 30 | R.PSCCH.2-1.1 | Static propagation condition without external noise | 1 |
| 3 | 30 / 30 | R.PSCCH.2-1.1 | Static propagation condition without external noise | 1 |
| 4 | 40 / 30 | R.PSCCH.2-1.1 | Static propagation condition without external noise | 1 |

----------------------END OF CHANGES 4----------------------------

----------------------START OF CHANGE 5----------------------------

11.1.9A	PSFCH decoding capability test for CA

11.1.9A.1	2RX requirements

11.1.9A.1.1	Minimum requirements

The purpose of this test is to verify the maximum number of PSFCHs received by UE per slot in group cast scenario by using ACK/NACK feedback mode. In each slot, a group of UEs transmits PSFCHs to the tested UE. Information transmitted in each PSFCH is randomly selected from Option A, Option B and Option C with probability of 50%, 25% and 25% respectively. Transmitted PSFCHs are related to one PSSCH which is transmitted by tested UE and occupies all the subchannels.

-	Option A: All the UEs in the group transmit ACKs

-	Option B: One UE transmits NACK and the rest of UEs transmit ACKs. The PSFCH resource index with NACK is random per slot

-	Option C: One UE transmits nothing (i.e.DTX) and the rest of UEs transmit ACKs. The PSFCH resource index of the DTX is random per slot.

The minimum requirements are specified in Table 11.1.9A.1.1-2 with the test parameters specified in Table 11.1.9A.1.1-1 [and the test procedure specified as follows for each carrier with index k:

-	Based on the selected Option, the UEs transmit PSFCHs to the tested UE per slot on xk PSFCH resources in each carrier index k.

Where the number xk is derived by evenly distributing the number of PSFCH(s) resources that the tested UE can receive in a slot (i.e. [IE psfch-RxNumber] specified in clause 4.2.16.1.6 of TS 38.306 [14]) across all carriers.]

Table 11.1.9A.1.1-1: Test parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| HARQ-ACK information |  |  | ACK or NACK |
| Source ID of tested UE |  |  | 0 |
| Sidelink UE i, 0 ≤ i ≤ xk -1(Note 3) | Sidelink transmissions for |  | PSFCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 Low |
|  | Member ID(Note 4) |  | i |
|  | PSFCH resource allocation(Note 5) |  | xk UEs transmit PSFCHs one by one on each RB with CS pair index 0. i.e. UE 0 transmits PSFCH on RB 0, UE 1 transmits PSFCH on RB 1,…, UE (xk -1) transmits PSFCH on RB xk -1 |
|  | PSFCH periodicity | Slots | 1 |
| Note 1: Time offset of transmitted Sidelink UE signal with respect to GNSS reference timing. Note 2: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. Note 3: [xk is derived by evenly distributed the number of PSFCH(s) resources that UE can receive in a slot as specified in Clause 4.2.16.1.6 of TS 38.306 [14] (IE psfch-RxNumber)) across all the carriers]. Note 4: Member ID is an identifier uniquely identifying a member Note 5: All PSFCHs in a slot are corresponding to one PSSCH that occupies all sub channels. |  |  |  |

Table 11.1.9A.1.1-2: Minimum requirement

| Test Number | Bandwidth (MHz) / Subcarrier spacing(kHz) | Propagation Channel | Reference value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Probability of success detection slot with ACK only (%) | Probability of success detection slot with NACK or DTX (%) |
| 1 | 10 / 30 | Static propagation condition without external noise | 99 | 99 |
| 2 | 20 / 30 | Static propagation condition without external noise | 99 | 99 |
| 3 | 30 / 30 | Static propagation condition without external noise | 99 | 99 |
| 4 | 40 / 30 | Static propagation condition without external noise | 99 | 99 |
| Note 1: The probability of success detection slot with ACK only is the probability that the corresponding PSSCH is not retransmitted when Option A is selected. Note 2: The probability of success detection slot with NACK or DTX is the probability that the corresponding PSSCH is retransmitted when Option B or option C is selected. |  |  |  |  |

----------------------END OF CHANGES 5----------------------------

----------------------START OF CHANGE 6----------------------------

A.6.2.2	Reference measurement channels for SCS 30 kHz FR1

Table A.6.2.2-1: PSSCH Reference Channel

| Parameter |  | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  | R.PSSCH.2-1.1 | R.PSSCH.2-1.2 | R.PSSCH.2-1.3 | R.PSSCH.2-1.4 | R.PSSCH.2-1.5 |
| Channel bandwidth |  | MHz | 20 | 20 | 20 | 20 | 20 |
| Subcarrier spacing |  | kHz | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks |  | RB | 20 | 20 | 10 | 10 | 10 |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  |  | 9 | 9 | 9 | 9 | 9 |
| CP-OFDM symbols for slot without PSFCH(Note 2) |  |  | 12 | 12 | 12 | 12 | - |
| Modulation order |  |  | QPSK | 16QAM | 64QAM | QPSK | 64QAM |
| MCS index |  |  | 4 | 11 | 17 | 4 | 27 |
| Number of MIMO layers |  |  | 1 | 1 | 1 | 1 | 1 |
| Number of DMRS REs |  |  | 21 | 15 | 12 | 15 | 12 |
| Number of REs for SCI format 1-A |  |  | 240 | 240 | 240 | 240 | 240 |
| 2nd stage SCI format 2-A configuration | Payloads | Bits | 35 | 35 | 35 | 35 | 35 |
|  | α |  | 1 | 1 | 1 | 1 | 1 |
|  | βoffset |  | 3.5 | 5 | 5 | 3.5 | 2.5 |
| Overhead for TBS determination |  |  | 0 | 0 | 0 | 0 | 0 |
| Transport Block Size for slot with PSFCH |  | Bits | 704 | 1800 | 984 | 208 | 3496 |
| Transport Block Size for slot without PSFCH |  | Bits | 1128 | 2856 | 1928 | 432 | - |
| Transport block CRC |  | Bits | 24 | 24 | 24 | 24 | 16 |
| Maximum number of HARQ transmissions |  |  | 1 | 1 | 1 | 1 | 2 |
| Binary Channel Bits for slots with PSFCH |  |  | 2304 | 4848 | 2232 | 744 | 3816 |
| Binary Channel Bits for slots without PSFCH |  | Bits | 3744 | 7728 | 4392 | 1464 | - |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC), PSFCH symbols, and guard symbols. Note 2: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and guard symbols. |  |  |  |  |  |  |  |

Table A.6.2.2-2: PSSCH Reference Channel with shared spectrum access

| Parameter |  | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  | R.PSSCH.2-1.6 |  |  |  |  |
| Channel bandwidth |  | MHz | 20 |  |  |  |  |
| Subcarrier spacing |  | kHz | 30 |  |  |  |  |
| Allocated resource blocks |  | RB | 11 |  |  |  |  |
| CP-OFDM symbols for slot with PSFCH(Note 1) |  |  | 9 |  |  |  |  |
| CP-OFDM symbols for slot without PSFCH(Note 2) |  |  | 12 |  |  |  |  |
| Modulation order |  |  | 16QAM |  |  |  |  |
| MCS index |  |  | 11 |  |  |  |  |
| Number of MIMO layers |  |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  | 12 |  |  |  |  |
| Number of REs for SCI format 1-A |  |  | 240 |  |  |  |  |
| 2nd stage SCI format 2-A configuration | Payloads | Bits | 35 |  |  |  |  |
|  | α |  | 1 |  |  |  |  |
|  | βoffset |  | 2.5 |  |  |  |  |
| Overhead for TBS determination |  |  | 0 |  |  |  |  |
| Transport Block Size for slot with PSFCH |  | Bits | TBD |  |  |  |  |
| Transport Block Size for slot without PSFCH |  | Bits | TBD |  |  |  |  |
| Transport block CRC |  | Bits | 24 |  |  |  |  |
| Maximum number of HARQ transmissions |  |  | 1 |  |  |  |  |
| Binary Channel Bits for slots with PSFCH |  |  | TBD |  |  |  |  |
| Binary Channel Bits for slots without PSFCH |  | Bits | TBD |  |  |  |  |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC), PSFCH symbols, and guard symbols. Note 2: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and guard symbols. |  |  |  |  |  |  |  |

----------------------END OF CHANGES 6----------------------------

----------------------START OF CHANGE 7----------------------------

B.5	Transmission Model for requirements on bands with shared spectrum access

Clause B.5.1 provides a description of the Downlink Transmission Model to be used in PDSCH Demodulation and CQI reporting performance tests on bands with shared spectrum access. The model as designed in the clause B.5.1 applies to both configurations for channelAccessMode=’semiStatic’ or ‘dynamic’.

Clause B.5.2 provides a description of the Sidelink Transmission Model to be used in PSSCH performance tests on bands with shared spectrum access.

B.5.1	Downlink Transmission Model for bands with shared spectrum access

The Downlink Period for performance tests on bands with shared spectrum access is defined as the duration included in the Test Configuration Parameters.

For tests configured with the RRC Parameter channelAccessMode=’semiStatic’, the duration of the Fixed Frame Period (FFP) equals the duration of the Downlink Period.

For each Downlink Period, the downlink signal to be transmitted is allocated according to the steps listed below:

1.	Select the Downlink Transmission Duration in number of slots, randomly and with equally distributed probability, from the set of possible Downlink Transmission Duration values as included in the Test Configuration Parameters;

a.	This duration includes occupied OFDM symbols and non-occupied OFDM symbols within the Downlink Transmission;

2.	Depending on the Downlink Transmission Duration chosen in the previous step:

b.	If the Downlink Transmission Duration equals 2 slots, all the OFDM symbols in both slots are fully allocated to downlink transmission, else;

c.	If the Downlink Transmission Duration is larger than 2 slots, the configuration of occupied symbols in the last slot included in the downlink duration is selected in number of symbols, randomly and with equally distributed probability, from the set of possible ‘Occupied OFDM symbols in the last slot of the downlink duration’ as included in the Test Configuration Parameters;

For each Downlink Period, the last Slot is not scheduled for downlink transmission. This is to comply with the Idle period requirement in case of channelAccessMode=’semiStatic’, and to align the test setup. In the case of channelAccessMode=’semiStatic’, it can be assumed that the Channel Occupancy Time (COT) covers the entire duration of the Downlink Period except for the last slot.

For each Downlink Period, a uniform random variable from [0, 1] is generated. If the random variable is less than the pLBT value included in the Test Configuration Parameters, the entire Downlink Period duration is muted across the entire bandwidth. This applies to all the signals that were scheduled for transmission, including but not limited to PDSCH, PDCCH, SSB, TRS, CSI-RS, etc.

B.5.2	Sidelink Transmission Model for bands with shared spectrum access

The sidelink transmission model is defined with following steps:

1.	Select the sidelink tranmisson duration randomly with equal distributed probability, from the set of possible Downlink Transmission Duration values as included in the Test Configuration Parameters.

2.   Uniform random variable from [0, 1] is generated.

- If the random variable is higer than the LBT failure probability as defned in the Test Configuration Parameters:

-	Start sidelink burst transmission with Sidelink Transmisson Duration selected in step1 at the end of last slot of previous transmission burst. For all slots within one transmission burst, not the first slot, CP of the first symbol with PSSCH/PSCCH is extended to make the gap located in last symbol of previous slot equal to 16us.

- Otherwise, the transmission is muted and the muting duration is the same as the number of slots for Sidelink Transmisson Duration selected in step1.

----------------------END OF CHANGES 7----------------------------