3GPP TSG-RAN WG4 Meeting #110bis	R4-2406354

Changsha, China, April 15-19, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR for eEMR core maintenance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update core requirements based on RAN4 agreed feature list and RAN2 signalling. Add missing idle-mode accuracy requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update core requirements based on RAN4 agreed feature list and RAN2 signalling. Add missing idle-mode accuracy requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Corresponding RRM requirement would be not accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7, 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

## 4.7	Measurement report for fast CA/DC setup

### 4.7.1	Introduction

The requirements of measurement report for fast CA/DC setup apply in this clause for UE supporting [FG 39-8 and/or 39-9] and configured with measReselectionCarrierListNR-r18 and/or measIdleCarrierListNR-r16 and/or measIdleCarrierListEUTRA-r16 by higher layers.

A UE supporting [FG-39-8] and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall report based on the idle mode measurement specified in clause 4.4, and according to the measurement reporting requirements specified in clause 4.7.3.

A UE supporting [FG 39-9] shall report based on the idle mode measurement requirements specified in the clause 4.2.2, and according to the measurement reporting requirements specified in the clause 4.7.3.

UE is not required to report for fast CA/DC setup from any measurements during IDLE mode for carriers which are not configured by higher layers.

4.7.2	Void

### 4.7.3	Measurement Report Requirements

A UE shall perform validity check and report valid measurement results:

-     if the UE is supporting [FG-39-8] and measIdleValidityDuration-r18 is configured for carriers in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16,

-    if the UE is supporting [FG-39-9] and measReselectionValidityDuration-r18 is configured for carriers in measReselectionCarrierListNR-r18

The measurement results are considered valid if the following conditions are met for the validity check:

-	the measurements are performed before msg1 transmission for RRC resume/setup request within the last:

-	 measIdleValidityDuration-r18 seconds for carriers configured in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, and/or

-	 measReselectionValidityDuration-r18 seconds for carriers configured in measReselectionCarrierListNR-r18,

-	the measurement results satisfy measurement accuracy requirement at the measurement instance.

Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured.

If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

RSRP, RSRQ measurements contained in the measurement reports shall meet the corresponding accuracy requirements at the measurement instance specified in the clauses 10.1.2B, 10.1.3B, 10.1.4B, 10.1.5B, 10.1.7B, 10.1.8B, 10.1.9B, 10.1.10B

End of Change 1

Start of Change 2

## 5.8	Measurement report for fast CA/DC setup

### 5.8.1	Introduction

The requirements of measurement report for fast CA/DC setup apply in this clause for UE supporting [FG 39-8 and/or 39-9] and configured with measReselectionCarrierListNR-r18 and/or measIdleCarrierListNR-r16 and/or measIdleCarrierListEUTRA-r16 by higher layers.

A UE supporting [FG-39-8] and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall report based on the inactive mode measurement specified in the clause 5.4 , and according to the measurement reporting requirements specified in the clause 5.8.3.

A UE supporting [FG 39-9] shall report based on the inactive-mode cell re-selection requirements specified in clause 5.1.2, and according to the measurement reporting requirements specified in the clause 5.8.3.

UE is not required to report for fast CA/DC setup from any measurements during INACTIVE mode for carriers which are not configured by higher layers.

### 5.8.2	Void

### 5.8.3	Measurement Report Requirements

The requirements in clause 4.7.3 shall apply.

End of Change 2