3GPP TSG-RAN4 Meeting #110-bis	R4-2406355

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on R18 NES core requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-Core |  |  |  |  |  | Date: |  |  | 04-08-2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - NES indication in DCI2-9 before nesEvent is not spec compliant. 38.212, clause 7.3.1.3.10 NES-mode indication – 1 bit indicating NES-specific CHO execution condition as defined in Clause 11.5 of [5, TS38.213], if the higher layer parameter nesEvent is configured and the associated serving cell of the block is primary cell; 0 bit otherwise.. - Current RAN4 spec regarding “UE shall start RRM measurement” can introduce unclear interpretation. First, RRM measurement is not clear. Second, the statement can imply the only condition for NES-CHO is the NES indication of DCI2-9 regardless of higher layer configuration for nesEvent. - NES-CHO is not different from legacy CHO but different execution condition. When NES-CHO is provided with legacy CHO the measurement is same but different condition will apply. UE perform measurement for legacy CHO but NES-mode indication will trigger new execution conditions for NES-CHO. But the statement may impliy UE needs to perform measurement separatly for NES-CHO. 38.331, clause 5.3.5.13.4 2> if one of the events associated to the measIds within condTriggerConfig for the applicable cell within the stored condRRCReconfig is not configured with nesEvent, and the other event associated to the measIds within condTriggerConfig for the applicable cell within the stored condRRCReconfig is configured with nesEvent, and at least one of them is fulfilled: 3> consider the applicable cell within the stored condRRCReconfig, associated to that condReconfigId, as a triggered cell; 3> initiate the conditional reconfiguration execution, as specified in 5.3.5.13.5;. |
| Summary of change: |  | - Remove spec non-compliant case for NES-CHO - Remove unclear UE beavior specified for NES-CHO |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 38.133 spec and 38.212 spec may not be aligned w.r.t NES indication in DCI2-9. 38.133 spec define unclear UE behavior. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.4.2.1, 6.1.4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of 1st change >>

### 6.1.4	NR Conditional Handover

#### 6.1.4.1	Introduction

The requirements in this clause are applicable to conditional handover to change the NR PCell to another NR cell.

#### 6.1.4.2	NR FR1 – NR FR1 conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR1 cell to NR FR1 cell.

##### 6.1.4.2.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command.

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until

-	a condition exists at the measurement reference point which will trigger the conditional handover, or

-	a condition exists at the measurement reference point which will trigger the NES-based conditional handover

Tmeasure is the measurements time stated in clause 6.1.4.2.2.

TCHO_execution is the conditional execution preparation time in clause 6.1.4.2.3.

Tinterrupt is the interruption time stated in clause 6.1.4.2.4.

The NES-based conditional handover delay requirements shall apply if UE receives a RRC message implying conditional handover before receiving the NES indication in DCI 2-9 command.

UE is not expected to receive a NES indication in DCI 2-9 command before receiving a RRC message implying NES-based conditional handover. When UE receives a RRC message implying NES-based conditional handover but no NES indication in DCI 2-9 command, no NES-based conditional handover requirement is applied.

The NES indication is specified in clause [5.5.4] in TS 38.331[2].

##### 6.1.4.2.2	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

For conditional inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4.

For NES-based conditional intra-frequency handover:

-	If UE successfully decodes DCI 2-9 command earlier than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equal to Tidentify_intra_with_index or Tidentify_intra_without_index

-	If UE successfully decodes DCI 2-9 command later than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-9 command.

For NES-based conditional inter-frequency handover:

-	If UE successfully decodes DCI 2-9 command earlier than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equal to Tidentify_inter_with_index or Tidentify_inter_without_index

-	If UE successfully decodes DCI 2-9 command later than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-9 command.

Editor Notes: The measurement time delay for NES-based conditional handover is FFS.

Editor Notes: TEvent_DU for NES-based conditional handover is FFS.

When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover. If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover becomes undetectable for a period and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200/2µ Tc while the measurement gap has not been available and the L3 filter has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

##### 6.1.4.2.3	Preparation time

TCHO_execution is the UE execution preparation time for conditional handover, and starts after UE realizes the condition of CHO is met and identity of the target cell is determined. TCHO_execution can be up to 10ms.

##### 6.1.4.2.4	Interruption time

The interruption time is the time between when the UE starts to execute the conditional handover to the target cell and the time the UE starts transmission of the new PRACH.

For intra-frequency or inter-frequency conditional conditional handover, the interruption time shall be less than

Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms

Where:

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3]

T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

#### 6.1.4.3	NR FR2 – NR FR1 conditional handover

The requirements in this clause are applicable to inter-frequency conditional handover from NR FR2 cell to NR FR1 cell.

The requirements defined in clause 6.1.4.2 applies assuming inter-frequency handover and:

Tprocessing is time for UE processing. Tprocessing can be up to 40ms.

#### 6.1.4.4	NR FR2 – NR FR2 conditional handover

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR2 cell to NR FR2 cell.

##### 6.1.4.4.1	Handover delay

Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2].

When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command.

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until

-	a condition exists at the measurement reference point which will trigger the conditional handover, or

-	a condition exists at the measurement reference point which will trigger the NES-based conditional handover if NES-based conditional handover applies

Tmeasure is the measurements time stated in clause 6.1.4.4.2.

TCHO_execution is the conditional execution preparation time in clause 6.1.4.4.3.	Tinterrupt is the interruption time stated in clause 6.1.4.4.4.

The NES-based conditional handover delay requirements shall apply if UE receives a RRC message implying conditional handover before receiving the NES indication in DCI 2-9 command.,

UE is not expected to receive a NES indication in DCI 2-9 command before receiving a RRC message implying NES-based conditional handover. When UE receives a RRC message implying NES-based conditional handover but no NES indication in DCI 2-9 command, no NES-based conditional handover requirement is applied.

The NES indication is specified in clause [5.5.4] in TS 38.331[2].

##### 6.1.4.4.2	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

For conditional inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4.

For NES-based conditional intra-frequency handover:

-	If UE successfully decodes DCI 2-9 command occurs earlier than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equal to Tidentify_intra_with_index or Tidentify_intra_without_index

-	If UE successfully decodes DCI 2-9 command occurs later than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-9 command.

For NES-based conditional inter-frequency handover:

-	If UE successfully decodes DCI 2-9 command occurs earlier than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equal to Tidentify_inter_with_index or Tidentify_inter_without_index

-	If UE successfully decodes DCI 2-9 command occurs later than the time at the end of TEvent_DU + Tidentify_inter_with_index or TEvent_DU + Tidentify_inter_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-9 command.

Editor Notes: The measurement time delay for NES-based conditional handover is FFS.

Editor Notes: TEvent_DU for NES-based conditional handover is FFS.

When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover. If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover becomes undetectable for a period and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200/2µ Tc while the measurement gap has not been available and the L3 filter has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

##### 6.1.4.4.3	Preparation time

TCHO_execution is the UE execution preparation time for conditional handover, and starts after UE realizes the condition of CHO is met and identity of the target cell is determined. TCHO_execution can be up 10ms.

##### 6.1.4.4.4	Interruption time

The interruption time is the time between when the UE starts to execute the conditional handover to the target cell and the time the UE starts transmission of the new PRACH.

For intra-frequency or inter-frequency conditional conditional handover, the interruption time shall be less than

Tinterrupt = Tprocessing + TIU + T∆ + Tmargin ms

Where:

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3]

T∆ is time for fine time tracking and acquiring full timing information of the target cell. TΔ = Trs.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

#### 6.1.4.5	NR FR1 – NR FR2 conditional handover

The requirements in this clause are applicable to inter-frequency conditional handover from NR FR1 cell to NR FR2 cell.

The requirements defined in clause 6.1.4.4 applies assuming inter-frequency handover and:

Tprocessing is time for UE processing. Tprocessing can be up to 40ms.

## << End of 1st change >>