3GPP TSG-WG4 Meeting #110bis	R4-2406373

# Changsha, China, April, 15 - 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR 38.133 Corrections to measurement period requirements for NR CPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Corrections to Measurement period requirements for CPP are needed as described in the summary of change and explained in DP R4-2405883. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 5.6.8.5 for inactive mode and subclause 9.9.8.5 for connected mode, in case of UL Tx timing change due to TA command or TA offset change, it is corrected, that the DL RSCP with UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the DL RSCP and UE Rx-Tx time difference measurement period requirements shall not apply. - Some names are corrected and typos are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear and inconstent measurement requirements for UE Rx-Tx time difference and DL RSCP in case of UL Tx timing change. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.8.5, 9.9.8.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The changes are based on Big CR R4-2403567 approved at RAN4 #103. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2405884. |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

### 5.6.8	Measurement requirements for DL RSCP reported with UE Rx-Tx time difference

#### <unchanges sections omitted>

#### 5.6.8.5	Measurement Period Requirements

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message with nr-UE-RSCP-Request from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 5.6.8.3) UE Rx-Tx and DL RSCP measurements, defined in TS 38.215 [4], during the time window configured to UE via nr-DL-PRS-MeasurementTimeWindowsConfig but the time window periodicity is not configured, and the start of the measurement period is the start of the window.

Otherwise, the UE shall be able to measure multiple (up to the UE capability specified in Clause 5.6.7.3) UE Rx-Tx and DL RSCP measurements, defined in TS 38.215 [4], during the measurement period TDL RSCP with UERxTx,Total defined as:

TDL RSCP with UERxTx,Total=i=1LTDL RSCP with UERxTx,i+ L-1*maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TDL RSCP with UERxTx,i is the measurement period for DL RSCP with UE Rx-Tx measurement in positioning frequency layer i as specified below:

TDL RSCP with UERxTx=Kcarrier_PRS*NRxTx,TEG,i*NRxBeam*NPRSslotN'Lavailable_PRSN*Nsample-1*Teffect+Tlast ,

where:

- Kcarrier_PRS, NRxTx,TEG, NRxBeam, NPRS,islot, N, N’, Nsample, Teffect and Tlast are defined in clause 5.6.4.5.

- DL RSCP performed during TDL RSCP with UERxTx is a single sample measurement where DL RSCP and UE Rx-Tx measurements are performed on the same PFL.

-	Lavailable_PRS is the time duration of available PRS resources in the positioning frequency layer, to be measured during TPRS, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	When periodic time window(s) are configured by the LMF,	Tavailable_PRS= LCMTPRS,TDRX, Twindow, the least common multiple between TPRS , TDRX and Twindow being the maximum periodicity of the indicated time window(s).

-	When periodic time window(s) are not configured by the LMF, Tavailable_PRS=LCMTPRS,TDRX, the least common multiple between TPRS and the DRX cycle length TDRX .

-	Lavailable_PRS and TPRS are calculated by only considering the PRS resources in the indicated resources sets overlapping with the indicated time window(s).

The time TDL RSCP with UERxTx starts from the first time window (TDL RSCP) configured by LMF within DRX cycle containing the DL PRS resources in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

If the RRC state transition occurs from RRC_INACTIVE to RRC_CONNECTED state during the measurement period then the UE shall continue the DL RSCP measurement and shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the serving cell.

If cell reselection occurs during the measurement period then the UE shall restart the DL RSCP and UE Rx-Tx time difference measurements after it obtains SRS configuration and Timing Advance command from the new serving cell.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If the DRX cycle is reconfigured during the measurement period then the measurement period can be longer.

If during the measurement period, PRS resources overlap with other DL signals/channels then the measurement period can be longer.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

The requirements in clause 5.6.8 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the DL RSCP with UE Rx-Tx measurement period, then the DL RSCP with UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the DL RSCP and UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the DL RSCP with UE Rx-Tx measurement period, then the DL RSCP with UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the DL RSCP and UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1.2 during the measurement period, then:

-	DL RSCP and UE Rx-Tx measurement period requirements in this clause shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	DL RSCP and UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. The DL RSCP with UE Rx-Tx time difference measurement period may be restarted in such case.

--- End of change 1 ---

--- Start of change 2 ---

#### 9.9.8.5	Measurement Period Requirements for DL RSCP and UE Rx-Tx time difference

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message with nr-UE-RSCP-Request from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 9.9.8.3) UE Rx-Tx and DL RSCP measurements, defined in TS 38.215 [4], during the time window configured to UE via nr-DL-PRS-MeasurementTimeWindowsConfig but the time window periodicity is not configured, and the start of the measurement period is the start of the window.

Otherwise, the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.8.3) UE Rx-Tx and DL RSCP measurements, defined in TS 38.215 [4], during the measurement period TRSCP with UERxTx,Total defined as:

TDL RSCP with UERxTx,Total=i=1LTDL RSCP with UERxTx,i+ L-1*maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in positioning frequency layer i

TDL RSCP with UERxTx,i is the measurement period for DL RSCP with UE Rx-Tx measurement in positioning frequency layer i as specified below:

TDL RSCP with UERxTx=kmultiTEG*CSSFPRS*ceil( Kp,PRS)*NRxBeam*NPRSslotN'Lavailable_PRSN*Nsample-1*Teffect+Tlast ,

where:

- DL RSCP performed during TDL RSCP with UERxTx is a single sample measurement where DL RSCP and UE Rx-Tx measurements are performed on the same PFL.

- NRxBeam, CSSFPRS, kmultiTEG, Nsample, NPRS,islot, N, N’, Teffect and Tlast are defined in clause 9.9.4.5.

- Nsample = 1 or 2 or 4 as defined in clause 9.9.4.5.

-	Kp,PRS is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS = 1 for UE not configured with concurrent measurement gap.

- When periodic time window(s) are configured by the LMF, for a window W of duration max(TPRS,  MGRP_max, Twindow), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion and the periodic time window:

Ntotal is the total number of associated gap occasions covering PRS occasions and the periodic time window(s) within the window W, including both dropped and non-dropped instances of the associated measurement gap within the window W, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions and the periodic time window(s) within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

- Otherwise, ror a window W of duration max(TPRS,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

Ntotal is the total number of associated gap occasions covering PRS occasions within the window W, including both dropped and non-dropped instances of the associated measurement gap within the window W, and

Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

Requirements do not apply if Navailable =0.

-	Lavailable_PRS is the time duration of available PRS resources in the positioning frequency layer, to be measured during Tavailable_PRS, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26].

- When periodic time window(s) are configured by the LMF, Tavailable_PRS=LCM(Tprs, MGRP, Twindow), otherwise  Tavailable_PRS=LCM(Tprs, MGRP). Twindow is the maximum periodicity of the configured time window(s). Tprs and MGRP are defined in clause 9.9.8.5.

- Lavailable_PRS and Tprs are calculated by only considering the PRS resources in the indicated resources sets overlapping with both the MG and the indicated time window(s).

Except for deferred MT-LR as defined in clause 4.1a.5 [TS 23.273], the time TDL RSCP with UERxTx starts from the first time window (TDL RSCP) configured by LMF within MG instance aligned with DL PRS resource(s) in the assistance data after both the NR-Multi-RTT-RequestLocationInformation message and NR-Multi-RTT-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

For deferred MT-LR with other event than “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the time TDL RSCP with UERxTx starts from the first time window (TDL RSCP) configured by LMF within MG instance aligned with DL PRS resource(s) in the assistance data after the associated event(s) occurs.

For deferred MT-LR with event “Periodic Location” as defined in clause 4.1a.5.1 [TS 23.273], the UE shall perform the DL RSCP with UE Rx-Tx time difference measurement in each reporting period and activate the location report at the time when the periodic timer expires.

The DL RSCP with UE Rx-Tx time difference measurement period is restarted if HO occurs during the measurement period and after SRS reconfiguration on the target cell is complete.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If during the measurement period of the positioning frequency layer, the MG pattern is reconfigured either per UE request or not per UE request, the measurement period can be longer.

The requirements in this section apply, provided no PRS symbols are dropped during the measurement period TDL RSCP with UERxTx within measurement gaps due to collisions with other signals; otherwise, a longer measurement period may be used.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

The requirements in clause 9.9.8 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

When PSCell or Scell addition or release does not cause SRS reconfiguration during the measurement period, UE shall continue and complete the DL RSCP and UE Rx-Tx time difference measurements, and the measurement period requirements apply.

When PSCell or Scell addition or release causes SRS reconfiguration during the measurement period, UE shall restart the DL RSCP and UE Rx-Tx time difference measurement after the SRS reconfiguration on the target cell is complete.

When SRS is reconfigured without serving cell change during the measurement period, UE shall restart the DL RSCP and UE Rx-Tx time difference measurement after the SRS reconfiguration is complete.

When a serving cell change occurs during the measurement period, the UE shall restart DL RSCP and UE Rx-Tx time difference measurements.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the measurement period, then the DL RSCP with UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the DL RSCP and UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the measurement period, then the DL RSCP with UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the DL RSCP and UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the UE autonomous timing adjustment defined in clause 7.1.2 during the measurement period, then:

-	DL RSCP and UE Rx-Tx measurement period requirements in this clause shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	DL RSCP and UE Rx-Tx measurement period requirements in this clause shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission. The DL RSCP with UE Rx-Tx time difference measurement period may be restarted in such case.

--- End of change 2 ---