3GPP TSG-RAN WG4 Meeting # 110-bis 	    R4-2406383

Changsha, China, 15 – 19 April 2024

Agenda Item:	6.12.4

Source: 	Ericsson

Title:	WF on RedCap positioning and PRS/SRS bandwidth aggregation

Document for:	Approval

# General Aspects

# RAN2 LS (R2-2401912) on positioning MAC agreements

Agreement

- TA validation requirement in clause 5.6.6 of TS38.133 also applies to TA validation for SRS aggregation in RRC_INACTIVE state.

# RAN1 LS (R1-2401801/ R4-2404109) on bandwidth used for RedCap positioning

Agreement

- No update is needed to RAN4#110 agreed minimum PRS bandwidth for RedCap positioning with Rx FH based on the RAN1 LS R1-2401801.

# RedCap Positioning

## Core requirement

# Requirements supported for RRC_INACTIVE/IDLE mode for RedCap positioning with Rx FH

Agreement

- Define requirements for RRC_INACTIVE/IDLE state supporting the following Rx FH cases:

  - 1 hop per slot.

  - 2 hops per slot

  - 1 hop every 2 slots

# Number of Rx hops measured by RedCap UE in RRC_INACTIVE/IDLE mode

Agreements

- For PRS measurements with FH in RRC_IDLE and RRC_INACTIVE:

  - Introduce a time window mechanism similar to Rel-17 RRC_INACTIVE PRS measurements

  - Define the number of hops within a time window leveraging the definition for number of hops within a single MG occasion in RRC_CONNECTED

  - The details of the respective time window are FFS.

  - The PRS measurements with FH for a given PRS resource are limited to a single time window (i.e., single measurement sample is based on single time window measurements).

# Minimum PRS BW measured by RedCap UE across all hops in RRC_INACTIVE/IDLE state

Agreements

- The minimum PRS BW expected to be measured with Rx hopping in RRC_INACTIVE/IDLE state is given by

  - minBWPRS,Nhops∙BWhop-BWoverlap∙Nhops-1

where

    - BWPRS is determined by the minimum among the configured PRS BW, UE capability reported as [component 1 of the FG 41-5-1], and the total BW of all hops requested by LMF.

    - BWhop is the BW per hop signalled in the UE capability.

    - BWoverlap is the minimum hop overlap signalled in the UE capability.

    - Nhops is the number of Rx hops measured by the UE in RRC_INACTIVE/IDLE state within a single time window.

# Applicability of core requirements for Rx FH in RRC_CONNECTED state

- Conclusion: no further discussion on the issue

# Applicability of core requirements for Rx FH in RRC_INACTIVE/IDLE state

Agreement

- For PRS measurements with FH in RRC_IDLE and RRC_INACTIVE, the measurement period requirement with FH (multiple hops) applies provided all PRS resources in a PFL have the same number of inter-slot repetitions within a time window instance and the same spacing between inter-slot repetitions. If these conditions do not apply the measurement period can be longer.

## Performance requirement

# Separate test cases for measurement delay and accuracy requirements for RedCap positioning

Agreement:

- Separate delay and accuracy TCs.

# Bandwidth for accuracy requirement for RedCap positioning

Agreement:

- Option 2c: combination (total measured PRS BW during all hops, per-hop BW), regardless of the overlapping BW size

# Accuracy requirement for RedCap positioning (reduced number of samples)

Agreements:

- Define reduced-samples accuracy requirements for 1 Rx RedCap UE with and without Rx FH in FR1:

- RSTD:

  - AWGN only

  - (-3 dB for reference cell, -6 dB for target cell)

- RSRP and UE Rx-Tx:

  - AWGN only

  - 0 dB, -6 dB.

- PRS-RSRPP:

  - Two-Tap channel model only

  - 0 dB, -6 dB.

- Define reduced-samples accuracy requirements for 2 Rx RedCap UE with and without Rx FH in FR2:

- RSTD/RSRP/UE Rx-Tx:

  - AWGN only

  - Rel. 17 side conditions

- PRS-RSRPP:

  - Two-Tap channel model only

  - Rel. 17 side conditions.

# Accuracy requirement for PRS-RSRP and PRS-RSRPP with Rx FH

Agreement:

- For PRS-RSRP and PRS-RSRPP measurements with FH, apply the measurement accuracy requirements without FH defined for the corresponding PRS BW per hop

# PRS/SRS aggregation

## Core requirement

# Clarification regarding nominal channel spacing in core requirements for PRS aggregation

- Conclusion: Further discuss the issue as a part of Performance requirements applicability conditions

## Performance Requirement

# Side condition for PRS aggregation accuracy requirements

Agreement:

Rel-17 side conditions for RSTD and UE Rx-Tx time difference is reused for PRS based positioning measurement with bandwidth aggregation.

# Bandwidth for PRS aggregation accuracy requirements

Agreement:

FFS: For RSTD and UE Rx-Tx measurements based on PRS aggregation, in addition to the agreed per-PFL BWs, requirements are also defined for per-PFL BW 400 MHz (272 PRBs, 120 kHz SCS) in FR2.

# Update to simulation assumption for PRS aggregation

Agreement:

Companies further check in this meeting and conclude by the end of the meeting whether the update in simulation assumptions is needed or not.

# Accuracy requirements for PRS-RSRP and PRS-RSRPP measurements based on aggregated PRS resources

Agreements:

FFS:

      - Option 1: The requirements are defined, based on the following principle: For a given side condition and propagation condition, existing accuracy requirement for the largest PRS BW among the aggregated PFLs applies to the PRS-RSRP/RSRPP measurements in case of PRS BW aggregation.

      - Option 2: The requirements are defined, based on the following principle: For a given side condition and propagation condition, existing accuracy requirement applies for the total aggregated BW to the PRS-RSRP/RSRPP measurements in case of PRS BW aggregation.

# Considerations for performance requirements for PRS aggregation

Agreements:

- The number of samples in accuracy requirements for PRS bandwidth aggregation:

- Option 1: both reduced number of samples and 4 samples.

- FFS: The measurement accuracy requirement for RSTD/UE Rx-Tx time difference should depend on the baseband sampling rate as well as the number of PFLs.

# Work split

# Timeline for DraftCR submissions

Agreements:

- LPHAP, SL positioning, CPP: all draft CRs are to be provided in May meeting.

- RedCap and PRS/SRS BW aggregation: phased approach applies

      - Phase I DraftCRs are submitted to RAN4#111

        - Accuracy requirements CRs

        - Selected test cases

      - Phase II DraftCRs are submitted to RAN4#112.

        - Selected test cases

Note: Discuss further during the meeting the selection of test cases for Phase I and Phase II.

# Testing principles for Rel. 18 positioning

Agreement:

- Testing principles, to limit the number of test cases the UE is expected to pass, are defined for Rel. 18 positioning. Companies to bring proposals on testing principles in RAN4#111.

# References

- R4-2406288, Ad-hoc minutes #1 on RRM requirements for NR_pos_enh2, Intel.

- R4-2406289, Ad-hoc minutes #2 on RRM requirements for NR_pos_enh2, Ericsson.

- R4-2406387, Ad hoc minutes for AH#3 on Rel-18 positioning, Ericsson.

- R4-2406388, Ad hoc minutes for AH#4 on Rel-18 positioning, Ericsson.

- R4-2406433, Ad hoc minutes for AH#5 on Rel-18 positioning, Ericsson.