3GPP TSG-RAN WG4 Meeting # 110bis													   R4-2406384

Changsha, CN, 15 April ‒ 19 April, 2024

Title:	WF on SL positioning and carrier phase positioning

Agenda item:			6.12.4

Source:	CATT

Document for:	Approval

- Introduction

This WF includes the agreements and open issues discussed in topic summary for [110bis][215] NR_pos_enh2_part2.

- Topic #1: Sidelink Positioning Core Requirements (agenda 6.12.2.2)

  - Sub-topic 1-1 SL Positioning Core Requirements maintenance

Issue 1-1-1: End point of SL-PRS based RSTD measurement period requirements

Agreements:

- The SL RSTD measurement period ends after the UE has measured SL PRS resources from at least two different Tx UEs including target and reference UEs.

- FFS whether any updates to the TS 38.133 are needed.

Issue 1-1-2: Applicability of SL PRS measurement period requirements

Proposals

- Option 1:

  - Limits on the number of active SL PRS resources per slot and number of active slots across all configured resource pools and across all bands defined by RAN1 in FG 41-1-1a should be included as applicability conditions for measurement period requirements for SL positioning measurements.

  - Measurement requirements for SL RSTD apply provided the time separation between the target and reference SL PRS resources is no larger than [160 ms].

Issue 1-1-3: UE behaviour and the impact on SL-PRS measurement requirements when synchronization reference source change occurs at Rx side

Agreements:

- When the synchronization reference source changes occurs during the measurement period at Rx side, i.e., at the UE which is performing the measurement,

  - For SL PRS RSTD measurements,

    - UE shall restart the measurement and measurement period can be longer

  - Do not define the exact measurement period after the measurement restart in the specification for SL PRS RSTD, Rx-Tx and RTOA measurements

Notes: For SL PRS based Rx-Tx measurements the agreement was reached in RAN4 #108bis (R4-2317308)

Issue 1-1-4: UE behaviour and the impact on SL-PRS measurement requirements when synchronization reference source change occurs at Tx side

Proposals

- Option 1a:

  - When the synchronization reference source change occurs during the measurement period at Tx side, e.g., UE receives an updated sl-RTD-Info, the measuring UE shall restart the SL PRS-based timing measurements (SL Rx-Tx, SL RSTD, and SL RTOA).

- Option 1b:

  - To ensure that the restart of the SL measurement due to the synch source change is not too much delayed, RAN4 specifies the time until which the UE shall report the restarted measurement (e.g., as in LTE) which is extended by the measurement period with each restart (K):

TSL RSTD/Rx-Tx/RTOA,restart=K+1*TSL RSTD/Rx-Tx/RTOA, Total .

  - No need to specify the maximum allowed value for the number K of restarts.

- Option 2:

  - For synchronization reference source change occurs at Tx side, measurement accuracy requirements do not apply and no specific UE behaviour is defined.

- Option 3:

  - RAN4 to send an LS to RAN1 for getting clarification on how the measuring UE is informed about the synchronization reference source change at the Tx UE side.

- Option 4:

  - If a UE receives an updated sl-RTD-Info while performing SL RSTD or SL RTOA measurements, the UE is allowed to restart the measurements and the measurement period can be longer.

- Option 5:

  - If the UE receives an indication of the synchronization source change at a transmitting anchor UE by SL-RTD-Info [37],  while the UE is performing the [TBD] measurements, then the measuring UE shall restart the SL RTOA measurement after the synchronization reference source change, in which case the UE is allowed to extend the TSL RTOA,total measurement period.

Issue 1-1-5: Impact of other channels/signals/SL procedures

Agreements:

- The SL-PRS based measurement requirements apply provided no SL-PRS symbols are dropped during the measurement period.

- If the reception of the slots containing SL-PRS is dropped, the measurement period can be extended.

Issue 1-1-6: Impact of network coverage change

Agreements:

- RAN4 not to define the SL-PRS based measurement period requirements when there is network coverage change.

- SL-PRS measurement requirements apply provided that reception of the slots containing SL-PRS is not interrupted due to network coverage change.

Issue 1-1-7: Impact of Uu link connection

FFS:

- Option 1:

  - NOTE 3: When a UE in RRC_CONNECTED state is performing transmissions and/or reception for SL positioning operation, the UE shall meet all the requirements specified in Clause 6, assuming that UE has a dedicated RX/TX chain for the sidelink operation. Otherwise, the UE may interrup the SL positioning measurements in order to meet the measurement requirements specified in Clause 6.

Issue 1-1-8: Necessity of independent Requirements for SL PRS-RSRP(P)

Agreements:

- Further check the need for removing dedicated clauses for requirements for SL PRS-RSRP(P).

  - Send LS to RAN2 to ask whether SL PRS-RSRP and SL PRS-RSRPP can be reported without other measurements.

Topic #2: Carrier Phase Positioning Core Requirements (agenda 6.12.2.2)

Sub-topic 2-1 Carrier Phase Positioning Core Maintenance

Issue 2-1-1: Measurement period requirements for DL RSCP/DL RSCPD with multiple PFLs configured

Proposals

- Option 1:

  - When DL RSCPD is reported with RSTD and DL RSCPD is measured in the indicated PFL j with periodic time windows, the measurement period is given by

TRSCPD with RSTD,Total=TRSCPD with RSTD,j+i=1,i≠jLTRSTD,i+ L-1*maxTeffect,i

    - FFS how to adapt the requirement for aperiodic time window

  - When DL RSCP is reported with UE Rx-Tx and DL RSCP is measured in the indicated PFL j with periodic time windows, the measurement period is given by

TRSCP with UERxTx, Total=TRSCP with UERxTx, j+i=1,i≠jLTUERxTx,i+ L-1*maxTeffect,i

    - FFS how to adapt the requirement for aperiodic time window

Issue 2-1-2: Clarification on the PRS measurement period requirements for DL RSCP/DL RSCPD

FFS:

- For UE configured for CPP measurement with legacy measurement with multiple PFLs, When LMF does not configure measurement time window(s) for a PFL or UE does not support FG 41-2-3,

  - Carrier phase positioning measurement is only performed on single PFL.

    - FFS which PFL is measured.

  - Existing requirements for RSTD/UE Rx-Tx without time window apply.

- For UE configured for CPP measurement with legacy measurement with multiple PFLs, when UE is configured with time window  and does not support FG 41-2-8/41-2-9,

  - Option 1:

    - Existing requirements for RSTD/UE Rx-Tx without time window apply.

  - Option 2:

    - existing requirements without time window apply for legacy measurements, but UE is not required to report CP if the resource set(s) associated to the window have occasions outside the time window.

  - Option 3:

    - Not to define requirements for this case.

Issue 2-1-3: The impact of carrier frequency offset

Proposals

- Option 1:

  - The accuracy requirements for CP measurement apply provided that the two PRS resources for calculating RSCPD or relative RSCP are located in the same set of symbols.

- Option 2:

  - Define a common reference time and refer the DL-PRS carrier phase measurements to this reference time by subtracting the phase rotation due to the carrier frequency offset in the time interval between the DL-PRS and the reference time from the carrier phase measurement.

  - Define the referred carrier phase difference as the difference between the referred carrier phase measurements.

  - Define the same common reference time for the UE and the PRU.

  - The UE and the PRU report either the referred carrier phase measurements or the carrier phase difference measurements computed using the referred carrier phase measurements.

  - Proposals 1-4 are equally applicable to remove the errors due to carrier frequency offsets when the UE and PRU take measurements in different slots.

- Option 3:

  - RAN4 to investigate measures for mitigating the impact due to carrier frequency offset in the TRP transmissions and UE/PRU receptions, in order to improve the quality of the CP estimate.

  - RAN4 to agree a model for CFO compensation at UE side in order to evaluate the CFO impact on DL RSCPD and DL RSCP measurement accuracy.

- Option 4:

  - RAN4 to not further discuss methods to mitigate impact of carrier frequency offset on CPP measurements.

- Option 5:

  - RSCPD accuracy derived from RAN4 simulations applies assuming zero carrier frequency error at the UE and TRPs.

  - RAN4 to add margin to the RSCPD accuracy derived from RAN4 simulations to account for residual frequency errors.

Issue 2-1-4: The impact of Tx timing change

Agreements:

- The proposal has already been implemented in the CR in last meeting (see big CR R4-2403567 clause 9.9.8.5). No need to discuss this issue.

Issue 2-1-5: The impact of AGC phase offset

Proposals

- Option 1:

  - RAN4 to specify UE behavior to keep single / same AGC level for both ToA measurements for reference TRP and target TRP in order to perform the RSCPD measurement.

Topic #3: Sidelink Positioning Performance Requirements (agenda 6.12.3.1)

Sub-topic 3-1 SL Positioning Performance requirements

Issue 3-1-1: SINR side conditions

Agreements:

- SL RSTD:

  - Option 1: (0, -6) dB for reference and another anchor UE

  - Option 2: (-3, -6) dB for reference and another anchor UE

- SL UE Rx-Tx and SL RSRP(P):

  - FFS -6 dB,

  - FFS -3 dB

- SL AoA and SL RTOA (for core requirements only):

  - -6 dB

Issue 3-1-2: General principles for defining accuracy requirements

Proposals

- Option 1:

  - When defining accuracy requirements, try to reduce the number of sets and align the grouping for different measurements as much as possible.

Issue 3-1-3: Assumptions to define SL PRS measurement accuracy requirements

Agreements:

- Define measurement accuracy requirements for SL-PRS based RSTD, SL-PRS based UE Rx-Tx time difference, SL-PRS based RSRP and SL-PRS based RSRPP:

  - independent of SL-PRS comb size,

  - for same channel profiles as defined for Uu

  - for measurement samples 1 or 4 depending on the RB number of PRS BW

Issue 3-1-4: Measurement accuracy requirements

Agreements:

- RAN4 to define accuracy requirement for SL RSTD and SL Rx-Tx by considering the framework of ±(X+Y[+Z]) Tc and ±(X[+]) Tc, respectively.

  - X is the simulated measurement accuracy for a given propagation condition and number of measurement samples,

  - Y is the frequency/clock drift margin,

  - FFS: Z and  are the RF calibration margins.

Issue 3-1-5: RF calibration margins

Proposals

- Option 1:

  - Same RF margins are considered for SL PRS accuracy requirements as in Uu, exact values for the margins are FFS.

- Option 2:

  - Define the RF calibration margin for SL RSTD measurements in FR1 using the following structure:

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 48 | ≥ 24 | N/A | Z1 |
| ≥ 96 | ≥ 48 | ≥ 24 | Z2 |
| N/A | ≥ 96 | N/A | Z3 |

  - Define the RF calibration margin for SL Rx-Tx measurements in FR1 using the following structure:

| Min(SL PRS Rx BW, SL SRS Tx BW) (RB) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 48 | ≥ 24 | N/A | 1 |
| ≥ 96 | ≥ 48 | ≥ 24 | 2 |
| N/A | ≥ 96 | N/A | 3 |

Issue 3-1-6: Test case list

Agreements:

- No test cases are defined to specifically verify measurement delay requirements for SL PRS-RSRP/RSRPP, regardless of whether the measurement delay requirements are specified for SL PRS-RSRP/RSRPP.

- Accuracy requirements for SL PRS-RSRP/RSRPP are verified when configured together with RSTD/RX-TX, respectively.

  - Option 1a: separate section for testing SL PRS-RSRP/PRS-RSRPP, without verifying the accuracy of the other (SL RSTD/Rx-Tx) measurement, respectively

  - Option 1b: separate section for testing SL PRS-RSRP/PRS-RSRPP, with verifying also the accuracy of the other (SL RSTD/Rx-Tx) measurement, respectively

  - Option 2: same section for SL PRS-RSRP/RSRPP and SL RSTD/Rx-Tx, but different test cases - with and without SL PRS-RSRP/RSRPP.

Issue 3-1-7: Test configurations

Agreements:

- Define SL positioning test cases using only AWGN and 2-tap channel (for SL PRS RSRPP) propagation conditions.

- RAN4 to define the test cases for dedicated resource pool and shared resource pool as test configurations, and the SL PRS configurations could apply to both types of resource pool.

Other proposals:

- Option 1:

  - Define SL PRS configurations for test as listed in table 3 and the other configurations can use the existing V2X test as baseline.

- Table 3: SL PRS configurations for test

| PRS Parameters | Values |
| --- | --- |
| SL PRS transmission for different SL UE | TDMed in different slots |
| SCS | 15kHz, 30kHz |
| PRS comb size | 4, 2 |
| Number of PRS symbol | 4 |
| SL PRS resource time gap (slot) | 1 |
| RB numbers containing PRS within channel BW | 48，96 |
| Resource pool | dedicated |
| Note 1: Unless otherwise specified in the test case |  |

- Option 2:

  - RAN4 to consider the following SL PRS related parameters for the test.

    - (symbol num, comb size): (4, 4) and (2, 4)

    - BW: 48 RB for delay test

    - Es/Iot: 3dB (to ensure 100% PSCCH decoding)

  - RAN4 to consider the following SL PRS transmission pattern for the delay test.

    - TX UE1: slot n

    - TX UE2: slot n + 1 and slot n + 100ms

- Option 3:

  - Define SL PRS configurations for SL positioning test cases that apply to both shared and dedicated resource pools

    - No TDM of SL PRS from different UEs in the same slot

    - No comb-based multiplexing of SL PRS from different UEs in the same slot

    - No FDM of SL PRS from different UEs in the same slot

Issue 3-1-8: Spec structure for test cases of SL positioning

Agreements:

- Discuss on the draft CR directly (R4-2405530 and its revisions).

Topic #4: Carrier Phase Positioning Performance Requirements (agenda 6.12.3.5)

Sub-topic 4-1 Carrier Phase Positioning Performance Requirements

Issue 4-1-1: Side condition

Agreements:

- Two sets of side conditions for DL RSCPD accuracy requirements:

  - [-3, -6] dB for AWGN, two-tap channel model,

  - [-6, -13] dB for AWGN.

- Relative DL RSCP:

  - [0, -6] dB for AWGN, two-tap channel model,

  - [-3, -13] dB for AWGN.

Issue 4-1-2: DL RSCPD absolute accuracy requirements

Agreements:

- Accuracy requirements for DL RSCPD and relative DL RSCP are defined using the same PRB numbers as used in existing RSTD and UE Rx-Tx accuracy requirements.

- FFS: On top of simulated CPP measurement results, additional margins for frequency drift and RF calibration should be considered when defining RSCPD and relative RSCP accuracy requirements.

- FFS: whether simulation assumptions need to be updated.

Issue 4-1-3: DL RSCP relative accuracy requirements

Agreements:

- Resolved under issue 4-1-1.

Issue 4-1-4: Accuracy requirements for UL carrier phase measurement:

Agreements:

- RAN4 not to define accuracy requirements for UL RSCP.

Issue 4-1-5: Test case list

Agreements:

- Include the following section into the work split:

| Set | PRS measurement test case scenarios | Impacted section in TS 38.133 |
| --- | --- | --- |
| 0-1 | General: Configuration for the indicated time window | A.3.x |

- Separate TCs are defined for:

  - RRC states:

    - RRC_CONNECTED.

    - RRC_INACTIVE.

    - FFS: TCs are defined for RRC_IDLE, including whether and how to specify.

  - FR1 and FR2

  - Delay and accuracy

  - RSTD with RSCPD, UE Rx-Tx with RSCP

  - Propagation conditions: AWGN

Issue 4-1-6: Test configurations

Agreements:

Applies for all RRC states:

- The test configurations in existing UE Rx-Tx and RSTD tests can be reused for RSCP with UE Rx-Tx and for RSCPD with RSTD test cases, respectively.

- RAN4 to define the tests for CPP with periodic time window

  - Periodicity: [x times of PRS resource periodicity]. FFS: for RRC_INACTIVE.

  - Offset: [same as PRS resource offset]

  - Duration: [covering all PRS resources from all TRPs]

Issue 4-1-7: Test requirements

Agreements:

- For carrier phase-based positioning, test cases are only defined for the case where the UE is configured to perform carrier phase measurement with legacy positioning measurements within the configured measurement time window.

- Further discussion is needed on whether to verify in the RSCPD/RSCP TC the accuracy of the other measurement configured and reported together with RSCPD/RSCP.

Reference

- R4-2404826 Topic summary for [110bis][215] NR_pos_enh2_part2, CATT, RAN4#110bis